# Narrowband LTE – Cell search design

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see  [1].

The study is open to both a non-legacy based design, and/or a backward compatible evolution of GSM/EDGE.

A non-legacy based design known as Narrowband LTE (NB LTE) has been described in [2] and [3]. In this contribution, a design for the cell search procedure in NB LTE is presented.

# Background

Essential functions of a cell search procedure are to detect a suitable cell to camp on, and for that cell, obtain the symbol timing and synchronize to the carrier frequency. When synchronizing to the carrier frequency, the mobile station needs to correct any erroneous frequency offsets that are present, and perform symbol timing alignment with the frame structure from the base station. In addition, in the presence of multiple cells, the mobile station also needs to distinguish a particular cell on the basis of a cell ID. Thus, a typical cell search procedure consists of determining the timing alignment, correcting the frequency offset, obtaining the correct cell ID, and the absolute frame number reference.

## LTE Cell Search

Cell search in LTE uses two synchronization sequences, the PSS (Primary Synchronization Sequence) and the SSS (Secondary Synchronization Sequence) in the 7th and 6th OFDM symbols respectively, of the 1st and 6th subframe of every 10 ms frame. This is depicted in Figure 1. Note that this design is for FDD systems only. In systems operating in TDD mode, the PSS and SSS are placed in a different position within the frame.

Figure 1 : Frame structure for PSS and SSS in LTE

504 unique cell identities are supported in LTE, and this is further divided into 168 cell identity groups, with three cell identities within each group. As a result, three PSSs are used to provide the cell identity within a cell identity group. For a given cell, the PSS always contains the same information, and hence can be considered to be repeated every 5 ms. This enables the user to determine the timing of the cell and also correct the frequency offset within 5 ms after initiating the cell search. The position of the PSS gives the location of the SSS, which occupies the previous OFDM symbol. The sequences used in the SSS are interleaved in a different manner in the two subframes within a frame to obtain the correct frame timing and determine the particular cell identity group from the 168 possible alternatives.

The three PSSs are length-63 Zadoff-Chu sequences extended with 5 zeros at the edges and mapped to the center 73 subcarriers. The DC subcarrier is not transmitted, therefore, only 62 elements of the Zadoff-Chu sequences are used. The SSS is composed of two length 31 m-sequences interleaved in a specific manner in the two subframes (swapped in the frequency domain) to determine the correct frame timing. Since 31 cyclic shifts of an m-sequence are possible and all of them are orthogonal to one another, the SSS for a cell identity group is composed of two specific cyclic shifts of the two m-sequences.

## NB LTE Cell Search

In order to maximize synergies with the LTE design, NB LTE relies on a time remapping of the LTE resources as described in [2]. However, a time domain remapping of the PSS and SSS would result in the synchronization sequences spread over multiple subframes. This may not be desirable because of the need for a contiguous sequence in the time domain to simplify receiver processing. Therefore, cell search in NB LTE follows the same principle as in LTE, but with suitable modifications in the design of the different sequences to improve the cell search capability. The structure of the cell synchronization sequences used in NB LTE is provided in Figure 2.

Figure 2 : Proposed frame structure PSS and SSS.

Depicted in Figure 2 are the

- Primary Synchronization Sequence (PSS): Three Primary Synchronization sequences are used as in LTE to determine the three cell identities within a group. The PSS spans 6 OFDM symbols and is used to determine the subframe timing as well as correcting the frequency offset. Note that in this design, the PSS is contiguous in time.

- Secondary Synchronization Sequence (SSS): The Secondary Synchronization Sequence spans 6 OFDM symbols and is used to determine the cell identity group and the M-frame timing. In order to support the same number of cell identity groups as in LTE, 168 different SSS are designed.

From the design, it can be seen that the PSS and SSS are repeated on average every 15 ms and occur 4 times within a 60 ms M-frame. Specifically, in the 2nd and 7th M-subframes, the synchronization sequences are present in the 3rd subframe and in the 4th and 9th M-subframes, the synchronization sequences are present in the 6th subframe. In the subframes containing the synchronization sequences, the PSS occupies the last 6 OFDM symbols, and the SSS occupies the 2nd to 7th OFDM symbol. While the 1st and 8th OFDM symbols can be used to carry other information, the design allows the 1st OFDM symbol to be left vacant in order to support legacy LTE (when NB-LTE is deployed within an LTE carrier).

With respect to LTE, the design of the synchronization sequences in NB LTE are very much similar in the sense that each instance of PSS and SSS occupies 72 subcarriers both in LTE as well as in NB LTE (one OFDM symbol contains 12 subcarriers in NB-LTE), except for the fact that the PSS/SSS are repeated 4 times within an M-frame compared to 2 times repetition within a frame in LTE. This is done to enable faster synchronization as well as to reduce memory consumption at the user terminal. However, because of the 4 times repetition, a slightly modified design is required for the SSS to obtain the M-frame timing, which we describe in the next section.

# Structure of PSS and SSS

In LTE, the structure of the PSS allows a low complexity design of the timing and frequency offset estimator, and the SSS is designed to obtain frame timing and enable support for 168 unique cell identity groups. We adopt the same design guidelines for NB-LTE.

## Primary Synchronization Sequence Design

Three different PSSs are specified in NB-LTE. In LTE PSS design, a certain number of frequency hypotheses are used for each PSS in order to determine the symbol timing and a coarse estimation of the frequency offset. Adopting the same design in NB LTE would give rise to a higher receiver processing complexity due to the use of multiple frequency hypotheses. In order to cater to this problem, a differentially encoded Zadoff-Chu sequence is proposed for the PSS. The base sequence is a length-71 Zadoff sequence, and three different sequences are obtained using three different roots u. Thus, we have

cun=e-jπunn+1N,      u=1,13,27, n={0,1,2,…,70}

The base sequence cu(n) is differentially encoded, upsampled and then filtered so that it is restricted to the 200 kHz bandwidth. The differentially encoded sequence du(n) is given as

dun+1=duncun,     n=0,1,2,…,70,du0=1,

The upsampled and filtered sequence pun corresponding to a root u is obtained from dun as follows

pun=IFFT(FFTdun,72,720)

Note that the above expression for pu(n) captures both the upsampling and filtering effect. The sequence pu(n) is then padded with 48 zeros to obtain a length-768 sequence tun. In the transmission phase, the length-768 sequence is first divided into 6 length-128 sub sequences, and cyclic prefix is added to each of the 6 sub sequences to give the PSS. The specific design allows backward compatibility with legacy LTE system. Note that each of the sub sequences constitutes one OFDM symbol, so that the PSS occupies 6 OFDM symbols.

A comparison with LTE reveals that the frequency of occurrence of the PSS in NB LTE gives a slightly larger overhead for the PSS in LTE. Specifically, the synchronization sequences used in LTE occupy 1.43 % of the total transmission resource, and the ones used in NB LTE occupy approximately 2.8 % of the total resource. This extra overhead comes with the advantage of reducing the synchronization time as well as the memory consumption by a factor of 2, leading to improved battery life and lower device cost.

## Secondary Synchronization Sequence Design

The Secondary Synchronization Sequence (SSS) is designed in the frequency domain and occupies 72 subcarriers corresponding to 6 OFDM symbols. The SSS is composed of two length 31 Zadoff Chu sequences padded with 5 zeros at the beginning and end. The two sequences are chosen in order to provide support for 168 unique cell identity groups. The reason for using Zadoff-Chu sequences in NB LTE compared to m-sequences in LTE is to provide robustness to the “phantom cell effect”, which arises due to the presence of a common sequence for two different cell identity groups.

Since the PSS/SSS occur 4 times within an M-frame, the LTE design of the SSS cannot be used to provide the frame timing. This is because of the special interleaving structure that can determine only 2 positions, whereas we need to determine 4 positions. Therefore, a scrambling code is used on top of the two Zadoff-Chu sequences in order to provide information about the frame timing. 4 scrambling codes are required to determine the 4 locations within the M-frame, which can be leveraged to obtain the correct frame timing.

Specifically, the SSS is given as sp,q(n)=apnbq(n), where p={0,1,…,167} denotes the cell identity group and q={0,1,2,3} determines the location of the SSS, i.e., the number of SSS in the M-frame that have occurred before the current SSS. We have

apn=0,                    n=0-4,67-71

=ap1modn-kp-5 , 31,      n=5,6,…,35

=ap2modn-kp-36 , 31,      n=36,37,…,66

ap1n=e-jπm1(p)nn+1N,                  n=0,1,…,30

ap2(n)=e-jπm2(p)nn+1N,                  n=0,1,…,30

and

bqn=bmodn-lq,63      n=0,1,…62,  q=0,1,2,3,

l0=0,l1=3,l2=7,l3=1

bn+6=modbn+bn+1,2,        n=0,1,…55,

b0=1,bm=0,     m={1,2,3,4,5}

Note that ap(n) is composed of two Zadoff Chu sequences and determines the cell identity group.  The cyclic shifts m1(p) and m2(p) are chosen to provide a specific cell identity and the cyclic shift kp is used to remove the “phantom cell effect” present in LTE design. bq(n) is the scrambling sequence composed of cyclic shifts of a base sequence b(n) and is used to indicate the SSS location within an M-frame in order to obtain the frame timing. Note that the cyclic shift lq is dependent on q. The values of m1(p),m2(p) and kp for a specific p are provided in Annex A.

# Conclusions

A cell search and synchronization signal design for NB LTE has been proposed. The design is very much in line with the cell search design in LTE and follows similar guidelines, along with suitable modifications to optimize the solution for IoT applications.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-150779, “Narrowband LTE – Concept description”, source Ericsson LM; Nokia Networks; AT&T; Alcatel-Lucent; Alcatel-Lucent Shanghai Bell; Intel; Sierra Wireless; Sequans, GERAN#67.

GP-150803, “Reuse of LTE L2/L3 layers for Cellular IoT”, source Alcatel-Lucent; Alcatel-Shanghai Bell; Ericsson LM; MediaTek Inc.; Panasonic Corporation; Sierra Wireless; S.A.; ZTE Corporation; Nokia Networks; Sequans, GERAN#67.

# Annex A : Secondary Synchronization Sequence Design

The values of the roots of the Zadoff-Chu sequences and their cyclic shifts to be used for the SSS of a particular cell identity group are provided in Table 1.

Table 1: (m1,m2, k) for a specific cell identity group p

| p | m1(p) | m2(p) | kp | p | m1(p) | m2(p) | kp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 0 | 84 | 1 | 5 | 12 |
| 1 | 2 | 3 | 0 | 85 | 2 | 6 | 12 |
| 2 | 3 | 4 | 0 | 86 | 3 | 7 | 12 |
| 3 | 4 | 5 | 0 | 87 | 4 | 8 | 12 |
| 4 | 5 | 6 | 0 | 88 | 5 | 9 | 12 |
| 5 | 6 | 7 | 0 | 89 | 6 | 10 | 12 |
| 6 | 7 | 8 | 0 | 90 | 7 | 11 | 12 |
| 7 | 8 | 9 | 0 | 91 | 8 | 12 | 12 |
| 8 | 9 | 10 | 0 | 92 | 9 | 13 | 12 |
| 9 | 10 | 11 | 0 | 93 | 10 | 14 | 12 |
| 10 | 11 | 12 | 0 | 94 | 11 | 15 | 12 |
| 11 | 12 | 13 | 0 | 95 | 12 | 16 | 12 |
| 12 | 13 | 14 | 0 | 96 | 13 | 17 | 12 |
| 13 | 14 | 15 | 0 | 97 | 14 | 18 | 12 |
| 14 | 15 | 16 | 0 | 98 | 15 | 19 | 12 |
| 15 | 16 | 17 | 0 | 99 | 16 | 20 | 12 |
| 16 | 17 | 18 | 0 | 100 | 17 | 21 | 12 |
| 17 | 18 | 19 | 0 | 101 | 18 | 22 | 12 |
| 18 | 19 | 20 | 0 | 102 | 19 | 23 | 12 |
| 19 | 20 | 21 | 0 | 103 | 20 | 24 | 12 |
| 20 | 21 | 22 | 0 | 104 | 21 | 25 | 12 |
| 21 | 22 | 23 | 0 | 105 | 22 | 26 | 12 |
| 22 | 23 | 24 | 0 | 106 | 23 | 27 | 12 |
| 23 | 24 | 25 | 0 | 107 | 24 | 28 | 12 |
| 24 | 25 | 26 | 0 | 108 | 25 | 29 | 12 |
| 25 | 26 | 27 | 0 | 109 | 26 | 30 | 12 |
| 26 | 27 | 28 | 0 | 110 | 1 | 6 | 16 |
| 27 | 28 | 29 | 0 | 111 | 2 | 7 | 16 |
| 28 | 29 | 30 | 0 | 112 | 3 | 8 | 16 |
| 29 | 1 | 3 | 4 | 113 | 4 | 9 | 16 |
| 30 | 2 | 4 | 4 | 114 | 5 | 10 | 16 |
| 31 | 3 | 5 | 4 | 115 | 6 | 11 | 16 |
| 32 | 4 | 6 | 4 | 116 | 7 | 12 | 16 |
| 33 | 5 | 7 | 4 | 117 | 8 | 13 | 16 |
| 34 | 6 | 8 | 4 | 118 | 9 | 14 | 16 |
| 35 | 7 | 9 | 4 | 119 | 10 | 15 | 16 |
| 36 | 8 | 10 | 4 | 120 | 11 | 16 | 16 |
| 37 | 9 | 11 | 4 | 121 | 12 | 17 | 16 |
| 38 | 10 | 12 | 4 | 122 | 13 | 18 | 16 |
| 39 | 11 | 13 | 4 | 123 | 14 | 19 | 16 |
| 40 | 12 | 14 | 4 | 124 | 15 | 20 | 16 |
| 41 | 13 | 15 | 4 | 125 | 16 | 21 | 16 |
| 42 | 14 | 16 | 4 | 126 | 17 | 22 | 16 |
| 43 | 15 | 17 | 4 | 127 | 18 | 23 | 16 |
| 44 | 16 | 18 | 4 | 128 | 19 | 24 | 16 |
| 45 | 17 | 19 | 4 | 129 | 20 | 25 | 16 |
| 46 | 18 | 20 | 4 | 130 | 21 | 26 | 16 |
| 47 | 19 | 21 | 4 | 131 | 22 | 27 | 16 |
| 48 | 20 | 22 | 4 | 132 | 23 | 28 | 16 |
| 49 | 21 | 23 | 4 | 133 | 24 | 29 | 16 |
| 50 | 22 | 24 | 4 | 134 | 25 | 30 | 16 |
| 51 | 23 | 25 | 4 | 135 | 1 | 7 | 20 |
| 52 | 24 | 26 | 4 | 136 | 2 | 8 | 20 |
| 53 | 25 | 27 | 4 | 137 | 3 | 9 | 20 |
| 54 | 26 | 28 | 4 | 138 | 4 | 10 | 20 |
| 55 | 27 | 29 | 4 | 139 | 5 | 11 | 20 |
| 56 | 28 | 30 | 4 | 140 | 6 | 12 | 20 |
| 57 | 1 | 4 | 8 | 141 | 7 | 13 | 20 |
| 58 | 2 | 5 | 8 | 142 | 8 | 14 | 20 |
| 59 | 3 | 6 | 8 | 143 | 9 | 15 | 20 |
| 60 | 4 | 7 | 8 | 144 | 10 | 16 | 20 |
| 61 | 5 | 8 | 8 | 145 | 11 | 17 | 20 |
| 62 | 6 | 9 | 8 | 146 | 12 | 18 | 20 |
| 63 | 7 | 10 | 8 | 147 | 13 | 19 | 20 |
| 64 | 8 | 11 | 8 | 148 | 14 | 20 | 20 |
| 65 | 9 | 12 | 8 | 149 | 15 | 21 | 20 |
| 66 | 10 | 13 | 8 | 150 | 16 | 22 | 20 |
| 67 | 11 | 14 | 8 | 151 | 17 | 23 | 20 |
| 68 | 12 | 15 | 8 | 152 | 18 | 24 | 20 |
| 69 | 13 | 16 | 8 | 153 | 19 | 25 | 20 |
| 70 | 14 | 17 | 8 | 154 | 20 | 26 | 20 |
| 71 | 15 | 18 | 8 | 155 | 21 | 27 | 20 |
| 72 | 16 | 19 | 8 | 156 | 22 | 28 | 20 |
| 73 | 17 | 20 | 8 | 157 | 23 | 29 | 20 |
| 74 | 18 | 21 | 8 | 158 | 24 | 30 | 20 |
| 75 | 19 | 22 | 8 | 159 | 1 | 8 | 24 |
| 76 | 20 | 23 | 8 | 160 | 2 | 9 | 24 |
| 77 | 21 | 24 | 8 | 161 | 3 | 10 | 24 |
| 78 | 22 | 25 | 8 | 162 | 4 | 11 | 24 |
| 79 | 23 | 26 | 8 | 163 | 5 | 12 | 24 |
| 80 | 24 | 27 | 8 | 164 | 6 | 13 | 24 |
| 81 | 25 | 28 | 8 | 165 | 7 | 14 | 24 |
| 82 | 26 | 29 | 8 | 166 | 8 | 15 | 24 |
| 83 | 27 | 30 | 8 | 167 | 9 | 16 | 24 |