# Pseudo CR 45.820 – Narrowband LTE Cell search design

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62.

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

In GERAN#67, a new system design, i.e., NB LTE, based on the current LTE system but use only 200 KHz bandwidth is proposed. GP-150785 describes the design for downlink synchronization channel in NB LTE. This has been evaluated and is seen can meet the performance requirement.

## 1.3	Summary of change

The M-PSCH design of NB LTE

# pCR to 3GPP TR 45.820-v1.4.0

### 7.4.2.3.2 Downlink common control channel

Omitted

### 7.4.2.3.3	 Synchronization channel

In NB LTE, the M-PSCH is used by the MS to time and frequency synchronization to the network, as well as for the MS to obtain the correct cell ID. The cell search in NB-LTE follows the same principle as in LTE, but with suitable modifications in the design of the M-PSCH to improve the cell search capability.

The structure of the cell synchronization sequences used in NB LTE is provided in Figure 7.4.2.3-8.

Figure 7.4.2.3-8: Frame structure PSS and SSS.

Depicted in Figure 7.4.2.3-8 are the

- Primary Synchronization Sequence (PSS): Three Primary Synchronization sequences are used as in LTE to determine the three cell identities within a group. The PSS spans 6 OFDM symbols and is used to determine the subframe timing as well as correcting the frequency offset. Note that in this design, the PSS is contiguous in time.

- Secondary Synchronization Sequence (SSS): The Secondary Synchronization Sequence spans 6 OFDM symbols and is used to determine the cell identity group and the M-frame timing. In order to support the same number of cell identity groups as in LTE, 168 different SSS are designed.

From the design, it can be seen that the PSS and SSS are repeated on average every 15 ms and occur 4 times within a 60 ms M-frame. Specifically, in the 2nd and 7th M-subframes, the synchronization sequences are present in the 3rd subframe and in the 4th and 9th M-subframes, the synchronization sequences are present in the 6th subframe. In the subframes containing the synchronization sequences, the PSS occupies the last 6 OFDM symbols, and the SSS occupies the 2nd to 7th OFDM symbol. While the 1st and 8th OFDM symbols can be used to carry other information, the design allows the 1st OFDM symbol to be left vacant in order to support legacy LTE (when NB-LTE is deployed within an LTE carrier).

With respect to LTE, the design of the synchronization sequences in NB LTE are very much similar in the sense that each instance of PSS and SSS occupies 72 subcarriers both in LTE as well as in NB LTE (one OFDM symbol contains 12 subcarriers in NB-LTE), except for the fact that the PSS/SSS are repeated 4 times within an M-frame compared to 2 times repetition within a frame in LTE. This is done to enable faster synchronization as well as to reduce memory consumption at the user terminal. However, because of the 4 times repetition, a slightly modified design is required for the SSS to obtain the M-frame timing, which we describe as follows.

Three different PSSs are specified in NB-LTE. The base sequence is a length-71 Zadoff sequence, and three different sequences are obtained using three different roots u. Thus, we have

cun=e-jπunn+1N,      u=1,13,27, n={0,1,2,…,70}

The base sequence cu(n) is differentially encoded, upsampled and then filtered so that it is restricted to the 200 kHz bandwidth. The differentially encoded sequence du(n) is given as

dun+1=duncun,     n=0,1,2,…,70,du0=1,

The upsampled and filtered sequence pun corresponding to a root u is obtained from dun as follows

pun=IFFT(FFTdun,72,720)

Note that the above expression for pu(n) captures both the upsampling and filtering effect. The sequence pu(n) is then padded with 48 zeros to obtain a length-768 sequence tun. In the transmission phase, the length-768 sequence is first divided into 6 length-128 sub sequences, and cyclic prefix is added to each of the 6 sub sequences to give the PSS. The specific design allows backward compatibility with legacy LTE system. Note that each of the sub sequences constitutes one OFDM symbol, so that the PSS occupies 6 OFDM symbols.

The Secondary Synchronization Sequence (SSS) is designed in the frequency domain and occupies 72 subcarriers corresponding to 6 OFDM symbols. The SSS is composed of two length 31 Zadoff Chu sequences padded with 5 zeros at the beginning and end. The two sequences are chosen in order to provide support for 168 unique cell identity groups.  A scrambling code is used on top of the two Zadoff-Chu sequences in order to provide information about the frame timing. Four scrambling codes are required to determine the 4 locations within the M-frame, which can be leveraged to obtain the correct frame timing.

Specifically, the SSS is given as sp,q(n)=apnbq(n), where p={0,1,…,167} denotes the cell identity group and q={0,1,2,3} determines the location of the SSS, i.e., the number of SSS in the M-frame that have occurred before the current SSS. We have

apn=0,                    n=0-4,67-71

=ap1modn-kp-5 , 31,      n=5,6,…,35

=ap2modn-kp-36 , 31,      n=36,37,…,66

ap1n=e-jπm1(p)nn+1N,                  n=0,1,…,30

ap2(n)=e-jπm2(p)nn+1N,                  n=0,1,…,30

and

bqn=bmodn-lq,63      n=0,1,…62,  q=0,1,2,3,

l0=0,l1=3,l2=7,l3=1

bn+6=modbn+bn+1,2,        n=0,1,…55,

b0=1,bm=0,     m={1,2,3,4,5}

Note that ap(n) is composed of two Zadoff Chu sequences and determines the cell identity group, and bq(n) is the scrambling sequence composed of cyclic shifts of a base sequence b(n) and is used to indicate the SSS location within an M-frame in order to obtain the frame timing. Note that the cyclic shift lq is dependent on q. The values of m1(p),m2(p) and kp for a specific p are provided in Table 7.4.2.3-1.

Table 7.4.2.3-1: (m1,m2, k) for a specific cell identity group p

| p | m1(p) | m2(p) | kp | p | m1(p) | m2(p) | kp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 0 | 84 | 1 | 5 | 12 |
| 1 | 2 | 3 | 0 | 85 | 2 | 6 | 12 |
| 2 | 3 | 4 | 0 | 86 | 3 | 7 | 12 |
| 3 | 4 | 5 | 0 | 87 | 4 | 8 | 12 |
| 4 | 5 | 6 | 0 | 88 | 5 | 9 | 12 |
| 5 | 6 | 7 | 0 | 89 | 6 | 10 | 12 |
| 6 | 7 | 8 | 0 | 90 | 7 | 11 | 12 |
| 7 | 8 | 9 | 0 | 91 | 8 | 12 | 12 |
| 8 | 9 | 10 | 0 | 92 | 9 | 13 | 12 |
| 9 | 10 | 11 | 0 | 93 | 10 | 14 | 12 |
| 10 | 11 | 12 | 0 | 94 | 11 | 15 | 12 |
| 11 | 12 | 13 | 0 | 95 | 12 | 16 | 12 |
| 12 | 13 | 14 | 0 | 96 | 13 | 17 | 12 |
| 13 | 14 | 15 | 0 | 97 | 14 | 18 | 12 |
| 14 | 15 | 16 | 0 | 98 | 15 | 19 | 12 |
| 15 | 16 | 17 | 0 | 99 | 16 | 20 | 12 |
| 16 | 17 | 18 | 0 | 100 | 17 | 21 | 12 |
| 17 | 18 | 19 | 0 | 101 | 18 | 22 | 12 |
| 18 | 19 | 20 | 0 | 102 | 19 | 23 | 12 |
| 19 | 20 | 21 | 0 | 103 | 20 | 24 | 12 |
| 20 | 21 | 22 | 0 | 104 | 21 | 25 | 12 |
| 21 | 22 | 23 | 0 | 105 | 22 | 26 | 12 |
| 22 | 23 | 24 | 0 | 106 | 23 | 27 | 12 |
| 23 | 24 | 25 | 0 | 107 | 24 | 28 | 12 |
| 24 | 25 | 26 | 0 | 108 | 25 | 29 | 12 |
| 25 | 26 | 27 | 0 | 109 | 26 | 30 | 12 |
| 26 | 27 | 28 | 0 | 110 | 1 | 6 | 16 |
| 27 | 28 | 29 | 0 | 111 | 2 | 7 | 16 |
| 28 | 29 | 30 | 0 | 112 | 3 | 8 | 16 |
| 29 | 1 | 3 | 4 | 113 | 4 | 9 | 16 |
| 30 | 2 | 4 | 4 | 114 | 5 | 10 | 16 |
| 31 | 3 | 5 | 4 | 115 | 6 | 11 | 16 |
| 32 | 4 | 6 | 4 | 116 | 7 | 12 | 16 |
| 33 | 5 | 7 | 4 | 117 | 8 | 13 | 16 |
| 34 | 6 | 8 | 4 | 118 | 9 | 14 | 16 |
| 35 | 7 | 9 | 4 | 119 | 10 | 15 | 16 |
| 36 | 8 | 10 | 4 | 120 | 11 | 16 | 16 |
| 37 | 9 | 11 | 4 | 121 | 12 | 17 | 16 |
| 38 | 10 | 12 | 4 | 122 | 13 | 18 | 16 |
| 39 | 11 | 13 | 4 | 123 | 14 | 19 | 16 |
| 40 | 12 | 14 | 4 | 124 | 15 | 20 | 16 |
| 41 | 13 | 15 | 4 | 125 | 16 | 21 | 16 |
| 42 | 14 | 16 | 4 | 126 | 17 | 22 | 16 |
| 43 | 15 | 17 | 4 | 127 | 18 | 23 | 16 |
| 44 | 16 | 18 | 4 | 128 | 19 | 24 | 16 |
| 45 | 17 | 19 | 4 | 129 | 20 | 25 | 16 |
| 46 | 18 | 20 | 4 | 130 | 21 | 26 | 16 |
| 47 | 19 | 21 | 4 | 131 | 22 | 27 | 16 |
| 48 | 20 | 22 | 4 | 132 | 23 | 28 | 16 |
| 49 | 21 | 23 | 4 | 133 | 24 | 29 | 16 |
| 50 | 22 | 24 | 4 | 134 | 25 | 30 | 16 |
| 51 | 23 | 25 | 4 | 135 | 1 | 7 | 20 |
| 52 | 24 | 26 | 4 | 136 | 2 | 8 | 20 |
| 53 | 25 | 27 | 4 | 137 | 3 | 9 | 20 |
| 54 | 26 | 28 | 4 | 138 | 4 | 10 | 20 |
| 55 | 27 | 29 | 4 | 139 | 5 | 11 | 20 |
| 56 | 28 | 30 | 4 | 140 | 6 | 12 | 20 |
| 57 | 1 | 4 | 8 | 141 | 7 | 13 | 20 |
| 58 | 2 | 5 | 8 | 142 | 8 | 14 | 20 |
| 59 | 3 | 6 | 8 | 143 | 9 | 15 | 20 |
| 60 | 4 | 7 | 8 | 144 | 10 | 16 | 20 |
| 61 | 5 | 8 | 8 | 145 | 11 | 17 | 20 |
| 62 | 6 | 9 | 8 | 146 | 12 | 18 | 20 |
| 63 | 7 | 10 | 8 | 147 | 13 | 19 | 20 |
| 64 | 8 | 11 | 8 | 148 | 14 | 20 | 20 |
| 65 | 9 | 12 | 8 | 149 | 15 | 21 | 20 |
| 66 | 10 | 13 | 8 | 150 | 16 | 22 | 20 |
| 67 | 11 | 14 | 8 | 151 | 17 | 23 | 20 |
| 68 | 12 | 15 | 8 | 152 | 18 | 24 | 20 |
| 69 | 13 | 16 | 8 | 153 | 19 | 25 | 20 |
| 70 | 14 | 17 | 8 | 154 | 20 | 26 | 20 |
| 71 | 15 | 18 | 8 | 155 | 21 | 27 | 20 |
| 72 | 16 | 19 | 8 | 156 | 22 | 28 | 20 |
| 73 | 17 | 20 | 8 | 157 | 23 | 29 | 20 |
| 74 | 18 | 21 | 8 | 158 | 24 | 30 | 20 |
| 75 | 19 | 22 | 8 | 159 | 1 | 8 | 24 |
| 76 | 20 | 23 | 8 | 160 | 2 | 9 | 24 |
| 77 | 21 | 24 | 8 | 161 | 3 | 10 | 24 |
| 78 | 22 | 25 | 8 | 162 | 4 | 11 | 24 |
| 79 | 23 | 26 | 8 | 163 | 5 | 12 | 24 |
| 80 | 24 | 27 | 8 | 164 | 6 | 13 | 24 |
| 81 | 25 | 28 | 8 | 165 | 7 | 14 | 24 |
| 82 | 26 | 29 | 8 | 166 | 8 | 15 | 24 |
| 83 | 27 | 30 | 8 | 167 | 9 | 16 | 24 |