DL System Level Simulation Results of GERAN CIoT Solutions: EC-GSM

1.  Introduction

A study item named “Cellular IoT” was approved in GERAN#62 for evaluating how to support low throughput and low complexity machine type communications [1].  Several proposals have been discussed under the categories of an evolved low-complexity EGPRS (EC-GSM) and clean slate solutions (NB M2M and NB OFDMA).  The evaluation methodology including the system assumptions, parameters and traffic models were captured in the draft TR 45.820 [2].

In this document, we present some DL system level simulation results for EC-GSM solution for GERAN CIoT systems. The full-buffer traffic model is assumed to verify whether the requirement of 160 bps throughput at the equivalent of the SNDCP layer in [2] can be satisfied.  Furthermore, the CIoT traffic model specified in [2] was employed in the simulations and system capacity and latency results were obtained.

2.  Simulation Setup and Parameters

The major system level simulation assumptions and parameters are listed in Table 1.

Table 1: Simulation parameters and assumptions.

| Parameter | Assumption |
| --- | --- |
| Cellular Layout | Hexagonal grid, 19 cell sites, 3 sectors per site |
| Frequency band | 900 MHz |
| Inter site distance | 1732 m |
| Frequency Reuse | 4/12 |
| Frequency Hopping | Off |
| Modulation coding schemes | MCS 1-4 |
| Doppler frequency | 1 Hz |
| User distribution | Devices dropped uniformly in each sector |
| BS transmit power per 200 KHz (at the antenna connector) | 43 dBm |
| Pathloss model | L=I + 37.6log10(R), R in kilometers I=120.9 dB for the 900 MHz band |
| Shadowing standard deviation | 8 dB |
| Correlation distance of Shadowing | 110 m |
| Shadowing correlation | 0.5 Between cell sites/ 1 Between sectors of the same cell site |
| Antenna pattern (horizontal) | See table 5-7, 3GPP TR 45.914, 65° H-plane |
| BS Antenna gain | 18 dBi |
| MS Antenna gain | -4 dBi |
| BS cable loss | 3 dB |
| Building Penetration Loss | Based on TR 45.820 Scenario 1 with correlation coefficient = 0.5 |
| Link to system mapping | Proposed in [3] |
| Noise figure | 5 dB |

In addition to the CIoT traffic models specified in [2], full-buffer traffic and proportional fair (PF) scheduling are considered in the simulations. The choice of full-buffer traffic model and uniform distribution of larger number (20, 30, 40) of devices per sector, although not a part of the agreed evaluation methodology, is primarily motivated by the aim of facilitating an easier analysis and better understanding of the performance of the EC-GSM design at the system level considering the minimum MAC throughput requirement for a challenging scenario w.r.t. system loading (when compared to practical traffic loading from CIoT traffic).  This assumes 160 bps at the equivalent of the SNDCP layer according to the requirements of [2] and approximately an additional 40 bps overhead for the physical layer/MAC overhead. The link-to-system mapping procedure proposed in [3] has been employed for the physical layer abstraction. Perfect time and frequency synchronization and channel estimation are assumed in this study. Note that, in this work, we considered a frequency reuse factor of 4/12.

3. Simulation Setup

In this proposal, we considered an EC-GSM system with blind repetition and scheduling penalty. The number of blind repetitions considered were 2, 4, 8, and 16. Each of these repetitions were decoded assuming coherent energy combining. The PF scheduler considers the channel quality of the devices and determines the number of repetitions based on predefined channel quality thresholds.

4. System Level Simulations results

## 4.1	Building penetration loss

Scenario 1 of the building penetration loss specified in [2] with 0.5 correlation has been considered.  The penetration loss values were generated for the 5000 uniformly distributed devices in the cell area to obtain sufficient statistics for penetration loss distribution. However, for better illustration, in Figure 1 we present the CDFs of the two BPL scenarios of [2] with different correlation coefficients. From this figure, it can be seen that the penetration loss varies between 5 and 35 dB with a median value of 19.8 dB for Scenario 1 with 0.5 correlation.

Figure 1.  CDF of the building penetration loss for the scenarios given in [2].

## 4.2	Results with full buffer traffic model

### 4.2.1	Throughput

#### 4.2.1.1		Full Buffer PF scheduling

To further evaluate the performance, the DL MAC throughput CDFs for the full buffer traffic, are presented in Figure 2. We assume 200 bps as the target MAC throughput for the GERAN CIoT devices. The results indicate that almost 100% of the devices may be able to achieve the 200 bps throughput at the equivalent of the SNDCP layer when the system concurrently supports 20, 30 and 40 UEs. In all three cases, the percentage of devices using blind repetition is between 2% and 3%. The average sector throughput for different scenarios is presented in Table 2. Note that the system throughput in all three cases is comparable due to the fact the 4/12 frequency reuse pattern results in a significantly low interference. Hence, most of the users are experiencing good channel conditions and the diversity obtained by the increase in the number of users per sector does not impact the system performance.

Figure 2.  The device throughput CDFs of the EC-GSM system without penalization.

Table 2.  Average DL MAC throughput for the EC-GSM systems with full buffer traffic, but without penalization.

|  | 20 UEs | 30 UEs | 40 UEs |
| --- | --- | --- | --- |
| System Throughput | 129.5Kbps | 130.2 Kbps | 129.7 Kbps |
| Percentage of UEs using blind repetitions | 2.9 | 2.54 | 2.3 |

## 4.2	Results with traffic models according to evaluation framework

Now we evaluate the DL performance of the EC-GSM with respect to the traffic model described in [2] for system capacity evaluation. In this traffic model, it is assumed that the UL traffic model will consist of 80% MAR periodic reporting traffic, whereas the remaining traffic will come from the network command (NC) traffic model, i.e., the remaining 20%. However, since 50% of the MAR periodic reporting traffic will require ACK in the downlink and since only half of the DL NC traffic will result in a response in the UL, this means that the DL traffic model will consist of two categories: 1) 50% of the reports will be network command, whereas the remaining 50% will be generated as an acknowledgement to the UL MAR periodic reporting traffic [2].  In addition, according to [2], each transmitted report includes 65 bytes header overhead before the SNDCP layer as well as a 15 bytes SNDCP to MAC overhead. Since the ACK payload is assumed to be zero, this means that the ACK report size will be equal to 80 bytes. Similarly, the size of the NC traffic report with the overhead in the DL will be equal to 100 bytes since the payload size is fixed to 20 bytes as suggested in [2]. Note that since both, the MAR exception reporting and the NC, have the same periodicity, it follows that approximately 6.81 reports per second per sector in the UL corresponds to an average of 52547 UEs per sector[4]. Subsequently, since only 50% of the NC results in UL traffic and since only 50% of the UL MAR periodic reports are acknowledged, it follows that in the DL 5.448 reports per second per sector on average corresponds to 52547 UEs per sector, i.e., 6.81*0.8 reports per second per sector.

In the following simulations, we consider a maximum of 50 simultaneous UEs in the system. The DL offered traffic versus the carried traffic and the packet delay CDFs of this traffic model are presented in Figure 3 and 4, respectively. When evaluating the packet delays, we included both the random access delay, i.e., the delay associated with requesting the UL resources from the base station, as well as the synchronization delays, i.e., the delay associated with detecting and decoding the PSCH. However, we note that the random access delay is applied only to the NC traffic since the MAR periodic reporting traffic is only an acknowledgment which implies that the devices are already in the ready state. In addition, when evaluating the random access delays, it is assumed that each device can perform up to a maximum of 6 random access attempts and that a collision between two or more devices would result in the base station not being able to decode the access requests and subsequently a back off delay of 1.5 seconds for the colliding devices [4]. The scheduling mechanism discussed in Section 3 was employed here as well. We see a linear relationship between the offered traffic and the carried traffic up to 250K users per sector. This relationship indicates that all the offered traffic can be transmitted without increasing the buffer size. Furthermore, up to 240K users per sector, around 98% of the reports are delivered in less than 2 seconds (thus meeting the maximum 10 seconds delay requirement).

Figure 3.  The number of UEs per sector versus carried traffic in the traffic model simulation of [2].

Figure 4.  Report delay CDFs in traffic model simulations.

5. Summary

In this contribution, we have presented our DL system simulation results for the EC-GSM systems with full-buffer traffic and realistic CIoT traffic model. Our full buffer simulation results indicate that around 100% of the devices would be able to receive more than the 200 bps MAC throughput requirement for the frequency reuse factor 4/12. We supported blind repetitions and scheduling enhancements in the study however, the percentage of devices employed blind repetitions in the DL is around 2.5%. In addition, we considered the NC/MAR periodic reporting traffic model and evaluated the system performance while including both, the random access and synchronization delays. In our simulations, it was shown that, up to a load level of approximately 200K UEs per sector, around 98% of the reports were delivered with a delay of less than 2 seconds. Thus, EC-GSM systems satisfy the requirements put forward in the TR 45.820 with a huge margin in the DL.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TR 45.820 v1.2.1, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

GP-150835, “GERAN CIoT Link-to-System Mapping Methodology,” Intel Corporation, GERAN #66.

GP-150433 “EC-GSM, EC-RACH System Capacity Evaluation (update of GPC150189),” source Ericsson LM, GERAN #66.