pCR for text inclusion in TR 45.820 :

Annex E.3: Protocol overhead assumption

1. Introduction

This document provides text for improving the content of annex E.3 of the Technical Report 45.820 revision 1.4.0.

2. Reason for change

2.1. Current status

The TR describes assumptions and methodologies to be used for performance evaluation of the various candidate solutions. Parts of these inputs are in the core of the TR, parts are in annexes. Annex E.3 is about assumptions for header overhead.

This overhead is related to communication protocols that are above the equivalent of the SNDCP layer of legacy systems. The selected stack of protocol is assumed to be CoAP (Constrained Application Protocol)/DTLS (Datagram Transport Layer Security/UDP (User Datagram Protocol) /IP (Internet Protocol).

As a matter of fact, this protocol overhead is used only by user data packets going from or going to the Internet. Other massages that are transmitted over the air don’t rely on this higher layer protocol stack (see figure 1).

Figure 1: Protocol stacks for legacy GSM/GPRS architecture

2.2. Concerns

This protocol overhead is a real concern for the FS_IoT_LC study because:

it increases the amount of data that is carried over the air interface,

it increases the complexity of the modem,

whereas two of the four main objectives of the FS_IoT_LC are:

low complexity for the modem in MSs

high density of MSs.

It can be easily understand that keeping the protocol overhead over the air is a conservative assumption for GSM evolution candidate solutions.

The situation is different for clean slate candidate solutions because they aim at the definition of a new radio interface optimized for the IoT. Therefore, they could greatly benefit from removing this protocol overhead from their radio implementation. Nevertheless, the protocol overhead has to be kept on the interface between the RAN and the core network for compliance with legacy architectures.

2.3 Suggestion

As the current definition of the assumptions related to protocol overhead doesn't say anything about the way the protocol overhead has to be implemented, we suggest adding a last paragraph at the end of the E.3 subclause. This paragraph shall state clearly that the implementation of the protocol overhead is left open to candidate solutions.

We consider that doing this will open new opportunities for solution improvement.

3. Proposed text for annex E.3 of TR 45.820 v1.4.0

E.3	Assumptions for header overhead

The total packet size at the equivalent of the SNDCP layer (excluding equivalent of SNDCP layer overhead) is made up of the application payload size and header overhead of protocols below the application layer and above the equivalent of the SNDCP layer.

For the purpose of this study, the example protocol stack is assumed to be CoAP (Constrained Application Protocol)/DTLS (Datagram Transport Layer Security/UDP (User Datagram Protocol) /IP (Internet Protocol)

The assumptions for header sizes for the different protocols are summarized in Table E.2-3.

Table E.2-3: Assumptions for header overhead above the equivalent of the SNDCP layer

| Protocol | Size in bytes |  |
| --- | --- | --- |
| COAP | 4 |  |
| DTLS | 13 |  |
| UDP | 8 |  |
| IP | 40 (without IP header compression) | 4 (with IP header compression) |
| Total header size | 65 | 29 |

Start of Changes

This protocol overhead, which is needed by user data to access the Internet, is supposed to be carried on the radio interface unless candidate solutions can manage it differently. In that case, candidate solutions shall detail their implementation.

End of Changes