pCR for text inclusion in TR 45.820 :

C-UNB Physical Layer Design

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the downlink and uplink physical layer design of the C-UNB solution.

It’s a revision of pCR GP-150693, where all material dealing with performance evaluation has been removed (this material is to be included under "concept evaluation" subclauses).

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

#### Types of DL channels

The C-UNB implements two types of DL channels:

the ad'hoc micro-channel that carries user plane packets for a given MS,

the beacon channel that broadcasts system information in each sector.

Radio design of these two channels is detailed here under, with common or separate clauses when appropriate.

#### Bit rate

The downlink bit rate is set to 600bps for ad'hoc micro-channels. This value is low enough to have UNB transmissions. It is higher that the UL transmission rate to get a reduced transmission time of DL radio packets, hence reducing the reception time in devices.

The beacon channel, which transmits control information in DL, is rated at 8 000bps. This high bit rate reduces the listening time in MSs that is good for power consumption. To keep the MCL at maximum value, the transmit power of beacon channel is increased.

#### Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

#### Transmission power of ad'hoc micro-channel

##### Maximum power

The sensitivity of receivers in devices is slightly lower than receivers in base stations, because the reception bandwidth can't be as narrow as what is in base stations. The ad'hoc micro-channel bit rate is higher than in UL. Therefore it is proposed to set the maximum transmission power of an ad'hoc micro-channel to +34dBm.

##### Downlink power management

The transmission power of ad'hoc micro-channels is adjusted according to the strength of the received signal in uplink. This very simple power management can be implemented because every DL packet is triggered by a UL packet.

If we consider the regular distribution of mobile stations in a cell, the mean distance to the base station is 2/3 of the max distance. This mean value improves the link budget by about 5 dB and gives room for carrier aggregation in DL (see next subclause).

##### Ad'hoc micro-channel aggregation

The power spectral density (PSD) of a GSM/GPRS base station is an important limiting factor for C-UNB, because the total available power is split among all the ad'hoc micro-channels, which are transmitted simultaneously. The worst case is where all ad'hoc micro-channels are aimed at devices close to the cell border: each ad'hoc microchannel is set to its maximum transmit power, i.e. +34dBm. Given the PSD of +43dBm in GSM/GPRS base stations, the total number of ad'hoc micro-channels, which can be transmitted simultaneously, is 8.

If we consider the mean distance of devices, the mean transmit power is +29dBm. Hence, the mean number of DL micro-channels is about 25.

The values (i.e. 8 max and 25 mean) are reasonable for carrying the user plane traffic in downlink.

#### Tramsmission power of beacon channel

The transmission power for the beacon channel uses the full power available in BTS, i.e. +43dBm. It means that beacon channel is transmitted alone because it requires the full available power of a BTS, whereas ad'hoc micro-channels can be aggregated, as depicted in figure 7.4.2-1.

Figure 7.4.2-1: Ad'hoc micro-channel and beacon channels

#### Center frequency in DL

The center frequency of each ad'hoc micro-channel is chosen according to the actual frequencies used by the mobile station during uplink transmission. The precise center frequency of a given ad'hoc micro-channel is not known a priori. It is calculated for each downlink message (see figure 7.4.2-2) according to the actual frequencies used in UL.

This feature helps reducing the complexity and cost of devices, because no automatic frequency control is needed during reception in devices: the AFC is performed by the base station itself.

The center frequency of the beacon channel is precisely defined. In a given area, it is the center frequency of the 200kHz block allocated to CIoT.

Figure 7.4.2-2: Relationship between frequencies in UL and DL

#### Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 1200 Hz. Pulse shaping filter is given in figure 7.4.2-3, whereas figure 7.4.2-4 gives the corresponding spectrum mask.

This filter is an optimized low-pass filter for the sake of low complexity in mobile stations. For improved performance, it can be changed to a root-raised-cosine filter with ß set about 0.5 to 0.6.

Figure 7.4.2-3: Pulse shaping filter in DL

Figure 7.4.2-4: Spectrum mask of DL in C-UNB

#### Error correction code

Each MAC-PDU is protected by an Error Correction Code (ECC) of 32 bits. Given the max length of a MAC-PDU of 384 bits, this ECC adds a 1dB gain to the MCL.

### Uplink physical layer design

#### Bit rate

The uplink bit rate is set to 250bps. Compared to the 900MHz, this value sets the ultra narrow band characteristics of C-UNB. Depending on the protocol overhead, it allows meeting the nominal throughput of 160bps at SNDCP interface.

#### Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

#### Transmission power

The transmission power is a key parameter for IoT devices because it sets most part or the power consumption of devices. Considering the very good selectivity and dynamic of C-UNB receivers in base stations, the Tx power in uplink may be set always at maximum power. Nevertheless, Tx power is adapted according to coverage class in order to increase the battery life of devices that experience good propagation conditions. UL transmit power adaptation mechanism is given in radio resource management subclauses.

#### Uplink frequency

In Cellular IoT, the objective is to re-use the legacy 200kHz blocks for IoT traffic. In usual cellular system, the center frequency of a channel is defined precisely and thus it requires costly TCXO or complex frequency correction features.

In the case of C-UNB technology, the legacy frequency block is used to carry uplink and downlink "ad'hoc micro-channels" that are ultra narrow band, that is to say, that have drift ranges larger that the modulation bandwidth. For this reason the center frequency of an ad'hoc micro-channel is not precisely defined, provided it is within the 200kHz block allocated to CIoT. Upon UL transmission, a mobile station sets pseudo-randomly the center frequency for each UL PHY-PDU. The precise frequency detection and correction is done by the Software Defined Radio base station in uplink that acts as a kind of automatic frequency correction.

#### Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 500Hz. Pulse shaping filter is given in figure 7.4.3-1.

The obtained spectrum and spectrum mask are illustrated in figure 7.4.3-2.

This filter is an optimized low-pass filter for the sake of low complexity in mobile stations. For improved performance, it can be changed to a root-raised-cosine filter with ß set about 0.5 to 0.6.

Figure 7.4.3-1: Pulse shaping filter in UL

Figure 7.4.3-2: Actual spectrum and spectrum mask for UL

#### Error correction code (ECC)

A 16 bit Reed-Solomon ECC protects each MAC-PDU. Given the max length of a MAC-PDU of 388 bits, this ECC adds a 1dB gain to the MCL with a limited complexity in the device. The most complex part of the Reed-Solomon code (i.e. the decoding part) is performed in the base station.

End of Changes