# Pseudo CR 45.820 – Narrowband LTE Introduction (Update of GP150953)

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62.

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

In GERAN#67, a new system design, i.e., NB-LTE, based on the current LTE system but use only 200 kHz bandwidth is proposed in GP-150779.

## 1.3	Summary of change

The introduction text of NB LTE design.

# pCR to 3GPP TR 45.820-v1.4.0

# 7.4	Narrow Band LTE (NB-LTE)

## 7.4.1	General

Narrowband LTE (NB-LTE) aims for a great degree of synergy with LTE. The entire system occupies only one LTE Physical Resource Block (PRB) and is operable within 200 kHz bandwidth. The downlink is based on orthogonal frequency division multiple access (OFDMA) and has the same subcarrier spacing, OFDM symbol duration, slot format, slot duration, and subframe duration as LTE.

Similar to LTE, uplink is based on SC-FDMA including single-tone transmission per MS as a special case of SC-FDMA. In addition, PAPR reduction techniques are considered for multi-tone transmission. Furthermore, the subcarrier spacing is 2.5 kHz, which is one sixth of the LTE subcarrier spacing. As a result, OFDM symbol duration, slot duration, and subframe duration are all 6 times of the LTE counterparts. The uplink system bandwidth is also 200 kHz, with a total transmission bandwidth of 180 kHz.

Further, depending on spectrum and resource availability, multiple narrowband carriers may be defined from the network’s perspective for both DL and UL to facilitate efficient system scalability with increase in the number of CIoT devices.

n addition to targeting a high synergy with the LTE physical layer, NB-LTE aims at reusing the higher user plane designs of LTE to a very large extent.

NB-LTE aims to maximize the re-use of LTE physical layer principles such as its DL numerology and the multiple access schemes. The coding, modulation, and rate matching framework already established in LTE is intended to be reused.

Changes to the UL numerology as well as to the PSS/SSS and PRACH structure are needed, aiming also here for commonality with LTE. For example, like LTE, Zadoff Chu sequences are used for PRACH and the synchronization channels, and cell search is based on the same cell identity grouping method.

The concept description is provided to show feasibility against the study item objectives, and could be seen as a baseline for further discussion and optimization in a potential work item phase. Simple design as well as synergy and compatibility with LTE are important optimization criteria.