pCR for text inclusion in TR 45.820 :

C-UNB Physical Layer Design

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the downlink and uplink physical layer design of the C-UNB solution.

It’s a revision of pCR GP-150999.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

#### Types of DL channels

The C-UNB implements two types of DL channels:

the ad'hoc micro-channel that carries packets for a given MS,

the beacon channel that broadcasts system information in each sector.

Radio design of these two channels is detailed here under, with common or separate clauses when appropriate.

#### Bit rate

The downlink bit rate is set to 600bps for ad'hoc micro-channels.

The beacon channel is rated at 8 000bps

#### Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

#### Transmission power of ad'hoc micro-channel in downlink

This subclause is FFS.

#### Center frequency in DL

The center frequency of each ad'hoc micro-channel is chosen according to the actual frequencies used by the mobile station during uplink transmission. The precise center frequency of a given ad'hoc micro-channel is not known a priori. It is calculated for each downlink message (see figure 7.4.2-1) according to the actual frequencies used in UL.

This feature helps reducing the complexity and cost of devices, because no automatic frequency control is needed during reception in devices: the AFC is performed by the base station itself.

The center frequency of the beacon channel is precisely defined. In a given area, it is the center frequency of the 200kHz block allocated to CIoT.

Figure 7.4.2-1: Relationship between frequencies in UL and DL

#### Pulse shaping and spectrum mask

This subclause is FFS.

#### Error correction code

Each MAC-PDU is protected by an Error Correction Code (ECC) of 32 bits.

### Uplink physical layer design

#### Bit rate

The uplink bit rate is set to 250bps.

#### Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

#### Transmission power

The transmission power is a key parameter for IoT devices because it sets most part or the power consumption of devices.

Its value is FFS.

#### Uplink frequency

In Cellular IoT, the objective is to re-use the legacy 200kHz blocks for IoT traffic. In usual cellular system, the center frequency of a channel is defined precisely and thus it requires frequency correction features.

In the case of C-UNB technology, the legacy frequency block is used to carry uplink and downlink "ad'hoc micro-channels" that are ultra narrow band, that is to say, that have drift ranges larger that the modulation bandwidth. For this reason the center frequency of an ad'hoc micro-channel is not precisely defined, provided it is within the 200kHz block allocated to CIoT. Upon UL transmission, a mobile station sets pseudo-randomly the center frequency for each UL PHY-PDU. The precise frequency detection and correction is done by the base station.

#### Pulse shaping and spectrum mask

This subclause is FFS.

#### Error correction code (ECC)

A 16 bit Reed-Solomon ECC protects each MAC-PDU in uplink.

End of Changes