TSG RAN ITU-R Ad Hoc	RT-100076

Source: 	Ericsson

Title: 	“LTE-Advanced” Overview for Rec. ITU-R M.[IMT.RSPEC]

[ITU member]

“LTE-Advanced” Overview for Rec. ITU-R M.[IMT.RSPEC]

# 1.	Overview of the Radio Interface Technology

## 1.1	Overview of the SRIT

The IMT-Advanced radio-interface specifications for LTE Rel-10 and beyond, also known as LTE-Advanced, are developed by a partnership of SDOs, named 3GPP.

LTE Rel-10&beyond (LTE-Advanced) is a Set of RITs (Radio Interface Technologies) consisting of one FDD RIT and one TDD RIT designed for operation in paired and unpaired spectrum, respectively. The TDD RIT is also known as TD-LTE Rel-10&beyond or TD-LTE-Advanced. The two RITs have been jointly developed, providing a high degree of commonality while, at the same time, allowing for optimization of each RIT with respect to its specific spectrum/duplex arrangement.

Both the FDD RIT and the TDD RIT individually meet all ITU IMT-Advanced requirements in all four test environments defined in all aspects of Services, Spectrum and Technical performance.

## 1.2	Overview of the Radio Interface Technology (RIT)

### 1.2.1	Overview of the FDD RIT

The FDD RIT is the evolution of LTE FDD. The FDD RIT uses Frequency-Division Duplex operation and therefore is applicable for operation with paired spectrum. Both full-duplex and half-duplex FDD are supported.

The FDD RIT meets all ITU IMT-Advanced requirements in all four test environments defined in all aspects of Services, Spectrum and Technical performance.

### 1.2.2	Overview of the TDD RIT

The TDD RIT, also known as TD-LTE-Advanced, is the evolution of TD-LTE. The TDD RIT uses Time-Division Duplex operation and therefore is applicable for operation with unpaired spectrum. The TDD RIT provides flexibility in terms of downlink-uplink resource allocation by supporting multiple uplink-downlink resource-allocation configurations that can be used to match different traffic scenarios. It is also designed to exploit the more extensive channel reciprocity inherent in case of TDD operation e.g. beamforming, and facilitates coexistence with TD-SCDMA as well as other TDD-based IMT-2000 technologies.

The TDD RIT meets all ITU IMT-Advanced requirements in all four test environments defined in all aspects of Services, Spectrum and Technical performance.

## 1.3	Overview of the System Aspects of the SRIT

The FDD and TDD RITs are high-data rate, low-latency, packet-optimized radio-access technologies, representing the evolution of the first releases of LTE FDD and TDD, respectively. The two RITs share many of the underlying structures to simplify implementation of dual-mode radio-access equipment. Transmission bandwidths up to 100 MHz are supported, yielding peak data rates up to roughly 3 Gbit/s in the downlink and 1.5 Gbit/s in the uplink.

The downlink transmission scheme is based on conventional OFDM with a cyclic prefix to provide a high degree of robustness against channel frequency selectivity while still allowing for low-complexity receiver implementations also at very large bandwidths. OFDM is also suitable when providing multimedia broadcast/multicast services (MBMS) using single-frequency transmission across multiple cells.

The uplink transmission scheme is based on DFT-spread OFDM (DFTS-OFDM). The use of DFTS-OFDM transmission for the uplink is motivated by the lower peak-to-average ratio of the transmitted signal compared to conventional OFDM. This allows for more efficient usage of the power amplifier at the terminal, which translates into an increased coverage and/or reduced terminal power consumption. The uplink numerology is aligned with the downlink numerology.

Channel coding is based on rate-1/3 Turbo coding and is complemented by Hybrid-ARQ with soft combining to handle decoding errors at the receiver side. Data modulation supports QPSK, 16QAM, and 64QAM.

The FDD and TDD RITs support bandwidths ranging from approximately 1.4 MHz up to more than 100 MHz. Carrier aggregations, i.e. the simultaneous transmission of multiple components carries in parallel to/from the same terminal, is used to support bandwidths larger than 20 MHz in both the downlink and uplink transmission directions. Component carriers do not have to be contiguous in frequency and can even be located in different frequency bands in order to enable exploitation of fragmented spectrum allocations by means of spectrum aggregation.

Channel-dependent scheduling in both the time and frequency domains is supported for both downlink and uplink with the base-station scheduler being responsible for (dynamically) selecting the transmission resource as well as the data rate. The basic operation is dynamic scheduling, where the base-station scheduler takes a decision for each 1 ms Transmission Time Interval (TTI), but there is also a possibility for semi-persistent scheduling to reduce the control-signaling overhead. The basic time–frequency unit in the scheduler corresponds to 180 kHz in the frequency domain and 1 ms in the time domain, also referred to as resource-block pair. To support downlink channel-dependent scheduling, the terminal may provide channel-status reports to the network, providing information about the  instantaneous channel status in the time and frequency domains, as well as information necessary to determine the appropriate antenna processing in case of spatial multiplexing. To support uplink channel-dependent scheduling uplink, terminals may transmit sounding reference signals, which the base station can use to estimate the uplink channel status. For the TDD RIT, where more accurate uplink/downlink channel reciprocity may hold due to uplink/downlink operation on the same frequency, uplink sounding can also be used to complement the estimate of the downlink channel status for downlink scheduling and signal processing.

Multi-antenna transmission scheme is an integral part of both RITs. Multi-antenna pre-coding with dynamic rank adaptation supports both spatial multiplexing (single-user MIMO, SU-MIMO) and beam-forming. Spatial multiplexing with up to eight layers in the downlink and four layers in the uplink is supported. Multi-user MIMO (MU-MIMO), where multiple users are assigned the same time-frequency resources and the interference is suppressed by the receiver, is also supported. Finally, transmit diversity based on Space-Frequency Block Coding (SFBC) or a combination of SFBC and Frequency Switched Transmit Diversity (FSTD) is supported.

Inter-cell interference coordination (ICIC), where neighboring cells exchange information aiding the scheduling in order to reduce the interference for terminals, is supported for the RITs. ICIC can be used for homogenous deployments with non-overlapping cells of similar transmission power, as well as for heterogeneous deployments where a higher-power cell overlays one or several lower-power nodes.

Relaying functionality is supported by the SRIT and within both the FDD and TDD RITs. The relay node appears as a conventional base station (e-Node B) to terminals but is wirelessly backhauled to the remaining part of the radio-access network using the LTE release 10 radio-interface technology.

### 1.3.1	Network architecture

The LTE radio-access network has a flat architecture with a single type of node – the eNodeB which is responsible for all radio-related functions in one or several cells. The eNodeB is connected to the core network by means of the S1 interface, more specifically to the serving gateway (S-GW) by means of the user-plane part, S1-u, and to the Mobility Management Entity (MME) by means of the control-plane part, S1-c. One eNodeB can interface to multiple MMEs/S-GWs for the purpose of load sharing and redundancy.

The X2 interface, connecting eNodeBs to each other, is mainly used to support active-mode mobility. This interface may also be used for multi-cell Radio Resource Management (RRM) functions such asICIC. The X2 interface is also used to support lossless mobility between neighboring cells by means of packet forwarding.

Figure 1.1 : Radio-access network interfaces.

### 1.3.2	Layer 2 protocol architecture

Layer 2 (L2) consists of several sublayers: Packet Data Convergence Protocol (PDCP), Radio Link Control (RLC) and Medium Access Control (MAC). The downlink and uplink protocol structures are illustrated in Figure 1.2 and Figure 1.3, respectively. Layer 2 provides one or more Radio Bearers to higher layers to which IP packets are mapped according to their Quality-of-Service requirements. L2/MAC PDUs, also referred to as transport blocks, are created according to instantaneous scheduling decisions and delivered to the physical layer on one or several transport channels (one transport channel per component carrier).

Figure 1.2: Downlink L2 protocol structure

Figure 1.3: Uplink L2 protocol structure.

#### 1.3.2.1	Packet Data Convergence Protocol (PDCP)

Packet Data Convergence Protocol (PDCP) is responsible for

- User plane:

  - Header compression and decompression using ROHC;

  - Transfer of user data;

  - In-sequence delivery of upper layer PDUs at PDCP re-establishment procedure for RLC AM;

  - Duplicate detection of lower layer SDUs at PDCP re-establishment procedure for RLC AM;

  - Retransmission of PDCP SDUs at handover for RLC AM;

  - Ciphering and deciphering;

  - Timer-based SDU discard in uplink.

- Control plane:

  - Ciphering and Integrity Protection;

  - Transfer of control plane data.

There is one PDCP entity per radio bearer configured for a UE.

#### 1.3.2.2	Radio Link Control (RLC)

Radio Link Control (RLC) is responsible for

- Transfer of upper layer PDUs;

- Error Correction through ARQ (only for AM data transfer);

- Concatenation, segmentation and reassembly of RLC SDUs (only for UM and AM data transfer);

- Re-segmentation of RLC data PDUs (only for AM data transfer);

- Reordering of RLC data PDUs (only for UM and AM data transfer);

- Duplicate detection (only for UM and AM data transfer);

- Protocol error detection (only for AM data transfer);

- RLC SDU discard (only for UM and AM data transfer);

- RLC re-establishment.

Depending on the mode-of-operation, a RLC entity may provide all, a subset of, or none of the services above. The RLC can operate in three different modes:

	Transparent mode (TM), where the RLC is completely transparent and is in essence bypassed. This configuration is used for control-plane broadcast channels such as BCCH, CCCH, and PCCH only where the information should reach multiple users. The size of these messages are selected such that all intended terminals are reached with a high probability and hence there is neither need for segmentation to handle varying channel conditions, nor retransmissions to provide error-free data transmission. Furthermore, retransmissions are not possible for these channels as there is no possibility for the terminal to feed back status reports as no uplink has been established.

	Unacknowledged mode (UM), where the RLC provides all the functionality above except error correction, is used when error-free delivery is not required, for example for MCCH and MTCH using MBSFN and for VoIP.

	Acknowledged mode (AM), where the RLC provides all the services above, is the main mode-of-operation for TCP/IP packet data transmission on the DL-SCH. segmentation/reassembly, in-sequence delivery and retransmissions of erroneous data are all supported.

The RLC offers services to the PDCP in the form of radio bearers and uses services from the MAC layer in the form of logical channels. There is one RLC entity per radio bearer configured for a terminal.

#### 1.3.2.3	Medium access control

The MAC layer is responsible for

- Mapping between logical channels and transport channels;

- Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks delivered to/from the physical layer on transport channels;

- Scheduling information reporting;

- Error correction through N-process stop-and-wait hybrid-ARQ (HARQ) with synchronous (uplink) asynchronous (downlink) retransmissions;

- Priority handling between logical channels of one UE;

- Priority handling between UEs by means of dynamic scheduling;

- MBMS service identification;

- Transport format selection;

- Padding.

The MAC offers services to the RLC in the form of logical channels. A logical channel is defined by the type of information it carries and is generally classified as a control channel, used for transmission of control and configuration information necessary for operating an LTE system, or as a traffic channel, used for the user data. The set of logical-channel types specified for LTE includes:

- Broadcast Control Channel (BCCH), used for broadcasting system control information.

- Paging Control Channel (PCCH), a downlink channel used for paging when the network does not know the location of the UE and for system information change notifications.

- Common Control Channel (CCCH), used for transmission of control information between UEs and network when the UE has no RRC connection.

- Dedicated Control Channel (DCCH), used for transmission of control information to/from a mobile terminal when the UE has a RRC connection.

- Multicast Control Channel (MCCH), used for transmission of control information required for reception of the MTCH.

- Dedicated Traffic Channel (DTCH), used for transmission of user data to/from a mobile terminal. This is the logical channel type used for transmission of all uplink and non-MBSFN downlink user data.

- Multicast Traffic Channel (MTCH), used for downlink transmission of MBMS services.

From the physical layer, the MAC layer uses services in the form of Transport Channels. A transport channel is defined by how and with what characteristics the information is transmitted over the radio interface. Data on a transport channel is organized into transport blocks. In each Transmission Time Interval (TTI), at most one or two (in case of spatial multiplexing) transport blocks are transmitted per component carrier.

Associated with each transport block is a Transport Format (TF), specifying how the transport block is to be transmitted over the radio interface. The transport format includes information about the transport-block size, the modulation scheme, and the antenna mapping. The scheduler is responsible for (dynamically) determining the uplink as well as downlink transport format in each TTI.

The following transport-channel types are defined for LTE:

- Broadcast Channel (BCH) has a fixed transport format, provided by the specifications. It is used for transmission of parts of the BCCH system information, more specifically the so-called Master Information Block (MIB).

- Paging Channel (PCH) is used for transmission of paging information from the PCCH logical channel. The PCH supports discontinuous reception (DRX) to allow the mobile terminal to save battery power by waking up to receive the PCH only at predefined time instants.

- Downlink Shared Channel (DL-SCH) is the main transport-channel type used for transmission of downlink data in LTE. It supports dynamic rate adaptation and channel-dependent scheduling in the time and frequency domains, hybrid-ARQ with soft combining, and spatial multiplexing. It also supports DRX to reduce mobile-terminal power consumption while still providing an always-on experience. The DL-SCH is also used for transmission of the parts of the BCCH system information not mapped to the BCH. In case of transmission to a terminal using multiple component carriers the UE receives one DL-SCH per component carrier.

- Multicast Channel (MCH) is used to support MBMS. It is characterized by a semi-static transport format and semi-static scheduling. In case of multi-cell transmission using MBSFN, the scheduling and transport format configuration is coordinated among the cells involved in the MBSFN transmission.

- Uplink Shared Channel (UL-SCH) is the uplink counterpart to the DL-SCH, that is the uplink transport channel used for transmission of uplink data.

In addition, the Random Access Channel (RACH) is also defined as an uplink transport channel although it does not carry transport blocks.

The mapping between logical channels, transport channels and physical channels (described in Section 1.3.3) is illustrated in Figure 1.4 for the downlink and Figure 1.5 for the uplink.

Figure 1.4: Downlink channel mapping.

Figure 1.5: Uplink channel mapping.

### 1.3.3	Physical layer

The physical layer is responsible for coding, physical-layer hybrid-ARQ processing, modulation, multi-antenna processing, and mapping of the signal to the appropriate physical time–frequency resources. It also handles mapping of transport channels to physical channels according to Figure 1.5. A simplified overview of the processing for the DL-SCH is given in Figure 1.6.

Figure 1.6: Simplified physical-layer processing for DL-SCH on one component carrier

#### 1.3.3.1	Time-domain structure and duplex schemes

Figure 1.7 illustrates the high-level time-domain structure for LTE transmission with each (radio) frame of length 10 ms consisting of ten equally sized subframes of length 1 ms.  Each 1 ms subframe consists of two equally sized slots of length Tslot = 0.5 ms with each slot consisting of a number of OFDM symbols including cyclic prefix. Scheduling decisions are taken per subframe, i.e. on a 1 ms basis.

Figure 1.7: LTE time-domain structure.

LTE can operate in both FDD and TDD as illustrated in Figure 1.8. Although the time-domain structure is, in most respects, the same for FDD and TDD there are some differences between the two duplex modes, most notably the presence of a special subframe in case of TDD. The special subframe is used to provide the necessary guard time for downlink-to-uplink switching as discussed below.

Figure 1.8: Uplink/downlink time/frequency structure in case of FDD and TDD.

##### 1.3.3.1.1	Time-domain structure for the FDD RIT

In case of FDD operation (upper part of Figure 1.8), there are two carrier frequencies for each component carrier, one for uplink transmission (fUL) and one for downlink transmission (fDL). During each frame, there are thus ten uplink subframes and ten downlink subframes and uplink and downlink transmission can occur simultaneously within a cell. Isolation between downlink and uplink transmissions is achieved by transmission/reception filters and a sufficiently large separation in the frequency domain. Half-duplex operation at the UE side is supported by the scheduler ensuring non-simultaneous reception and transmission at the UE.

##### 1.3.3.1.2	Time-domain structure for the TDD RIT

In case of TDD operation (lower part of Figure 1.8), there is only a single carrier frequency per component carrier and uplink and downlink transmissions are always separated in time also on a cell basis. As seen in the figure, some subframes are allocated for uplink transmissions and some subframes for downlink transmission with the switch between downlink and uplink occurring in the special subframe (subframe 1 and, in some cases, subframe 6). The switch between downlink and uplink occurs in the special subframe which is split into three parts: a downlink part (DwPTS), a guard period (GP) where the switch occurs, and an uplink part (UpPTS). The DwPTS is in essence treated as a normal downlink subframe, although the amount of data possible to transmit is smaller due to the reduced length of the DwPTS compared to a normal subframe. The UpPTS can be used for channel sounding or random access. The duration of the DwPTS, GP and UpPTS is configurable to support different deployment scenarios.

Different asymmetries in terms of the amount of resources allocated for uplink and downlink transmission, respectively, are provided through seven different downlink/uplink configurations as shown in Figure 1.9. In case of carrier aggregation, the downlink/uplink configuration is identical across component carriers.

Coexistence between the LTE and other (IMT-2000) TDD systems such as TD-SCDMA is catered for by aligning the switch points between the two systems and selecting the appropriate special subframe configuration and uplink-downlink asymmetry.

Figure 1.9: Uplink-downlink asymmetries supported by the TDD RIT.

.

#### 1.3.3.2	Physical layer processing

To the transport block(s) to transmit on a DL-SCH or UL-SCH, a CRC is attached, followed by rate-1/3 Turbo coding for error correction. Rate matching is used not only to match the number of coded bits to the amount of resources allocated for the DL-SCH/UL-SCH transmission, but also to generate the different redundancy versions as controlled by the hybrid-ARQ protocol. In case of spatial multiplexing, the processing is duplicated for each of the two transport blocks. After rate matching, the coded bits are modulated using QPSK, 16QAM, or 64QAM. In case of multi-antenna transmission, the modulation symbols are mapped to multiple layers and precoded  before being mapped to the different antenna ports. Alternatively, transmit diversity can be applied. Finally, the (precoded) modulation symbols are mapped to the time-frequency resources allocated for the transmission.

Downlink transmission is based on conventional OFDM with a cyclic prefix. The subcarrier spacing is f = 15 kHz and two cyclic prefix lengths are supported: normal cyclic prefix 4.7 µs and extended cyclic prefix 16.7 µs. In the frequency domain, the number of resource blocks can range from 6 to 110 per component carrier, where a resource block is 180 kHz in the frequency domain. There can be up to five component carriers transmitted in parallel implying an overall bandwidth up to roughly 100 MHz.

Uplink transmission is based on DFT-spread OFDM (DFTS-OFDM). DFTS-OFDM can be seen as a DFT precoder, followed by conventional OFDM with the same numerology as in the downlink. Multiple DFT precoding sizes, corresponding to transmission with different scheduled bandwidths, can be used.

The remaining downlink transport channels (PCH, BCH, MCH) are based on the same general physical-layer processing as the DL-SCH, although with some restrictions in the set of features used. The MCH is used for MBMS transmissions with single-frequency network operation as described in Section 1.3.3.7.

#### 1.3.3.3	Reference signals

Downlink reference signals are pre-defined signals transmitted within the downlink time/frequency grid. Multiple types of downlink reference signals, transmitted in different ways and used for different purposes by the receiving mobile terminal, are supported:

- Cell-specific reference signals (CRS), used for demodulation of control and data transmissions, channel-state reporting, and mobility measurements. These reference signals are transmitted in every downlink subframe and across the full bandwidth of each component carrier.

- UE-specific reference signals (UE-RS), used for demodulation of data transmissions, especially in conjunction with non-codebook-based beam-forming. These reference signals are only transmitted in the resource blocks and transmission layers used for DL-SCH transmission and are jointly precoded together with the data.

- CSI reference signals (CSI-RS), used to support reporting of channel-state information from the terminals to the network. CSI reference signals are sparse in the time and frequency and transmitted at a configurable periodicity.

- Positioning reference signals, used to enhanced positioning functionality.

- MBSFN reference signals, used for channel estimation for MBSFN transmission.

The cell-specific reference signals are present in every downlink subframe while the presence and periodicity of the other reference signal types depends on the configuration and transmission mode.

In the uplink, two types of reference signals are supported:

- Demodulation reference signals, used for demodulation of UL-SCH transmission. These reference signals are transmitted in conjunction with the UL-SCH only.

- Sounding reference signals, transmitted at configurable intervals or upon request and used for uplink channel sounding in order to support uplink channel-dependent scheduling. For TDD, channel reciprocity can be used to derive the downlink channel conditions from the uplink channel conditions.

#### 1.3.3.4	Multi-antenna transmission

A wide range of multi-antenna transmission schemes are supported in the downlink:

- Single-antenna transmission using a single cell-specific reference signal.

- Closed-loop spatial multiplexing, also known as codebook-based beam-forming or precoding, of up to four layers using cell-specific reference signals. Feedback reports from the terminal are used to assist the eNodeB in selecting a suitable precoding matrix.

- Open-loop spatial multiplexing, also known as large-delay cyclic delay diversity, of up to four layers using cell-specific reference signals.

- Spatial multiplexing of up to eight layers using UE-specific reference signals. The eNodeB may use feedback reports or exploit channel reciprocity to set the beam-forming weights.

- Transmit diversity based on space-frequency block coding (SFBC) or a combination of SFBC and Frequency Switched Transmit Diversity (FSTD)

- Multi-user MIMO where multiple terminals are assigned overlapping time-frequency resources and receiver processing in combination with the spatial properties of the channel is used to handle the interference between the different transmissions.

The following multi-antenna transmission schemes are supported in the uplink:

- Single-antenna transmission

- Pre-coding supporting rank-adaptive spatial multiplexing with one up to four layers

#### 1.3.3.5	Link adaptation and power control

According to the radio channel conditions, the modulation and channel coding schemes (MCS) can be adapted flexibly. The same modulation and coding is applied to all resources units assigned to the same L2 PDU within a TTI Downlink power control determines the energy per resource element (EPRE). Uplink power control determines the average power over a DFTS-OFDM symbol in which the physical channel is transmitted.

#### 1.3.3.6	L1/L2 control signaling

Downlink control information (DCI) is transmitted in the first one to three OFDM symbols of each downlink subframe in each component carrier with the number of OFDM symbols being indicated on the PCFICH. Downlink and uplink scheduling grants (consisting of UE identity, time-frequency resources and transport format) and hybrid-ARQ acknowledgements are transmitted on the PDCCH and PHICH, respectively. Each grant is transmitted on a separate PDCCH using QPSK modulation.

Uplink control information (UCI), consisting of channel-status information, scheduling requests and hybrid-ARQ acknowledgements, is transmitted at the band edges of the primary uplink component carrier the PUCCH. Alternatively, parts of the control signaling can be multiplexed with data on the PUSCH.

#### 1.3.3.7	MBSFN operation

Multicast/Broadcast over Single Frequency Network (MBSFN) transmission, where the same signal is transmitted from multiple, time-synchronized cells, is supported by the MCH transport channel. One component carrier can support simultaneous unicast and broadcast support through time-domain multiplexing of MCH and DL-SCH transmissions. Terminals are provided with MBSFN scheduling information such that they can exploit DRX and only wake up when the service of interest is transmitted.

# 2.	Detailed Specification of the Radio Interface Technology

The standards contained in this Annex for the FDD RIT and the TDD RIT of the SRIT are derived from the global core specifications for IMT-Advanced as developed by 3GPP and are contained at “URL – this is the ITU-R link where the submit GCS is stored ”.

______________

____________________