3GPP TSG RAN Meeting #100	RP-231036

Taipei, June 12 - 14, 2023

Agenda Item:	9.2.2

Source:	Futurewei

Title:	Remaining details for RAN Ambient-IoT Study

Document for:	Discussion and decision

# Introduction

RAN plenary level study item on Ambient IoT was approved in RAN plenary #97e meeting with the latest SID in [2]. The study aims at a new 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications.

The main objectives of the SI include device categorization, identification of the suitable deployment scenarios and their characteristics for use cases/services agreed in the related SA1 SI [3], formulation of set of RAN design targets, and identification of required functionality to be supported.

During the RAN plenary #99 meeting, progress was made on TR skeleton, grouping of SA1 use cases, deployment scenarios, connectivity topologies, and device categorization.

In this contribution, we address the remaining open issues in the RAN SI and then present our views on the Rel-19 RAN-WG level study/work item.

# Discussion

## Ambient IoT Device Categorization

According to TR 38.848, three Ambient IoT devices are categorized along with their energy storage and RF signal generation capabilities as presented in Table 1.

Table 1: Device categorization

| Ambient IoT Device | Energy Storage | Signal Generation Capability |
| --- | --- | --- |
| Device A | No | No capability to generate and amplify RF signals and rely on backscattering transmissions |
| Device B | Yes | No capability to generate RF signals and rely on backscattering transmissions; however, it can amplify reflected signals through its stored energy |
| Device C | Yes | Yes, it has independent active RF components to generate signals |

Referring to Table 1, Devices A, B and C vary in their energy storage and RF signal generation capabilities, which will result in different design targets, considerations and approaches. Among the devices, Device C can achieve better coverage/range, data rates, latency and other performance requirements at the expense of higher cost, complexity and power consumption compared to Devices A and B. Refer to Section 2.4 for the complexity and power consumption of the devices.

Network access procedures and upper layer protocols can be easily simplified and tailored to support Device C while Devices A and B may require a completely new/different design.

Proposal 1: For the Rel-19 RAN-WG level study/work item, decide between Device A (and Device B) versus Device C.

## Ambient IoT Deployment Scenarios

During RAN#99, five deployment scenarios were agreed and captured in TR 38.848. The column labelled as “Description” in Table 4.2.2.1-2 to Table 4.2.2.1-5 in TR 38.848 is empty. Note that Table 4.2.2.1-2 to Table 4.2.2.1-5 in TR 38.848 correspond to Deployment Scenarios 2 to 5. As a starting point, we propose to adopt the suggested modifications by Rapporteur in “Results of offline discussions on ambient IoT [6].

Proposal 2: For the remaining issues/details in Deployment Scenarios 2 to 5, adopt the suggested modifications by Rapporteur in the offline discussion of RAN#99:

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) Note: topology (3) where either the uplink or the downlink coverage is extended |
|  | Spectrum | Licensed FDD, or Licensed TDD or unlicensed (for topology (2) and (3)) |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 3: Device indoors, UE indoors based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | NoneNo base station in the topology |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS-[Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] Note: a device C could also support such deployment |

Deployment 4: Device outdoors, Outdoor base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 5: Device outdoors, UE outdoors based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | No base station in the topologyNone |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS-[Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Examining all the deployment scenarios, we observe that Deployment Scenarios 3 and 5 are a special case of Deployment Scenarios 1, 2 and 4 because they can reuse the Topology 1 design for Topology 4.

Proposal 3: For the Rel-19 RAN-WG level study/work item, focus on Deployment Scenarios 1, 2 and 4.

## Connectivity topologies

In relation to the ambient IoT network architecture, four connectivity topologies are captured in TR 38.848 as shown in Figure 1.

Topology 1: BS ↔ Ambient IoT device

Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS (with downlink assistance)

Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS (with uplink assistance)

Topology 4: UE ↔ Ambient IoT device

Figure 1: Connectivity topologies (from TR 38.848)

It is important to point out that Topology 1 is the basic topology among the four connectivity topologies. It can be reused as a component for Topologies 2 and 4. In Topology 3, the link between the BS (and assisting node) and the Ambient IoT can also be similar to that of Topology 1 while the further details of Topology 3 are yet to be clarified.

Topology 4 can be viewed as a special case of Topologies 2 without the BS, and the intermediate node corresponds to the UE. However, it is not clear how to establish a link between the Ambient IoT device and the UE when the BS is absent.

Based on the above discussion, RAN WG should start with the basic topology (i.e., Topology 1) in Rel-19 SI. Technical studies on Topologies 2, 3 and 4 can start once the study on Topology 1 is completed.

Proposal 4: For the Rel-19 RAN-WG level study/work item, focus on (or prioritize) the basic topology (Topology 1: BS  Ambient IoT device).

## RAN Design Targets for Ambient IoT

At the RAN#99 meeting, a consensus was reached on a set of design target parameters for Ambient IoT in the RAN study item. The set of design target parameters is captured in TR 38.848, Section 5, as summarized in Table 2.

Table 2: RAN design targets (from TR 38.848)

| RAN Design Parameter | RAN Design Target |
| --- | --- |
| Device power consumption | Device A: The power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW] |
|  | Device B: Device A power consumption ≪ Device B power consumption < Device C power consumption, or Device A power consumption ≤ Device B power consumption < Device C power consumption |
|  | Device C: ≤ 1 mW to ≤ 10 mW |
| Device complexity | Device A: The complexity target is to be comparable to UHF RFID ISO18000-6C (EPC C1G2). |
|  | Device B: Device A complexity < Device B complexity < Device C complexity |
|  | Device C: The complexity target is to be orders-of-magnitude lower than NB-IoT |
| Coverage | No consensus |
| User experienced data rate | At least for uplink, maximum not less than 5 kbps, and minimum not less than 0.1 kbps |
| Maximum message size (or maximum ‘TB’ size) | No consensus |
| Latency | No consensus |
| Positioning accuracy | No consensus |
| Connection/device density | No consensus |
| Moving speed of device | No consensus |

As can be observed from Table 2, a number of RAN design targets, except for the ‘Device power consumption’, ‘Device complexity’ and ‘User experienced data rate’, are not resolved. Herein, we further evaluate and provide our views on the RAN design targets.

- Coverage: In view of the low power consumption characteristics of an Ambient IoT device, its transmission power is very low. In addition, in the case of energy harvesting from RF signals, the distance between the power source node and the device needs to be short. Furthermore, the processing power at the device will also be very limited to ensure very low cost. All these contribute to potentially very short communication range. On the other hand, it is technically and commercially desirable to extend the range. Overall, from radio access perspective, the target communication range is likely the most fundamental (and yet sometimes trickiest) parameter of the design. It also depends on other targets, for example data rate, latency, deployment scenarios (indoors or outdoors) and reliability as secondary factors. Consequently, such a parameter should be evaluated and determined by the RAN WG after initial study and evaluation.

- Maximum message size: It depends on use cases and deployment scenarios (indoors or outdoors).

- Latency: In general, latency depends on data rates, signalling protocol overhead, use cases and deployment scenarios (indoors or outdoors).

- Positioning accuracy: Positioning service-related requirements are for a subset of use cases, not necessarily relying on Ambient IoT technology, and likely putting unnecessary burden and complexity to the design. Therefore, they are not at this stage fundamental to the RAN design.

- Connection/device density: can be heavily dependent on deployment choices, resource availability, etc.

- Moving speed of device: For most of the use cases, the Ambient IoT device is either static or with low velocity, and continuous service is not required when moving across service/coverage areas of the network. Therefore, mobility (as currently understood) is not a basic design target.

Based on the above analysis, we make the following proposal:

Proposal 5: The final design targets should be left to the RAN-19 RAN WG level study/work item as part of the evaluation methodology and KPI definitions.

# Conclusions

We make the following proposals to address the remaining details in the RAN SI.

Proposal 1: For the Rel-19 RAN-WG level study/work item, decide between Device A (and Device B) versus Device C.

Proposal 2: For the remaining issues/details in Deployment Scenarios 2 to 5, adopt the suggested modifications by Rapporteur in the offline discussion of RAN#99:

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) Note: topology (3) where either the uplink or the downlink coverage is extended |
|  | Spectrum | Licensed FDD, or Licensed TDD or unlicensed (for topology (2) and (3)) |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 3: Device indoors, UE indoors based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | NoneNo base station in the topology |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS-[Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] Note: a device C could also support such deployment |

Deployment 4: Device outdoors, Outdoor base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 5: Device outdoors, UE outdoors based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | No base station in the topologyNone |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | FFS-[Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Proposal 3: For the Rel-19 RAN-WG level study/work item, focus on Deployment Scenarios 1, 2 and 4.

Proposal 4: For the Rel-19 RAN-WG level study/work item, focus on (or prioritize) the basic topology (Topology 1: BS  Ambient IoT device).

Proposal 5: The final design targets should be left to the RAN-19 RAN WG level study/work item as part of the evaluation methodology and KPI definitions.

# References

RP-230572, Views on Ambient IoT towards focused RAN work, Futurewei

RP-223396, SID revision: Study on Ambient IoT, Huawei, HiSilicon

3GPP TR 22.840 V1.1.0 (2023-03), Study on Ambient power-enabled Internet of Things (Release 19)

RP-223402, Workplan for RAN Study on Ambient IoT, Huawei

RP-230795, Results of offline discussions on ambient IoT, Moderator (Huawei)

RP-230801, Results of offline discussions on ambient IoT, Moderator (Huawei)