3GPP TSG RAN Meeting #100								         RP-231310

Taipei, Taiwan, June 12-14, 2023

## Status Report to TSG

Agenda item:			10.2.2

| WI / SI Name | Perf. part: NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_NBIoT_eMTC_NTN_req-Perf |  |  |  |  |
| Unique ID | 950274 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223437 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2022 | Performance part: 06/2023 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 100% | Performance Part: Overall: 100% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Hsuanli Lin |
|  | Company | MediaTek Inc. |
|  | Email | hsuanli.lin@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### N/A

## 2.2	RAN2

#### N/A

## 2.3	RAN3

#### N/A

## 2.4	RAN4

#### 2.4.1	Agreements

##### RAN4 #106bis-e

SAN RF conformance testing

- The Big TP to TS 36.181 SAN conformance testing in R4-2305949 agreed [1].

- WF on remaining issues for the TS 36.181 drafting in R4-2305936 agreed [2].

RRM performance requirements

- Draft Big CR on RRM performance requirements and test cases were agreed in [3].

- For RRM performance requirements, further agreements and test cases were captured in WF [4], as follows:

#### Issue 1-1: Measurement accuracy

- RAN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define Power Headroom for UE Category NB1 and M1 for Satellite Access in separate clause

#### Issue 1-2: Measurement Reporting Table

The legacy RSRP/RSRP/NRSRP/NRSRQ measurement report mapping can be reused for IoT NTN.

#### Issue 2: PHR reporting for NB-IoT

- No need to discuss PHR reporting tables for M1, as it has been agreed to reuse the legacy tables.

- For NB-IoT LEO, legacy PHR reporting values can be reused for IoT NTN.

- For NB-IoT GEO, continue discussion.

  - Option 1: reuse legacy PHR reporting values for IoT NTN

  - Option 2: modify the values in exiting tables to include more negative PHR reporting values

- Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH [-10] |
| POWER_HEADROOM_1 | [-10] PH [-2] |
| POWER_HEADROOM_2 | [-2] PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH [-30] |
| POWER_HEADROOM_1 | [-30] PH [-20] |
| POWER_HEADROOM_2 | [-20] PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

#### Issue 3: GNSS Position Acquisition Method

- [GNSS or GNSS signal emulation] test approach for UE UL transmission timing accuracy requirements and adopts AT command-based test approach for the rest of the test cases.

- Test parameter for GNSS signal power levels defined in [B.6.1] is reused.

- How UE receives the GNSS signals and acquires the UE position before/during the test is up to UE implementation. And the details are expected to be discussed/determined it in RAN5.

#### Issue 4: Test for Segmented UL Transmissions

- Proposal 1: Do not define dedicated UE initial transmission timing test cases for UL segmented transmission.

- Proposal 2: Define UE initial transmission timing test for UL segmented transmission, the test parameter in A.7.1.18 Test 1 can be the starting point.

  - In test case, set Nsegmentprecompensation  (npusch-TxDuration) as 32ms, and Ngapprecompensation (ntn-SegmentedPrecompensationGaps) as 2 slots.

  - For the test requirement of UL segmented transmission,

    - The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in core requirements until the UE transmit timing offset is withinNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN respective to the first detected path (in time) of DL SSB. The adjustment will be only performed at the start of a transmission segment boundary.

    - For the first transmission in each segment, the test system shall verify that the UE transmit timing offset stays within NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN of the first detected path of DL SSB.

#### Issue 6-1: Configuration of Satellite parameters

- re-use the parameter settings (if applicable) for SIB19 as specified in A.3.36.4 in 38.133.

  - FFS how to set common delay for LEO.

#### Issue 6-2: High level test procedure for SAN RRM tests

- re-use the applicable parts in “High level test procedure for SAN RRM tests” as specified in B.5 in 38.133

#### Issue 6-3 (new): Antenna configuration

- Define requirements with 1Tx1Rx for both Cat-M1 and NB-IoT over NTN

#### Issue 7: Tests involving neighbor cell measurement

- Introduce the following test cases of intra-frequency measurements for GEO, for NB-IoT and M1, respectively:

| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 |
| --- | --- | --- | --- |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | NB 1-4 |
| RRC Connection Control | RRC Re-establishment | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | NB 2-1 |

| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 |

| RRC CONNECTED Mode Mobility | Handover and Conditional Handover | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-2 |
|  |  | E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-5 |
|  |  | E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-6 |

UE demodulation requirements

- Big CR to TS36.102 for IoT-NTN UE demodulation is agreed and captured in [5].

For UE demodulation requirements, further agreements were captured in WF [6], as follows:

Issue 1: Specification skeleton

Agreements:

- Add “Annex E: propagation conditions” in TS36.102

Issue 2: PDSCH scheduling for Cat-M1 PDSCH demodulation requirements

Agreements:

- Agree with Cat-M1 PDSCH test 1 from Ericsson’s proposal (R4-2305175). (schedule two PDSCH subframes in one radio frame)

- Agree with Cat-M1 PDSCH test 2 and test 3 from Huawei’s proposal (R4-2302193).

- Add a note in TS36.102: MPDCCH and PDSCH are dropped when colliding with SIB2/SIB3.

Issue 3: NPDCCH repetition factor in test 2 for NB-IoT PDSCH demodulation requirements

Agreements:

- For NB-IoT NPDSCH demodulation requirements, set NPDCCH repetition factor to be 128 in test 2.

  - Interested companies can verify this repetition number ensures the NPDCCH BLER becomes low enough.

Issue 4: UE location information

Agreements:

- Leave RAN5 to discuss this issue. If the decision of RAN5 impacts the UE demodulation requirements in RAN4, RAN4 can come back for discussion.

SAN demodulation requirements

- CR work split is agreed in [7].

For SAN demodulation requirements, agreements were captured in WF [7], as follows:

# Topic #1: General

  - Sub-topic 1-1: pre-compensation gap

Agreement: Whether to consider the multi-segment scenario in the SAN demodulation requirements

  - No (UL channel transmission duration should not exceed TxDuration parameter)

  - Sub-topic 1-2: Frequency offset drift

Agreement: Doppler offset drift modeling

  - Apply Doppler offset drift model on PUSCH, PUCCH, NPUSCH format 1 and NPUSCH format 2.

  - Not consider Doppler offset drift model for PRACH and NPRACH performance requirements.

  - Use fixed (the largest) frequency offset value for the SAN demodulation requirement. See the table below for the additional largest Doppler shift.

| Channel | Repetition | Tx duration in one segment | Largest frequency offset value (Hz) |
| --- | --- | --- | --- |
| eMTC PUSCH CE mode A | 8 | 8ms | 4 |
| eMTC PUSCH CE mode B | 256 | 256ms | 128 |
| eMTC PUCCH format 1a | 8 | 8ms | 4 |
| NPUSCH format 1, 3.75KHz, 1 tone | 4 | 256ms | 160 |
| NPUSCH format 1, 15KHz, 12 tones | 16 | 16ms | 8 |
| NPUSCH format 2, 3.75KHz, 1 tone | 16 | 128ms | 64 |
| NPUSCH format 2, 15KHz, 1 tone | 16 | 32ms | 16 |

  - Sub-topic 1-3: Timing offset drift

Agreement: Whether to consider timing offset drift model

  - The total timing offset doesn’t exceed the CP.

  - Not consider timing offset drift model in TS36.108 but consider it in TS36.181

Agreement: Timing offset drift model

  - Apply the additional transmitting timing drift to PUSCH, PUCCH, NPUSCH format 1 and NPUSCH format 2.

  - Not consider additional transmitting timing drift for PRACH and NPRACH performance requirements

  - Apply the gradual transmission timing offset model with step size Δt. See the table below for the step size per channel.

| Channel | Repetition | Tx duration in one segment | Step size Δt |
| --- | --- | --- | --- |
| eMTC PUSCH CE mode A | 8 | 8ms | [0.01] µs per subframe |
| eMTC PUSCH CE mode B | 256 | 256ms | [0.01] µs per subframe |
| eMTC PUCCH format 1a | 8 | 8ms | [0.01] µs per subframe |
| NPUSCH format 1, 3.75KHz, 1 tone | 4 | 256ms | [0.32] µs per RU |
| NPUSCH format 1, 15KHz, 12 tones | 16 | 16ms | [0.01] µs per RU |
| NPUSCH format 2, 3.75KHz, 1 tone | 16 | 128ms | [0.08] µs per RU |
| NPUSCH format 2, 15KHz, 1 tone | 16 | 32ms | [0.02] µs per RU |

  - Sub-topic 1-4: Requirement scope

Agreement: Requirement scope with timing offset drift model

  - Specify the SAN type 1-O radiated demodulation performance requirements for both NB-IoT and eMTC, by reusing the conducted performance requirements.

# Topic #2: SAN demodulation requirement for eMTC

  - Sub-topic 3-1: PUSCH requirements

Way forward: Updated simulation assumptions for PUSCH CE mode A (for alignment purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5 |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Fraction of maximum throughput | 70% |

Way forward: Simulation assumption for PUSCH CE mode B (for alignment purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Fraction of maximum throughput | 70% |

  - Sub-topic 3-2: PUCCH requirements

Way forward: Simulation assumption for PUCCH format 1a (for alignment purpose)

  - The ACK missed detection probability shall not exceed 1%

  - Assume TxDuration is same as the number of repetitions

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |

  - Sub-topic 3-3: PRACH requirements

Way forward: Simulation assumptions for PRACH requirements (for alignment purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Frequency hopping: OFF

  - Assume TxDuration is same as the number of repetitions

  - Timing estimation error: 2.08us

  - RAN4 will set the same propagation channel for PUCCH format 1a and PRACH fading scenario.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-5 | 270Hz | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

# Topic #3: SAN demodulation requirement for NB-IoT

  - Sub-topic 4-1: NPUSCH format 1 requirements

Way forward: Simulation assumptions for NPUSCH format 1 requirements (for alignment purpose)

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | TS36.104 A16-1 | 4 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1 | TS36.104 A16-5 | 16 | 70% |

  - Sub-topic 4-2: NPUSCH format 2 requirements

Way forward: Simulation assumptions for NPUSCH format 2 requirements (for alignment purpose)

  - The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

  - The ACK missed detection probability shall not exceed 1%.

  - Assume TxDuration is same as the number of repetitions.

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 |

  - Sub-topic 4-3: NPRACH requirements

Way forward: Simulation assumptions for NPRACH demodulation requirements (for alignment purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Assume TxDuration is same as the number of repetitions

  - Timing estimation error: 3.646us

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | NTN-TDLA100-1 | 200Hz | YES | YES |
| 1 | 1 and 2 | 16 | NTN-TDLA100-1 | 200Hz | YES | YES |

##### RAN4 #107

SAN RF conformance testing

- The final Big TP to TS 36.181 SAN conformance testing is agreed as in [8].

Issue 2-1: MU for radiated NB-IoT requirements

- Proposals

  - Option 1: It is reasonable to have the same 0.3 dB offset for NB IoT where defined as in TS36.141 for NB IoT SAN OTA test in TS36.181

- Agreement: Option 1 confirmed as agreement.

The agreements as follows:

RRM performance requirements

- The Big CR on NB-IoT/eMTC RRM performance requirements for NTN is agreed as in [9].

Issue 1: PHR reporting for NB-IoT in GEO

- Agreements

  - Introduce a new PHR reporting table for NB-IoT in GEO and use legacy values in []

    - Note: whether any revision of values is needed can be discussed in the maintenance stage

Issue 2-1: RMC for Cat-M1 with 1.4 MHz

- RAN4 to introduce reference channels for Cat-M1 UEs in Annex A.3.1.3, for both MPDDCH and MPDSCH.

Issue 3: Test for Segmented UL Transmissions

- Agreements

  - Define UE transmit timing accuracy test for UL segmented transmission in HD-FDD NB enhanced coverage test, only NGSO test configuration applies for this test. The test setup and test requirements in A.13.4.1.2.1 (HD-FDD NB1 standalone enhanced coverage test) Test 1 (DRX off) can be the starting point

  - Introduce a similar test for eMTC

Issue 4: Test case list

- Agreements

  - Define intra-frequency RRC Re-establishment TC for both NC and EC for NB-IoT

  - For Cat-M1, introduce test cases of measurement procedure and measurement performance for intra-frequency in GEO, including the following test cases

| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |

Agreements were captured in WF [10], as follows:

UE demodulation requirements

- Big CR to TS36.102 for IoT-NTN UE demodulation requirements is agreed as in [11].

Issue 1: NPDCCH repetition factor in NPDSCH test 2

- Proposals

  - Option 1 (Ericsson): 128

  - Option 2 (Huawei): 256

- Agreement: Option 1 agreed

The agreements as follows:

SAN demodulation requirements

- Big CR to TS 36.108 for IoT over NTN SAN demodulation requirement introduction is agreed as in [12].

Agreements were captured in WF [13], as follows:

# Topic #1: Common issue for eMTC and NB-IoT

How to introduce measurement uncertainties and test tolerance for IoT NTN conducted and radiated performance requirements in TS 36.181

Agreement:

- The eMTC and NB-IoT performance relevant measurement uncertainties and test tolerances defined in TS36.141 Table 4.1.2-3 and Table G.3-1 could be reused for IoT NTN. The content of the table can be further checked.

# Topic #2: SAN demodulation requirement for eMTC

Channel model for PUSCH demodulation requirements

Agreement:

- Both NTN-TDLA100-5 and NTN-TDLC5-5 for PUSCH CE mode A and mode B

Whether a manufactory declaration and an applicability rule are needed for PUSCH CE mode A and B

Agreement:

- Reusing the existing manufacture declaration approach for PUSCH, PUCCH and PRACH requirements in TS 36.104/141.

Preamble parameters for PRACH BF0/1/2/3

Agreement:

- Reuse legacy LTE eMTC PRACH parameters

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

PRACH configuration index (prach-ConfigurationIndex)

Agreement:

- Apply (3, 19, 35, 51) for PRACH format (0, 1, 2, 3) separately. (Same as the existing eMTC PRACH requirements in TS36.104).

# Topic #3: SAN demodulation requirement for NB-IoT

Channel model for NPUSCH format 1 demodulation requirements (3.75kHz single tone and 15kHZ 12 tones)

Agreement:

- NTN_TDLA100-1 and NTN_TDLC5-1

Largest Doppler offset for NPUSCH format 1 (3.75kHz SCS)

Agreement:

- Set 128Hz (to align with PUSCH CE mode B with same transmission duration)

NPRACH preamble test parameters

Agreement:

- Reuse the configuration as the exiting NPRACH test parameters in TS36.141

| Parameters | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |

#### 2.4.2	Remaining Open issues

Performance part

None, all Perf part objectives are complete. Any further discussion can be discussed as part of maintenance activity, if needed.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN4 #106bis-e (Apr. 2023)

- R4-2305949,	 Big TP to TS 36.181 SAN conformance testing, Huawei, HiSilicon

- R4-2305936, WF on remaining issues for the TS 36.181 drafting, Huawei

- R4-2306386, Draft Big CR on NB-IoT/eMTC RRM performance requirements for NTN, MediaTek

- R4-2306370, WF on NB-IoT/eMTC RRM requirements for NTN, MediaTek

- R4-2305961, Big CR to 36.102: Demodulation requirements for IoT-NTN, MediaTek

- R4-2305960, WF on IoT-NTN UE demodulation requirements, MediaTek

- R4-2305965, WF on IoT NTN SAN demodulation requirements, Ericsson

The list of all related TDocs:

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2305715 | Work-split arrangement for TS 36.181 development | Huawei, HiSilicon |
| R4-2305937 | TP for TS 36.181: Clause 4.2~4.5 | ZTE Corporation |
| R4-2305938 | TP for TS 36.181: Clause 4.7~4.8 | ZTE Corporation |
| R4-2305939 | TP for TS 36.181: Clause 4.9 | ZTE Corporation |
| R4-2305940 | TP for TS 36.181: Clause 6.3 and 9.4 | ZTE Corporation |
| R4-2305941 | TP for TS 36.181: Clause 7.3 and 10.4 | ZTE Corporation |
| R4-2305942 | TP for TS 36.181: Clause 7.4 and 10.5 | ZTE Corporation |
| R4-2305943 | TP for TS 36.81: Annex D | ZTE Corporation |
| R4-2305944 | TP to TS 36.181: ACLR and OTA ACLR sections | NEC |
| R4-2305945 | TP to TS 36.181: OBUE and OTA OBUE sections | NEC |
| R4-2305946 | TP to TS 36.181: In-channel selectivity and OTA in-channel selectivity sections | NEC |
| R4-2305947 | TP to TS 36.181: Reference measurement channels | NEC |
| R4-2305948 | TP to TS 36.181: General sections | Huawei, HiSilicon |
| R4-2305949 | Big TP to TS 36.181 SAN conformance testing | Huawei, HiSilicon |
| R4-2305950 | TP to TS 36.181: Annex B | Huawei, HiSilicon |
| R4-2305951 | TP to TS 36.181: MU and TT values | Huawei, HiSilicon |
| R4-2305952 | TP to TS 36.181: Manufacturer declarations (4.6) | Huawei, HiSilicon |
| R4-2305953 | TP to TS 36.181: Requirements for (non-)contiguous spectrum, multi-band, and Reference coordinate system (4.10, 4.11, 4.12) | Huawei, HiSilicon |
| R4-2305954 | TP to TS 36.181: Conducted and OTA SAN output power (6.1, 6.2, 9.1, 9.2, 9.3) | Huawei, HiSilicon |
| R4-2305955 | TP to TS 36.181: Conducted and OTA Tx signal quality (6.5, 9.6, Annex F) | Huawei, HiSilicon |
| R4-2305956 | TP to TS 36.181: Conducted and OTA unwanted emissions: OBW (6.6.1, 6.6.2, 9.7.1, 9.7.2) | Huawei, HiSilicon |
| R4-2305957 | TP to TS 36.181: Conducted and OTA Tx spur (6.6.5, 9.7.5) | Huawei, HiSilicon |
| R4-2305958 | TP to TS 36.181: Conducted and OTA Refsens (7.1, 7.2, 10.1, 10.2, 10.3) | Huawei, HiSilicon |
| R4-2305959 | TP to TS 36.181: Conducted and OTA OOB blocking (7.5, 10.6) | Huawei, HiSilicon |
| R4-2304375 | GNSS position acquisition method of IoT NTN | Qualcomm Korea |
| R4-2304504 | Discussion on remaining open issues for the Perf. Part of IoT NTN | Nokia, Nokia Shanghai Bell |
| R4-2304649 | Discussion on RRM test cases for LTE NB-IoT and eMTC NTN | CMCC |
| R4-2304820 | Discussion on RRM performance requirement for IoT NTN | MediaTek inc. |
| R4-2305060 | DraftCR on TS 36.133 Random Access Test Cases for NB-IoT over NTN | Nokia, Nokia Shanghai Bell |
| R4-2305261 | Discussion on performance requirements for IoT NTN | Huawei, HiSilicon |
| R4-2305262 | DraftCR on timing test cases of NB-IoT for IoT NTN | Huawei, HiSilicon |
| R4-2305735 | Discussions on NTN IoT RRM performance requirements | Ericsson |
| R4-2306376 | Introduction of test cases of NB-IoT Radio Link Monitoring and reference configurations for satellite access | MediaTek inc. |
| R4-2306377 | draft Cat-B CR CQI for NB-IoT NTN | Qualcomm Korea |
| R4-2306378 | draftCR on TCs for random access for eMTC over NTN | Huawei, HiSilicon |
| R4-2306379 | DraftCR on TS 36.133 Timing Test Cases for eMTC over NTN | Nokia, Nokia Shanghai Bell |
| R4-2306380 | RLM test cases for eMTC over NTN | Ericsson |
| R4-2306381 | draft CR on E-UTRAN RLM test for Cat-M1 UE in CEMode A in DRX | CMCC |
| R4-2306382 | Channel quality reporting accuracy test cases for eMTC over NTN | Ericsson |
| R4-2306383 | DraftCR on conditions for UE Cat-M1 bands in annex B | Ericsson |
| R4-2306384 | Draft CR on conditions for NB-IoT bands in annex B | Ericsson |
| R4-2306385 | Introduction of requirement for measurement accuracy and power headroom for Satellite Access | MediaTek inc. |
| R4-2304881 | NTN NB-IoT/eMTC demodulation performance requirements | Qualcomm India Pvt Ltd |
| R4-2304918 | Discussion on IoT-NTN UE demodulation requirements | MediaTek inc. |
| R4-2304919 | Summary of simulation results for IoT-NTN UE demodulation requirements | MediaTek inc. |
| R4-2305175 | Discussion on eMTC/NB-IoT UE demodulation requirements for NTN | Ericsson |
| R4-2305489 | Simulation results on UE demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2305879 | Draft CR to TS36.102, measurement channels for IoT-NTN | Qualcomm |
| R4-2305962 | Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN | MediaTek inc. |
| R4-2305963 | draft CR: Introduction of eMTC UE demodulation requirements for NTN | Ericsson |
| R4-2305964 | Draft CR on IOT NTN demodulation performance requirements (TS36.102, Rel-18) | Huawei,HiSilicon |
| R4-2304046 | Comment on UL Pre-Compensation Gap and Number of Segments | Nokia, Nokia Shanghai Bell |
| R4-2305177 | Simulation results of SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2305178 | Discussion on SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2305180 | Summary of SAN simulation results for IoT-NTN | Ericsson |
| R4-2305490 | Discussion on SAN demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2305491 | Simulation results on SAN demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2305536 | Discussion and initial simulation results for eMTC and NB-IoT over NTN | Samsung |
| R4-2305961 | Big CR to 36.102: Demodulation requirements for IoT-NTN | MediaTek |
| R4-2305936 | WF on remaining issues for the TS 36.181 drafting | Huawei |
| R4-2305960 | WF on IoT-NTN UE demodulation requirements | MediaTek |
| R4-2305965 | WF on IoT NTN SAN demodulation requirements | Ericsson |
| R4-2305985 | Topic Summary [106bis-e][316] IoT_NTN_SANRFConformance | Moderator (Huawei) |
| R4-2305995 | Topic Summary [106bis-e][326] IoT_NTN_Demod_Part1 | Moderator (MediaTek) |
| R4-2305996 | Topic Summary [106bis-e][327] IoT_NTN_Demod_Part2 | Moderator (Ericsson) |
| R4-2306261 | Topic summary for [106-bis-e][230] LTE_NBIOT_eMTC_NTN_req | Moderator (MediaTek) |
| R4-2306370 | WF on NB-IoT/eMTC RRM requirements for NTN | MediaTek |
| R4-2306386 | Draft Big CR on NB-IoT/eMTC RRM performance requirements for NTN | MediaTek |
| R4-2306645 | WF on LTE_NBeMTC_NTN_UERF | MediaTek |

RAN4 #107 (May 2023)

- R4-2309627,	 The final Big TP to TS 36.181 SAN conformance testing, Huawei, HiSilicon

- R4-2310180, Big CR on NB-IoT/eMTC RRM performance requirements for NTN, MediaTek

- R4-2310098, WF on RRM requirements for NTN NB-IoT/eMTC, MediaTek

- R4-2309904, Big CR to TS36.102: Introduction of IoT-NTN UE demodulation requirements, MediaTek

- R4-2309903, Big CR to TS 36.108 for IoT over NTN SAN demodulation requirement introduction, Ericsson

- R4-2309827, WF for IoT over NTN SAN demodulation requirements, Ericsson

The list of all related TDocs:

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2309601 | about NB IoT NTN SAN MU TT values | Keysight Technologies UK Ltd |
| R4-2309627 | The final Big TP to TS 36.181 SAN conformance testing | Huawei, HiSilicon |
| R4-2307495 | TP to TS 36.181: Characteristics of the interfering signals | NEC |
| R4-2307496 | TP to TS 36.181: Output power dynamics | NEC |
| R4-2309180 | TP for TS 36.181: Clause 4.8 | ZTE Corporation |
| R4-2309628 | Cleanup TP to TS 36.181 | Huawei, HiSilicon |
| R4-2309732 | TP to TS 36.181: test models clarifications (4.9) | Huawei, HiSilicon |
| R4-2309733 | TP to TS 36.181: Protection of the own Satellite Access Node receiver (6.6.5.6) | Huawei, HiSilicon |
| R4-2309734 | TP to TS 36.181: In-channel Tx test (Annex G) | Huawei, HiSilicon |
| R4-2307601 | Discussion on RRM test cases for LTE NB-IoT and eMTC NTN | CMCC |
| R4-2307602 | Draft CR on E-UTRAN RLM in DRX test and HO and CHO test for Cat-M1 UE in CEMode A | CMCC |
| R4-2307896 | Discussion on RRM performance requirement for IoT NTN | MediaTek inc. |
| R4-2307897 | Draft Big CR on NB-IoT/eMTC RRM performance requirements for NTN | MediaTek inc. |
| R4-2307898 | Introduction of test cases of Cell Re-Selection for NB-IoT | MediaTek inc. |
| R4-2307899 | Introduction of test cases of Intra-frequency Measurements and RSRP for M1 | MediaTek inc. |
| R4-2308313 | Discussion on performance requirements for IoT NTN | Huawei, HiSilicon |
| R4-2308314 | DraftCR on RRC reestablishment test cases of NB-IoT for GEO | Huawei, HiSilicon |
| R4-2308355 | Discussion on performance aspects of IoT NTN | Nokia, Nokia Shanghai Bell |
| R4-2308357 | DraftCR on Cell Reselection Test cases for M1 UEs in satellite access | Nokia, Nokia Shanghai Bell |
| R4-2308359 | CR on reference configurations for Cat-M1 in Satellite Access | Nokia, Nokia Shanghai Bell |
| R4-2309222 | Discussions on NTN IoT RRM performance requirements | Ericsson |
| R4-2309223 | RRC re-establishment test cases for eMTC over NTN | Ericsson |
| R4-2309224 | RMC for eMTC test cases over NTN | Ericsson |
| R4-2310180 | Big CR on NB-IoT/eMTC RRM performance requirements for NTN | MediaTek |
| R4-2307055 | Simulation results for NB-IoT NTN | Lockheed Martin |
| R4-2307116 | Draft CR on IoT-NTN demodulation performance requirements | MediaTek inc. |
| R4-2307436 | NTN NB-IoT/eMTC demodulation performance requirements | QUALCOMM Europe Inc. - Spain |
| R4-2308424 | Discussion on eMTC/NB-IoT UE demodulation requirements for NTN | Ericsson |
| R4-2308894 | Discussion on UE demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2308895 | Draft CR on IOT NTN demodulation performance requirements (TS36.102, Rel-18) | Huawei,HiSilicon |
| R4-2308896 | Simulation results on UE demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2307021 | pCR for introduction of SAN Demodulation requirements | Nokia, Nokia Shanghai Bell |
| R4-2307022 | Discussion on SAN demodulation | Nokia, Nokia Shanghai Bell |
| R4-2308425 | Simulation results of SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2308426 | Discussion on SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2308427 | draft CR: Introduction of SAN demodulation requirements for IoT-NTN | Ericsson |
| R4-2308428 | Summary of SAN simulation results for IoT-NTN | Ericsson |
| R4-2308828 | Discussion and initial simulation results for eMTC and NB-IoT over NTN | Samsung |
| R4-2308829 | Draft CR on SAN demodulation requirements for NB-IoT over NTN | Samsung |
| R4-2308897 | Discussion on SAN demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2308898 | Simulation results on SAN demodulation requirements for LTE NTN IOT | Huawei,HiSilicon |
| R4-2308899 | pCR on IOT NTN demodulation performance requirements (TS36.181, Rel-18) | Huawei,HiSilicon |
| R4-2309981 | Topic summary for [107][236] LTE_NBIOT_eMTC_NTN_req | Moderator (MediaTek) |
| R4-2310453 | Summary for [107][316] IoT_NTN_SANRFConformance | Moderator, Huawei |
| R4-2310466 | Summary for [107][329] IoT_NTN_Demod_Part1 | Moderator, Mediatek |
| R4-2310467 | Summary for [107][330] IoT_NTN_Demod_Part2 | Moderator, Ericsson |