3GPP TSG RAN meeting #100										RP-231314

Taipei, Taiwan, June 12-14, 2023

## Status Report to TSG

Agenda item:			9.3.4.7

| WI / SI Name | Further enhancements on NR and MR-DC measurement gaps and measurements without gaps |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MG_enh2 |  |  |  |  |
| Unique ID | 950082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230755 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 65% RAN4: 65% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ato (Tsang-Wei) YU, Meng ZHANG |
|  | Company | MediaTek Inc., Intel Corporation |
|  | Email | ato.yu@mediatek.com, meng.zhang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #106b (Apr, 2023, Electronic)

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following agreements were captured in the WF [67]

  - General issues

    - If Type-1 MG is considered in this WI, similar to that of Rel-17 Concurrent MG WI, “no requirements” could be applied for the scenario when Type-1 MG is colliding with Pre-MG/ NCSG in the same FR.

  - Case 1 (Pre-configured MG and concurrent MG)

    - Gap combinations

      - For UE capable of [Concurrent Pre-MG], one FR can be configured with up to 2 Pre-MGs, regardless they are per-UE or per-FR configured and the configuration in the other FR.

      - UE capabilities for [Concurrent Pre-MG] are FFS.

      - FFS the wording of the notes to be added in the specification.

      - Discuss the notes for the detail measurement gaps combinations for UE supporting per-FR gap during the CR drafting phase

    - Pre-MGs activation/deactivation procedure

      - The fully overlapped simultaneous multiple Pre-MGs activation/deactivation delay equals the BWPs/SCells/RRC reconfiguration delay plus existing processing time (5ms) plus the additional post-processing time T1, where T1 value is FFS.

  - Case 2 (NCSG and concurrent MG)

    - Gap combinations

      - For UE capable of [Concurrent NCSG], one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured and the configuration in the other FR.

      - UE capabilities for [Concurrent NCSG] are FFS.

      - For gap combination configuration of case 2, the per-UE gap in gap combination #3, #4 and #5 is associated with PRS measurement and cannot be NCSG.

      - FFS the wording of the notes to be added in the specification

      - Discuss the notes for the detail measurement gaps combinations for UE supporting per-FR gap during the CR drafting phase.

- RRM core requirements for measurements without gaps

  - The following agreements were captured in the WF [68]

  - Requirements of NeedforGap

    - Interruption ratio under single frequency layer

    - When RAN4 defining the measurement requirements for intra/inter-freq measurement without gap when interruption allowed (case 2), the following key aspects needs to be updated at least.

      - Updated the definition of intra/inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘nogap-withinterruption [TBD]’ via ‘needForGap-r18 [TBD]’

      - Updated the scaling factor because of the measurement gap overlapping (Kp)

      - Updates on CSSFoutside_gap

      - Updates on Klayer1_measurement

    - When RAN4 defining the measurement requirements for intra/inter-freq measurement without gap when interruption not allowed (case 1), reusing 9.3.9 framework in TS38.133.

    - All the scheduling causes in 9.3.10.3 could be considered when we define the scheduling requirements for NFG, including

      - whether the UE supports simultaneousRxTxInterBandCA in FR1

      - whether the UE supports simultaneousRxDataSSB-DiffNumerology for intra-band

      - whether IBM is supported in FR2

    - Scheduling restriction when UE indicated ‘nogap-withinterruption [TBD]’ and ‘nogap-withoutinterruption [TBD] via ‘NeedforGaps-r18[TBD] shall be defined. Take NCSG requirements as a starting point

  - Inter-RAT gapless measurements

    - Reuse NeedForNCSG-InfoEUTRA-r17 to support Case b-1. Define requirements for case when UE reports “nogap-noncsg” in NeedForNCSG-InfoEUTRA-r17 for indicating no-gap without interruption.

    - No additional UE capability is defined for inter-RAT measurement with mixed numerology; instead it can be considered for scheduling restriction

    - Inter-RAT NR measurement cases with gap

      - RAN4 needs not to define any new requirements for inter-RAT measurements when UE support the gap based measurement only (e.g. indicated by “gap”) in Rel18 WI.

      - The existing requirements in TS38.133 9.4.2&3 and TS36.133 8.1.2.4.21&22 can be completely reused for inter-RAT LTE and inter-RAT NR measurement respectively.

    - The existing requirements in TS38.133 9.4.2&9.4.3 can be reused for for case b-1 when no gap but interruption allowed.

    - The existing scheduling restriction requirements in clause 9.3.10.3 can be taken as baseline for the inter-RAT NR measurement without measurement gaps. With this agreed baseline, companies can provide the more detail proposals on how to define the scheduling restriction in the next meeting

RAN4 #107 (May, 2023, Incheon, South Korea)

- The CR worksplit was agreed in [135]

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following agreements were captured in the WF [138]

  - Case 1 (Pre-configured MG and concurrent MG)

    - Pre-MGs activation/deactivation procedure

      - For fully overlapped simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG, support the following cases

        - Case a: Both Pre-MGs are triggered by the same event.

        - Case b: Two Pre-MGs are triggered by two events of the same type at the same time.

      - Under the assumption that the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated, extend the delay by T1 ms for fully overlapped simultaneous activation/deactivation for Pre-MG + Pre-MG

        - T1 = 2ms.

        - FFS if this activation delay collide with existing gaps

    - Collision handling

      - Collision and priority rule on Pre-MG are considered only when Pre-MG is activated (i.e., deactivated Pre-MG is not considered in collisions)

      - A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG [and UE should continue the measurement within the MG#2]

        - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used

    - Requirement

      - UE is allowed to restart measurement upon Pre-MG status change

  - Case 2 (NCSG and concurrent MG)

    - Do not support parallel measurements upon NCSGs collision in Rel-18 MG Enhancements WI.

    - •	Upon gap collision, RAN4 not to consider additional enhancements on top of the existing agreements.

- RRM core requirements for measurements without gaps

  - The following agreements were captured in the WF [134] and LS [137] to RAN2

  - Requirements of NeedforGap

    - Do not define any restriction on interruption location

  - Inter-RAT gapless measurements

    - The same requirements apply for both CRS rate matching and CRS-IM use cases

    - UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

      - Introduce a new per-UE capability to support case b-2 similar as Rel-16 inter-frequency measurement without gap

      - “No gap with interruption” is not considered for case b-2

      - No interruption is considered for case b-2

      - FFS on power imbalance side conditions for Case b-2 measurements

      - FFS whether scheduling restriction shall be defined for inter-RAT LTE measurement case b-2 with mixed numerology.

    - Introduce the effective measurement window for inter-RAT LTE measurement, including offset, duration and periodicity.

      - The effective measurement window is used to determine the location of scheduling and measurement restriction.

      - FFS whether the effective measurement window is used to handle the collision between the SSB and LTE measurement

      - Candidate values for effective measurement window duration: 5ms

        - Other values are not precluded

      - Candidate values for periodicity of effective measurement window: 40ms and 80ms

        - Other values are not precluded

#### 2.4.2	Remaining Open issues

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following open issues were captured in the WF [67] and [138]

  - Case 1 (Pre-configured MG and concurrent MG)

    - Detail notes in gap combination

    - Detail in simultaneous pre-MGs activation/deactivation delay, including the scenarios and the duration for additional time extension

      - Scenarios to be considered for partially overlapped simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG and the corresponding delay

    - Extension of pre-MGs activation/deactivation delay when overlapping with the other MG

    - Collision rule: Whether de-activated pre-MG is considered in collisions handling and whether to introduce corresponding UE capability

      - Collision handling on other cases without conclusions and the corresponding UE capability, if needed

    - Detail in delay requirements

  - Case 2 (NCSG and concurrent MG)

    - Whether to introduce UE capability for NCSG + NCSG in an FR

    - Whether to consider parallel measurements upon two NCSG collision, including whether same RF chain is assumed for 2 NCSGs and whether to introduce a corresponding UE capability

    - Collision handling

    - Clarification of UE behavior and potential changes for NCSG during deactivated SCell and upon SCell activation

    - Gap interruption

    - Whether per FR PRS gap should be considered in case 2 NCSG and conMG

- RRM core requirements for measurements without gaps

  - The following open issues were captured in the WF [68] and [134]

  - Requirements of NeedforGap

    - Interruption length, ratio (under multiple frequency layers and/or DRX) and potential location limitation

    - Whether to introduced a NW indication of measurement cycle.

    - UE behaviour when UE reports ‘no gap with interruption’ in some/all bands and NW configures MG

    - Detail of measurement delay requirements when interruptions are allowed

    - Detail of measurement delay requirements when interruptions are not allowed

    - TBD whether to introduce mapping between NeedForGap and NCSG capabilities when UE supports both of them

    - TBD whether a default SMTC pattern should be defined to restrict the scheduling restriction occasions

  - Inter-RAT gapless measurements

    - FFS whether to differentiate Inter-RAT LTE measurement for LTE CRS rate-matching feature and for CRS-IM receiver

    - TBD whether to introduce a per-UE capability to support Case b-2, including indication of “no gap with interruption” because of AGC,T/F offset issue

    - Whether searcher limitation is needed. E.g., for inter-RAT LTE without gap (case b-2) to be performed in parallel with NR measurement

    - Detail in measurement delay requirements, including an effective window for measurement

    - Scheduling restriction due to inter-RAT LTE measurement

    - Interruption requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN4 #106b (Apr, 2023, Electronic)

- R4-2304074	Further consideration on scope and general issues for Pre-MG, concurrent MGs and NCSG	vivo

- R4-2304230	Discussion on using scenarios for pre-configured MGs, multiple concurrent MGs and NCSG	Intel Corporation

- R4-2304284	On R18 gap enhancement scope and general issues	Apple

- R4-2304990	Discussion on general and scope of enhancements on measurement gaps	ZTE Corporation

- R4-2305665	On joint requirements for Rel-17 measurement gap enhancements - scope and general issues	Qualcomm Incorporated

- R4-2305684	Discussion on general issues for Case 1 and Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2304075	Further consideration on case 1 requirements for Pre-MG and concurrent MGs	vivo

- R4-2304231	Discussion on RRM requirements for combinations of pre-configured MGs and multiple concurrent MGs	Intel Corporation

- R4-2304285	On R18 gap enhancement case 1 requirements	Apple

- R4-2304416	Discussion on case 1 requirements for combination of pre-MG and concurrent MGs	CATT

- R4-2304590	Discussion on case 1 requirements (Pre-configured MG and concurrent MG)	MediaTek inc.

- R4-2304763	RRM requirements for the combination of Pre-MG and concurrent MG	Xiaomi

- R4-2304837	Discussion on Pre-MG MG and concurrent MG (case 1)	CMCC

- R4-2304888	Discussion on PreMG and ConMGs	Ericsson

- R4-2304991	Discussion on Case 1 RRM requirements	ZTE Corporation

- R4-2305023	Discussion on Case 1 requirements for Pre-MG and concurrent MG	China Telecom

- R4-2305215	Discussion on case 1 requirements for Pre-MG and concurrent MG	OPPO

- R4-2305326	Discussion on joint working of pre-MG and con-MG	Huawei, HiSilicon

- R4-2305666	On joint requirements for Rel-17 measurement gap enhancements - Case 1	Qualcomm Incorporated

- R4-2305685	Discussion on Case 1 requirements	Nokia, Nokia Shanghai Bell

- R4-2304076	Further consideration on case 2 requirements for concurrent MGs and NCSG	vivo

- R4-2304232	Discussion on RRM requirements for combinations of NCSG and multiple concurrent MGs	Intel Corporation

- R4-2304286	On R18 gap enhancement case 2 requirements	Apple

- R4-2304417	Discussion on case 2 requirements for combination of NCSG and concurrent MGs	CATT

- R4-2304481	Discussion on RRM core requirement for case 2 (NCSG and concurrent MG)	LG Electronics Finland

- R4-2304591	Discussion on case 2 requirements (NCSG and concurrent MG)	MediaTek inc.

- R4-2304764	RRM requirements for the combination of NCSG and concurrent MG	Xiaomi

- R4-2304838	Discussion on NCSG and concurrent MG (case 2)	CMCC

- R4-2304889	Discussion on NCSG and ConMGs	Ericsson

- R4-2304992	Discussion on Case 2 RRM requirements	ZTE Corporation

- R4-2305024	Discussion on Case 2 requirements for NCSG and concurrent MG	China Telecom

- R4-2305216	Discussion on case 2 requirements NCSG and conMG	OPPO

- R4-2305327	Discussion on joint working of NCSG and con-MG	Huawei, HiSilicon

- R4-2305667	On joint requirements for Rel-17 measurement gap enhancements - Case 2	Qualcomm Incorporated

- R4-2305686	Discussion on Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2304055	Discussion on measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2304077	Further considerations on Measurement without gaps for UEs reporting NeedForGapsInfoNR	vivo

- R4-2304233	Discussion on measurements without gaps when UE reporting NFG	Intel Corporation

- R4-2304287	On measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2304288	LS on measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2304382	Discussion on requirements for measurement without gaps for R18needforgap	Qualcomm Incorporated

- R4-2304418	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	CATT

- R4-2304592	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	MediaTek inc.

- R4-2304775	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	Xiaomi

- R4-2304839	Discussion on measurements without gaps for UEs reporting NeedForGapsInfoNR	CMCC

- R4-2304890	Discussion on NeedForGaps measurement	Ericsson

- R4-2304993	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	ZTE Corporation

- R4-2305217	On measurement without gaps for UEs reporting NeedForGapsInfoNR	OPPO

- R4-2305328	Discussion on requirements for NeedForGaps	Huawei, HiSilicon

- R4-2304078	"Further considerations on 	Inter-RAT measurement without gap"	vivo

- R4-2304214	Discussion on Inter-RAT measurements	Nokia, Nokia Shanghai Bell

- R4-2304234	Discussion on inter-RAT measurement without gaps	Intel Corporation

- R4-2304289	On R18 inter-RAT measurement without gap	Apple

- R4-2304383	Discussion on requirements of inter-RAT measurement without gaps.	Qualcomm Incorporated

- R4-2304419	Discussion on RRM requirements for Inter-RAT measurement without gap	CATT

- R4-2304593	Discussion on inter-RAT measurements	MediaTek inc.

- R4-2304776	Discussion on inter-RAT measurement without gap	Xiaomi

- R4-2304840	Discussion on inter-RAT measurements without gaps	CMCC

- R4-2304891	Discussion on Inter-RAT measurement without gap	Ericsson

- R4-2304994	Discussion on inter-RAT measurement without gaps	ZTE Corporation

- R4-2305218	On RRM requirements for Inter-RAT measurement without gap	OPPO

- R4-2305329	Discussion on inter-RAT MG-less measurement	Huawei, HiSilicon

- R4-2306154	Topic summary for [106-bis-e][206] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2306155	Topic summary for [106-bis-e][207] NR_MG_enh2_part2	Moderator (Intel)

- R4-2306237	Topic summary for [106-bis-e][206] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2306238	Topic summary for [106-bis-e][207] NR_MG_enh2_part2	Moderator (Intel)

- R4-2306330	WF on Rel-18 NR MG enhancements (part 1)	MediaTek

- R4-2306331	WF on Rel-18 NR MG enhancements (part 2)	Intel Corporation

- R4-2306332	LS on interRAT measurements without gap	Intel Corporation

RAN4 #107 (May, 2023, Incheon, South Korea)

- R4-2309562	General discussion for CR work split	MediaTek inc., Intel Corporation

- R4-2307168	Draft CR to 38.101-1 CA_n41-n79	Huawei, HiSilicon, CMCC

- R4-2307442	Consideration on issues on scope and general issues for Pre-MG, concurrent MGs and NCSG	vivo

- R4-2307635	Discussion on scope and general issues of R18 gap enhancement	Apple

- R4-2308720	Discussion on general and scope of enhancements on measurement gaps	ZTE Corporation

- R4-2309123	On joint requirements for Rel-17 measurement gap enhancements - scope and general issues	Qualcomm Incorporated

- R4-2309673	Discussion on general issues for Case 1 and Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2307406	Discussion on case 1 requirements for combination of pre-MG and concurrent MGs	CATT

- R4-2307443	Consideration on issues  on case 1 requirements for Pre-MG and concurrent MGs	vivo

- R4-2307552	Discussion on Pre-MG MG and concurrent MG (case 1)	CMCC

- R4-2307636	Discussion on case 1 requirements of R18 gap enhancement	Apple

- R4-2307946	RRM requirements for the combination of Pre-MG and concurrent MG	Xiaomi

- R4-2308018	Discussion on Case 1 requirements for Pre-MG and concurrent MG	China Telecom

- R4-2308441	Discussion on PreMG and ConMGs	Ericsson

- R4-2308461	Discussion on case 1 requirements for Pre-MG and concurrent MG	OPPO

- R4-2308658	Discussion on joint working of pre-MG and con-MG	Huawei, HiSilicon

- R4-2308721	Discussion on Case 1 RRM requirements	ZTE Corporation

- R4-2309124	On joint requirements for Rel-17 measurement gap enhancements - Case 1	Qualcomm Incorporated

- R4-2309563	Discussion on case 1 requirements (Pre-configured MG and concurrent MG)	MediaTek inc.

- R4-2309674	Discussion on Case 1 requirements	Nokia, Nokia Shanghai Bell

- R4-2307407	Discussion on case 2 requirements for combination of NCSG and concurrent MGs	CATT

- R4-2307433	Discussion on RRM core requirement for case 2 (NCSG and concurrent MG)	LG Electronics Inc.

- R4-2307444	Consideration on issues on case 2 requirements for concurrent MGs and NCSG	vivo

- R4-2307553	Discussion on NCSG and concurrent MG (case 2)	CMCC

- R4-2307637	Discussion on case 2 requirements of R18 gap enhancement	Apple

- R4-2307947	RRM requirements for the combination of NCSG and concurrent MG	Xiaomi

- R4-2308019	Discussion on Case 2 requirements for NCSG and concurrent MG	China Telecom

- R4-2308442	Discussion on NCSG and ConMGs	Ericsson

- R4-2308462	Discussion on case 2 requirements NCSG and conMG	OPPO

- R4-2308659	Discussion on joint working of NCSG and con-MG	Huawei, HiSilicon

- R4-2308722	Discussion on Case 2 RRM requirements	ZTE Corporation

- R4-2309125	On joint requirements for Rel-17 measurement gap enhancements - Case 2	Qualcomm Incorporated

- R4-2309564	Discussion on case 2 requirements (NCSG and concurrent MG)	MediaTek inc.

- R4-2309675	Discussion on Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2307191	Discussion on measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2307408	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	CATT

- R4-2307445	Considerations on issues on measurement without gaps for UEs reporting NeedForGapsInfoNR	vivo

- R4-2307554	Discussion on measurements without gaps for UEs reporting NeedForGapsInfoNR	CMCC

- R4-2307638	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2307805	Discussion on measurements without gaps for UE reporting NFG	Intel Corporation

- R4-2307958	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	Xiaomi

- R4-2308443	Discussion on NeedForGaps measurement	Ericsson

- R4-2308463	On measurement without gaps for UEs reporting NeedForGapsInfoNR	OPPO

- R4-2308660	Discussion on requirements for NeedForGaps	Huawei, HiSilicon

- R4-2308723	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	ZTE Corporation

- R4-2309423	Discussion on requirements for measurement without gap	Qualcomm Incorporated

- R4-2309565	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	MediaTek inc.

- R4-2307192	Discussion on interRAT measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2307409	Discussion on RRM requirements for Inter-RAT measurement without gap	CATT

- R4-2307446	Consideration on issues on inter-RAT measurement without gap	vivo

- R4-2307555	Discussion on inter-RAT measurements without gaps	CMCC

- R4-2307639	Discussion on R18 inter-RAT measurement without gap	Apple

- R4-2307806	Discussion on inter-RAT measurement without gaps	Intel Corporation

- R4-2307959	Discussion on inter-RAT measurement without gap	Xiaomi

- R4-2308444	Discussion on Inter-RAT measurement without gap	Ericsson

- R4-2308464	On RRM requirements for Inter-RAT measurement without gap	OPPO

- R4-2308661	Discussion on inter-RAT MG-less measurement	Huawei, HiSilicon

- R4-2308724	Discussion on inter-RAT measurement without gaps	ZTE Corporation

- R4-2309424	Disccusion on requirements of inter-RAT measurement without gaps.	Qualcomm Incorporated

- R4-2309566	Discussion on inter-RAT measurements	MediaTek inc.

- R4-2309956	Topic summary for [107][211] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2309957	Topic summary for [107][212] NR_MG_enh2_part2	Moderator (Intel)

- R4-2310044	Ad-hoc minutes for NR and MR-DC measurement gaps and measurements without gaps	Moderator (MediaTek)

- R4-2310051	WF on NR MG enhancements	MediaTek

- R4-2310052	WF on measurements without gaps	Intel

- R4-2310053	WF on the CR work split for MG Enhancements WI	MediaTek, Intel

- R4-2310054	LS on inter-RAT measurements without gaps	Intel

- R4-2310158	LS on inter-RAT measurements without gaps	Intel

- R4-2310175	WF on NR MG enhancements	MediaTek

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template