3GPP TSG RAN meeting #100	RP-231315

Taipei, Taiwan, June 12-14, 2023

## Status Report to TSG

Agenda item:			10.3.6

| WI / SI Name | Introduction of FDD LTE band (L+S band) for IoT NTN operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_FDD_LS_band |  |  |  |  |
| Unique ID | 991043 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230792 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 30% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Daniel Hsieh |
|  | Company | MediaTek Inc. |
|  | Email | daniel.hsieh@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106bis-e

- Work plan for WI IoT_NTN_FDD_LS_band core part

Approve revised R4-2304496 with following modification:

  - RAN4#108bis (October 2023, F2F)

- Draft TR for IoT NTN bands

  - The Draft TR skeleton R4-2304497 is endorsed

- Sytem parameter, UE RF and CR

  - Band 254 revised to Band [254]

  - The addition of Band [254] does not have any impact on the channel bandwidth sub-clauses for category M1 or category NB1/NB2.

  - Channel spacing sub-clauses keep untouched with the addition of Band [254] for category M1 and category NB1/NB2.

  - Channel raster:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [254] | 100 | 2483.5 | 228571 | 228571-<1>-228735 | 1610 | 261339 | 261339-<1>-261503 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

- RAN4 to set the default Tx-Rx frequency separation for symmetric UL/DL operation for Band 254 as 873.5 MHz as starting point and discuss variable duplex and asymmetric UL/DL operation next RAN4#107 meeting if there are more inputs.

- UE RF requirements

- UE maximum output power: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Tables below, respectively. And the inclusion of PC5 is under study.

Table: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [254] |  |  | 23 | +/-2 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [254] | 23 | +/-2 |  |  |

- UE maximum output power reduction: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2 as the starting point

- In-band blocking: Reuse the 36.102 in-band blocking requirements

- Narrow-band blocking: Reuse the 36.102 narrow-band blocking requirements

- RX spurious emission: Reuse the 36.102 spurious emission requirements

RAN4#107

- For minimum output power, should the current TS 36.102 requirements be reused for band b254

- For transmit OFF power, should the current TS 36.102 requirements be reused for band b254

- For ON/OFF time mask, should the current TS 36.102 requirements be reused for band b254

- For protected bands for UE coexistence, should the UE co-existence emission requirements for band b255 be reused as a baseline and additional bands of NR band n54, n105 and E-UTRA band 31, 54, 72, 73, 87, 88, 103 should be added for band b254.

- For maximum input level, should the current TS 36.102 requirements be reused for band b254

- R4-2310296 endorsed as running draft CR for TS 36.102

- R4-2308596 endorsed for TS 36.108

#### 2.4.2	Remaining Open issues

- Remaining UE RF requirements.

- Common TR for all IoT NTN bands.

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN WG4#106bis-e meeting

- R4-2304496	Workplan for IoT NTN FDD LS band, Mediatek

- R4-2306581	Workplan for IoT NTN FDD LS band, Mediatek

- R4-2304497	Draft of TR Skeleton for IoT NTN bands, Mediatek

- R4-2305698	System parameters for L+S band for IoT NTN, MediaTek

- R4-2304491	Discussion on NTN FDD band (L+S band) UE RF requirements, Mediatek

- R4-2304802	Discussion on UE RF requirements for FDD band (L+S band) for IoT NTN operation, ZTE Corporation

- R4-2304803	Draft CR to TS36.102 Introduction of a new FDD band (L+S band) for IoT NTN operation, ZTE Corporation

- R4-2306580	Draft CR to TS36.102 Introduction of a new FDD band (L+S band) for IoT NTN operation, ZTE Corporation

- R4-2304804	Discussion on SAN RF requirements for FDD band (L+S band) for IoT NTN operation, ZTE Corporation

- R4-2304805	Draft CR to TS36.108 Introduction of a new FDD band (L+S band) for IoT NTN operation, ZTE Corporation

- R4-2306656	Draft CR to TS36.108 Introduction of a new FDD band (L+S band) for IoT NTN operation, ZTE Corporation

- R4-2306199	Topic summary for [106-bis-e][119] IoT_NTN_FDD_LS_band, Moderator (Mediatek)

- R4-2306282	Topic summary for [106-bis-e][119] IoT_NTN_FDD_LS_band, Moderator (Mediatek)

- R4-2306579	WF on IoT_NTN_FDD_LS_band, MediaTek

## 4.2	RAN WG4#107 meeting

- R4-2308391 	General aspects for IoT NTN band	MediaTek Inc.

- R4-2308392 	Further discussion on system parameters for L+S IoT NTN band	MediaTek Inc.

- R4-2308393 	Further discussion on UE RF requirements for L+S IoT NTN band	MediaTek Inc.

- R4-2308394 	Draft running CR to TS 36.102 on introducing L+S FDD band for IoT NTN operation	MediaTek Inc.

- R4-2308395 	TP to TR 36.xxxx on introducing L+S FDD band for IoT NTN operation	MediaTek Inc.

- R4-2308396 	Revised WID: FDD band (L+S band) for IoT NTN operation	MediaTek Inc.

- R4-2308594 	Discussion on UE RF requirements for FDD band (L+S band) for IoT NTN operation	ZTE Corporation

- R4-2308595 	CR to TS36.102 Introduction of a new FDD band (L+S band) for IoT NTN operation	ZTE Corporation

- R4-2308596 	CR to TS36.108 Introduction of a new FDD band (L+S band) for IoT NTN operation	ZTE Corporation

- R4-2310006 	Topic summary for [107][123] IoT_NTN_FDD_LS_band, Moderator (MediaTek)

- R4-2310297 	WF on IoT_NTN_FDD_LS_band, MediaTek

- R4-2310296 	Draft running CR to TS 36.102 on introducing L+S FDD band for IoT NTN operation, MediaTek

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template