3GPP TSG-RAN4 Meeting #107	R4-2308934

# Incheon, Korea,  May 22nd  - May 26th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3324 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update (Rel-17) of RRM DCCA SCell activation and deactivation test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Perf |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some DCCA parameters and tables require correction for completing them in RAN5 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Defining A3-offset parameter for A.4.5..3.1/4 and A.6.5.3.1/4/5. - T3 time period is removed in A.4.5.3.5 - Noc unit is corrected in A.6.5.3.4 - Corrected 2 NR carriers instead of three carriers in A.6.5.3.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Above changes are required to define the test cases correctly in TS 38.533 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.3.1, A.4.5.3.5, A.6.5.3.1, A.6.5.3.4, A.6.5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

## A.4	EN-DC tests with all NR cells in FR1

## A.4.5	Signaling characteristics

### A.4.5.3	SCell Activation and Deactivation Delay

#### A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle

##### A.4.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations for LTE PCell and NR PSCell are shown in table A.4.5.3.1.1-1 below. Supported test configurations for NR SCell are shown in table A.4.5.3.1.1-1A below. Test configuration for LTE PCell and NR PSCell and test configuration for NR SCell are chosen independently. The test parameters are given in Tables A.4.5.3.1.1-2 and cell-specific parameters in A.4.5.3.1.1-3 and A.4.5.3.1.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes configured on NR. The UE now starts monitoring the SCell. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PSCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [14] clause 7.32.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any PSCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any E-UTRA PCell interruption due to the deactivation shall occur in the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length, where n1 and n2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot n.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.4.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell3 timing offset to cell2 | s | 0 |  |
| Time alignment error between cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | k1NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| K | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

## << Unskipped Sections Skipped >>

#### << Start of Second change >>

#### A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1

##### A.4.5.3.5.1	Test Purpose and Environment

The purpose of this test is to verify that the direct SCell activation time is within the requirements stated in clause 8.3.4, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations for LTE PCell and NR PSCell are shown in table A.4.5.3.5.1-1 below. The supported test configurations for NR SCell are shown in table A.4.5.3.5.1-1A below. Test configuration for LTE PCell and NR PSCell and test configuration for NR SCell are chosen independently. The test parameters are given in Tables A.4.5.3.5.1-2 and cell-specific parameters in A.4.5.3.5.1-3 and A.4.5.3.5.1-4 below. The test consists of two successive time periods, with duration of T1 and T2, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the measurement on Cell 3 is configured. The UE now starts measuring the Cell 3. During T1, Cell 3 should be detected and measured by the UE such that it meets the condition for known cell defined in clause 8.3.4 for direct SCell activation. At the end of T1, the test equipment sends an RRC message for direct SCell activation of the Cell 3.

The point in time at which the RRC message for direct SCell actvation is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+NdirectNR slot length, as defined in clause 8.3.4. The UE shall start reporting CSI in PSCell in slot (m+k+TRRC_process) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in the slot m+1 to slot m+1+TRRC_Process+T1+TXNR slot length+Ninterruption, as defined in clause 8.3.4, where Ninterruption is the interruption length given in clause 8.2. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1 to subframe m2+1+TRRC_Process+T1+TXNR slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [14] clause 7.32.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation of SCell.

The test equipment verifies the activation time by counting the slots from the time when the direct SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.4.5.3.5.1-2: General test parameters for known FR1 direct SCell activation

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| SCell |  | Cell 3 | Secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every four slots. |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell3 timing offset to cell2 | s | 0 |  |
| Time alignment error between cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the Cell 3 shall be known. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.4.5.3.5.1-3: Cell specific test parameters for NR PSCell for known FR1 direct SCell activation

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| SSB ARFCN |  |  | freq1 |  |
| Duplex mode | Config 1,4 |  | FDD |  |
|  | Config 2,3,5,6 |  | TDD |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 2,5 |  | 10: NRB,c = 52 |  |
|  | Config 3,6 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration | Config 1,2,3,4,5,6 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | Config 1,2,3,4,5,6 |  | DLBWP.1.1 |  |
| UL initial BWP configuration | Config 1,2,3,4,5,6 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | Config 1,2,3,4,5,6 |  | ULBWP.1.1 |  |
| DRx Cycle |  | ms | Not Applicable |  |
| PDSCH Reference | Config 1,4 |  | SR.1.1 FDD |  |
| measurement channel | Config 2,5 |  | SR.1.1 TDD |  |
|  | Config 3,6 |  | SR.2.1 TDD |  |
| RMSI CORESET | Config 1,4 |  | CR.1.1 FDD |  |
| Reference Channel | Config 2,5 |  | CR.1.1 TDD |  |
|  | Config 3,6 |  | CR.2.1 TDD |  |
| RMC CORESET | Config 1,4 |  | CCR.1.1 FDD |  |
| Reference Channel | Config 2,5 |  | CCR.1.1 TDD |  |
|  | Config 3,6 |  | CCR.2.1 TDD |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration |  |  | SMTC.1 |  |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 |  |
|  | Config 3,6 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH | Config 1,2,4,5 | kHz | 15 kHz |  |
| subcarrier spacing | Config 3,6 |  | 30kHz |  |
| CSI-RS configuration for CSI reporting | Config 1,4 |  | CSI-RS.1.1 FDD |  |
|  | Config 2,5 |  | CSI-RS.1.1 TDD |  |
|  | Config 3,6 |  | CSI-RS.2.1 TDD |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 |  |
|  | Config 3,6 |  | 30 |  |
| reportConfigType | Config 1-6 |  | periodic |  |
| reportQuantity | Config 1-6 |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | Config 1,2,4,5 | slot | 5 |  |
|  | Config 3,6 |  | 10 |  |
| CSI reporting offset | Config 1,2,4,5 | slot | 2 |  |
|  | Config 3,6 |  | 4 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 |  |
| Note2 | Config 1,2,4,5 | dBm/SCS | -104 |  |
|  | Config 3,6 |  | -101 |  |
|  |  | dB | 17 |  |
|  |  | dB | 17 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | dBm/SCS | -87 |  |
|  | Config 3,6 |  | -84 |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |
| Propagation condition |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |

Table A.4.5.3.5.1-4: Cell specific test parameters for NR SCell for known FR1 direct SCell activation

| Parameter |  |  | Unit | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| SSB ARFCN |  |  |  | freq2 |  |
| Duplex mode | ConfigSCell 1 |  |  | FDD |  |
|  | ConfigSCell 2,3 |  |  | TDD |  |
| TDD configuration | ConfigSCell 1 |  |  | Not Applicable |  |
|  | ConfigSCell 2 |  |  | TDDConf.1.1 |  |
|  | ConfigSCell 3 |  |  | TDDConf.2.1 |  |
| BWchannel | ConfigSCell 1 |  | MHz | 10: NRB,c = 52 |  |
|  | ConfigSCell 2 |  |  | 10: NRB,c = 52 |  |
|  | ConfigSCell 3 |  |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration | ConfigSCell 1,2,3 |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | ConfigSCell 1,2,3 |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration | ConfigSCell 1,2,3 |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | ConfigSCell 1,2,3 |  |  | ULBWP.1.1 |  |
| DRX Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference | ConfigSCell 1 |  |  | SR.1.1 FDD |  |
| measurement channel | ConfigSCell 2 |  |  | SR.1.1 TDD |  |
|  | ConfigSCell 3 |  |  | SR.2.1 TDD |  |
| RMSI CORESET | ConfigSCell 1 |  |  | CR.1.1 FDD |  |
| Reference Channel | ConfigSCell 2 |  |  | CR.1.1 TDD |  |
|  | ConfigSCell 3 |  |  | CR.2.1 TDD |  |
| RMC CORESET | ConfigSCell 1 |  |  | CCR.1.1 FDD |  |
| Reference Channel | ConfigSCell 2 |  |  | CCR.1.1 TDD |  |
|  | ConfigSCell 3 |  |  | CCR.2.1 TDD |  |
| TRS configuration | ConfigSCell 1 |  |  | TRS.1.1 FDD |  |
|  | ConfigSCell 2 |  |  | TRS.1.1 TDD |  |
|  | ConfigSCell 3 |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC configuration |  |  |  | SMTC.1 |  |
| SSB configuration | ConfigSCell 1,2 |  |  | SSB.1 FR1 |  |
|  | ConfigSCell 3 |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH | ConfigSCell 1,2 |  | kHz | 15 |  |
| subcarrier spacing | ConfigSCell 3 |  |  | 30 |  |
| CSI-RS configuration for CSI reporting | ConfigSCell 1 |  |  | CSI-RS.1.1 FDD |  |
|  | ConfigSCell 2 |  |  | CSI-RS.1.1 TDD |  |
|  | ConfigSCell 3 |  |  | CSI-RS.2.1 TDD |  |
| PDSCH/PDCCH subcarrier spacing | ConfigSCell 1,2 |  | kHz | 15 |  |
|  | ConfigSCell 3 |  |  | 30 |  |
| reportConfigType | ConfigSCell 1-3 |  |  | periodic |  |
| reportQuantity | ConfigSCell 1-3 |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | ConfigSCell 1,2 |  | slot | 5 |  |
|  | ConfigSCell 3 |  |  | 10 |  |
| CSI reporting offset | ConfigSCell 1,2 |  | slot | 2 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104 |  |
| Note2 |  | ConfigSCell 1,2 | dBm/SCS | -104 |  |
|  |  | ConfigSCell 3 |  | -101 |  |
|  |  |  | dB | 17 |  |
|  |  |  | dB | 17 |  |
| SS-RSRPNote3 |  | ConfigSCell 1,2 | dBm/SCS | -87 |  |
|  |  | ConfigSCell 3 |  | -84 |  |
| SCH_RP Note 3 |  |  | dBm/15 kHz | -87 |  |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

## << Unskipped Sections Skipped >>

## << Start of Third change >>

## A.6.5	Signalling characteristics

### A.6.5.3	SCell Activation and Deactivation Delay

#### A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle

##### A.6.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations for NR PCell are shown in table A.6.5.3.1.1-1 below. Supported test configurations for NR SCell are shown in table A.6.5.3.1.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 and A.6.5.3.1.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

## << Unskipped Sections Skipped >>

## << Start of Fourth change >>

#### A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1

##### A.6.5.3.4.1	Test Purpose and Environment

The purpose of this test is to verify fulfillment of direct SCell activation delay and interruption requirements at SCell addition as defined in clause 8.3.4 and 8.2.2, respectively. The supported test configurations are shown in Table A.6.5.3.4.1-1.

The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as outlined in Table A.6.5.3.4.1-2. Cell-specific parameters are provided in Table A.6.5.3.4.1-3.

The test consists of two successive time periods with duration T1 and T2, respectively. There are two carriers, each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), and Cell 2 (SCell) is on RF channel 2 (SCC). Cell 1 and Cell 2 both operate according to one of the configurations in Table A.6.5.3.4.1-1.

Before the test starts the UE is connected to Cell 1 on RF channel 1. The UE is only monitoring RF channel 1 and is not aware of Cell 2 on RF channel 2.

The UE is continuously scheduled in PCell throughout the test.

At the beginning of T1 the UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE by which it configures the SCell (Cell 2) in activated state (sCellState is set to activated). The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

Time period T2 starts when the UE receives the RRC connection reconfiguration message at the UE antenna connector. The corresponding slot at which the message is received at the UE antenna connector is denoted n. The UE shall complete activation of the SCell no later than in slot n + NdirectNR slot length, as specified in clause 8.3.4. From slot n+ NdirectNR slot length and onwards the UE shall report valid CSI both for PCell and SCell.

The test equipment verifies the activation time by counting the slots between the RRC connection reconfiguration message is sent and until CSI report with non-zero CQI for both PCell and SCell is received.

The test equipment verifies that interruptions on other serving cells are within the requirements by counting ACK/NACKs transmitted in PCell.

Table A.6.5.3.4.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Inter-frequency neighbor cell (SCell to-be) |  | Cell 2 | Inter-frequency neighbor cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern |  | gp0 | Measurement gap is used during parts of time period T1 for detection of Cell 2. |
| CSI reporting periodicity | ms | 2 | CSI reporting periodicity for periodic reporting of CQI for PCell and, when added, SCell. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | Measurement cycle for SCell does not come into effect in direct activation at SCell addition. |
| Timing offset between Cell 1 and Cell 2 | s | MRTD | The value of maximum timing offset depends upon the carrier aggregation scenario. |
| T1 | s | 7 | During this time period the PCell shall be known and Cell 2 shall be detected as an inter-frequency neighbor cell. |
| T2 | s | 1 | During this time period Cell 2 shall be configured and directly activated as SCell. |
| A3-offset | dB | -15 |  |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.4.1-3: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration |  | Initial DL |  | DLBWP.0.1 |  |  | --- | DLBWP.0.1 |
|  |  | Initial UL |  | ULBWP.0.1 |  |  |  | --- |
|  |  | Dedicated DL |  | DLBWP.1.1 |  |  |  | DLBWP.1.1 |
|  |  | Dedicated UL |  | ULBWP.1.1 |  |  |  | --- |
| TCI state |  |  |  | TCI.State.0 |  |  | --- | TCI.State.0 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  | --- | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  | CSI-RS.2.1 TDD |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |  | --- | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  | --- | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  |  | --- | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  |  | --- |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  |  |  | OP.1 |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  | SMTC.1 |  |
| reportConfigType |  |  |  | periodic |  |  | N/A |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  | N/A |  |
| CSI reporting periodicity |  | Config 1,2 | slot | 5 |  |  | N/A |  |
|  |  | Config 3 |  | 10 |  |  | N/A |  |
| CSI reporting offset |  | Config 1,2 | slot | 3 |  |  | N/A |  |
|  |  | Config 3 |  | 5 |  |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/SCS | -104 |  |  | -104 |  |
|  | Config 3 |  |  | -101 |  |  | -101 |  |
| Ês/Iot |  |  | dB | 17 |  |  | 17 |  |
| Ês/Noc |  |  | dB | 17 |  |  | 17 |  |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |  |  | -87 |  |
|  | Config 3 |  |  | -84 |  |  | -84 |  |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |  |  | -59.0 |  |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |  |  | -52.9 |  |
| Propagation condition |  |  |  | AWGN |  |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

## << Unskipped Sections Skipped >>

## << Start of Fifth change >>

#### A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1

##### A.6.5.3.5.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD and TDD-TDD intra frequency handover with direct SCell activation requirements specified in subclause 8.3.5.

Supported test configurations for NR PCell are shown in table A.6.5.3.5.1-1. Supported test configurations for NR SCell are shown in table A.6.5.3.5.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently. Both handover with direct SCell activation requirements are tested by using the parameters in table A.6.5.3.5.1-2, A.6.5.3.5.1-3 and A.6.5.3.5.1-4.

The test scenario comprises of  two NR carriers and the 3 cells as given in tables A.6.5.3.5.1-1 and A.6.5.3.5.1-2. The test consists of three successive time periods, with time durations of T1, T2, and T3 respectively.

At the start of time duration T1, the UE is in connected mode with PCell and SCell1 (cell 2) is in activated state and UE is reporting CQI for both PCell and SCell1.

Time period T2 starts when UE receives a handover command to Cell 3 that also activates SCell1 (Cell2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

Table A.6.5.3.5.1-2: General test parameters Intra-frequency handover with direct SCell activation from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | PCell |  | Cell 1 |  |
|  | SCell |  | Cell 2 |  |
|  | Target cell |  | Cell 3 |  |
| Final condition | PCell |  | Cell 3 |  |
|  | SCell |  | Cell 2 |  |
|  | neighbour cell |  | Cell 1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 | UE is in connected mode with PCell and SCell1 (cell 2) is in activated state. UE receives a handover command |
| T2 |  | s | Ndirect | UE shall accomplish the activation of the SCell |
| T3 |  | s | 1 |  |
| A3-offset |  | dB | -15 |  |
| THARQ |  | slot | k | k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting |  | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| K |  | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

## << End of changes >>