3GPP TSG-RAN4 Meeting #107	R4-2310473

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1591 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhancements for unlicensed access to 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Skyworks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh-Core. |  | Date: |  |  | 2023-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR implements the agreed enhancements for NR-U based on WID RP-221813. The NR-U feature was standardized in Rel-16 whereupon two unlicensed bands were added, 5GHz and 6GHz. Unlike the 5GHz unlicensed band, the 6GHz band has several operational modes: standard power (SP), low power indoor (LPI), and very low power (VLP). With introducing enhancements for NR-U several requirements are introduced: - Introduction of PC3 requirements for NR-U single UL and MIMO operation - Added new network signalling flags to support several new regulatory requirements for SP, LPI and VLP. - Introduction of extended additionalSpectrumEmission to support more than eight network signalling flags for a single band. - Added 100MHz channel to several existing network signalling flags. Details on changes can be found in Summary of change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of PC3 to NR-U UE power class table (6.2F.1-1) Adding NR-U PC3 MPR tables for 1Tx and 2Tx Aligned NR-U contiguous UL-CA MPR tables with NR-U MPR tables Label the highest NS value "7" as "RESERVED" for all existing NS values. Introducing PI/2 BPSK A-MPR for NS_28, NS_29, NS_30, NS_31, NS_53, NS_54, NS_60 and NS_61. Notes are added to clarify applicability of requirements with respect to IE powerBoostPi2BPSK. A-MPR values are increase to match MPR for NS_29 (40MHz CBW, Partial), NS_60 (20MHz CBW, 256QAM, Full). NS_29: Added note to clarify that larger CBW than 80MHz are not applicable for this network signalling. NS_30: Added A-MPR table for PC3 single Tx. NS_53: Added 100MHz CBW to PC5 A-MPR table and added A-MPR table for PC3 single Tx. NS_54: Added A-MPR table for PC3 single Tx. NS_59: The Note “Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0.” is introduced. NS_60: Added A-MPR table for PC3 single Tx and dual Tx. Introduction of new NS flags: NS_63 (Japan LPI), NS_64 (VLP EU/CEPT, HK), NS_65 (UK, Morocco, Jordan, Costa Rica, Malaysia), NS_66 (Canada, Dominican Republic), NS_67 (Brazil), NS_68 (Australia, Kenya, New Zealand), NS_69 (Japan) Added table for the extension of additionalEmissionRequirements (Table 6.2F.3.1-1B) Added missing requirement (clause) to Table 6.2F.3.1-1 for several NS flags. Added clarification in clause 6.2F.2 that for wideband operation the sub-bands need to be contiguous transmitted as defined in clause 6.1F. Added ACLR of 27dB for PC3 Adding requirements for PC3 NR-U UL MIMO in clause 6.2F.1D.This content has been endorsed in RAN4#106bis-e (R4-2306559). Updated wording for Full and Partial MPR in Tables Table 6.2F.2-1, Table 6.2F.2-3, Table 6.2F.3.8-1 and Table 6.2F.3.8-2. Simplified Table 6.2F.2-2 to only contain the exception wideband sub-band configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancements for shared spectrum access will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2F, 6.2F.1, 6.2F.1D, 6.2F.2, 6.2F.2A.2, 6.2F.3, 6.2F.3.1, 6.2F.3.2, 6.2F.3.3, 6.2F.3.4, 6.2F.3.5, 6.2F.3.6, 6.2F.3.7, 6.2F.3.8, 6.2F.3.9, 6.2F.3.10, 6.2F.3.12, 6.2F.3.13, 6.2F.3.14, 6.2F.3.15, 6.2F.3.16, 6.2F.3.17, 6.2F.3.18 6.5F.2.4.1, 6.5F.3.3.5, 6.5F.3.3.8, 6.5F.3.3.9, Annex L |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same AMPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK  is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41, n90 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.4 | n12, n85 | 5, 10, 15 |  | Clause 6.2.3.3212 |
|  |  | n14 | 5,10 |  |  |
| NS_07 | 6.5.3.3.26 | n13 | 5,10 | Table 6.2.3.29-1 | Table 6.2.3.29-2 |
| NS_10 |  | n20, n82 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5,10 | Table 6.2.3.21-1 | Table 6.2.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.22-1 | Table 6.2.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10,15,20 | Table 6.2.3.23-1 | Table 6.2.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5,10,15,20 | Table 6.2.3.24-1 | Table 6.2.3.24-2 |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
|  |  |  | 25, 30 |  | Table 6.2.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.3.3.12 | n30 | 5, 10 |  | Clause 6.2.3.14 |
| NS_24 | 6.5.3.3.13 | n65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.3.15-1 | Clause 6.2.3.15 |
| NS_27 | 6.5.2.3.8 6.5.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.16-1 | Table 6.2.3.16-2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | Clause 6.2.3.3111 |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25, 30, 40 | Table 6.2.3.20-1 | Table 6.2.3.20-1 |
| NS_45 | 6.5.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.25 |
| NS_46 | 6.5.3.3.25 | n7 | 25, 30, 35, 40, 50 | Table 6.2.3.17-1 | Table 6.2.3.17-2 |
| NS_47 | 6.5.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.18-1 | Table 6.2.3.18-2 |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.16 | n39, n98 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.19 |
| NS_51 | 6.5.3.3.22 | n65 | 50 | Table 6.2.3.28-1 | Table 6.2.3.28-2 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.27 | n24, n99 | 5, 10 |  | Clause 6.2.3.30 |
| NS_57 | NOTE 10 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz,10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. NOTE 9: Void NOTE 10: This NS value is applicable for cells in the range 3650-3980 MHz for operations in Canada. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 11: Applicable only for power class 1 operation. NOTE 12: Applicable only for power class 1 operation on band n85. |  |  |  |  |  |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n3 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n5 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n7 | NS_01 | NS_46 |  |  |  |  |  | Reserved |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  | Reserved |
| n12 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  | Reserved |
| n14 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n18 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n20 | NS_01 | Void | NS_10 |  |  |  |  | Reserved |
| n24 | NS_01 | NS_56 |  |  |  |  |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  | Reserved |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  | Reserved |
| n30 | NS_01 | NS_21 |  |  |  |  |  | Reserved |
| n34 | NS_01 |  |  |  |  |  |  | Reserved |
| n38 | NS_01 | NS_44 |  |  |  |  |  | Reserved |
| n39 | NS_01 | NS_50 |  |  |  |  |  | Reserved |
| n40 | NS_01 |  |  |  |  |  |  | Reserved |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  | Reserved |
| n48 | NS_01 | NS_27 |  |  |  |  |  | Reserved |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  | Reserved |
| n51 | NS_01 | NS_40 |  |  |  |  |  | Reserved |
| n53 | NS_01 | NS_45 |  |  |  |  |  | Reserved |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  | Reserved |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n70 | NS_01 | NS_03 |  |  |  |  |  | Reserved |
| n71 | NS_01 | NS_35 |  |  |  |  |  | Reserved |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  | Reserved |
| n77 | NS_01 | NS_55 | NS_57 |  |  |  |  | Reserved |
| n78 | NS_01 |  |  |  |  |  |  | Reserved |
| n79 | NS_01 |  |  |  |  |  |  | Reserved |
| n80 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  | Reserved |
| n82 | NS_01 | Void | NS_10 |  |  |  |  | Reserved |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  | Reserved |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  | Reserved |
| n85 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |
| n89 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n91 | NS_01 |  |  |  |  |  |  | Reserved |
| n92 | NS_01 |  |  |  |  |  |  | Reserved |
| n93 | NS_01 |  |  |  |  |  |  | Reserved |
| n94 | NS_01 |  |  |  |  |  |  | Reserved |
| n95 | NS_01 |  |  |  |  |  |  | Reserved |
| n97 | NS_01 |  |  |  |  |  |  | Reserved |
| n98 | NS_01 | NS_50 |  |  |  |  |  | Reserved |
| n99 | NS_01 | NS_56 |  |  |  |  |  | Reserved |
| n1042 | NS_01 |  |  |  |  |  |  | Reserved |
| n105 | NS_01 |  |  |  |  |  |  | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

<<< Unchanges sections are skipped >>>

## 6.2F	Transmitter power for shared spectrum channel access

### 6.2F.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of shared spectrum channel access carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2F.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| n96 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| n102 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80, 100 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80, 100 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80, 100 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
|  | NS_66 | 20, 40, 60, 80, 100 | 5925 - 7125 | -8 |
|  | NS_67 | 20, 40, 60, 80, 100 | 5925 - 7125 | -5 |
| n102 | NS_58 | 20, 40, 60, 80, 100 | 5945 – 6425 | 10 |
|  | NS_64 | 20, 40, 60, 80, 100 | 5945 – 6425 | 1 |
|  | NS_65 | 20, 40, 60, 80, 100 | 5945 – 6425 | 11 |
|  | NS_68 | 20, 40, 60, 80, 100 | 5945 – 6425 | 1 |

<<< Unchanges sections are skipped >>>

### 6.2F.1D	UE maximum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2F.1D-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2F.1D-1 UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| n96 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| n102 |  |  |  |  | 23 | +2/-3 | 20 | +2/-3 |
| NOTE 1: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

For UE supporting uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2F.1D-1 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

<<< Unchanges parts are skipped >>>

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2F.1D-1 is specified in Table 6.2F.2-1 for power class 5, and in Table 6.2F.2-3 and Table 6.2F.2-4 for power class 3. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

For UE supporting uplink full power transmission (ULFPTx) for UL MIMO, the allowed MPR for the maximum output power in Table 6.2F.1D-1 is specified in Table 6.2F.2-1 for power class 5, and in Table 6.2F.2-3 and Table 6.2F.2-4 for power class 3, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2F.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

<<< Unchanges sections are skipped >>>

### 6.2F.2	UE maximum output power reduction

For UE maximum output power reduction, the general requirements of clause 6.2.2 do not apply but instead the UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for power class 5 according to Table 6.2F.2-1 and Table 6.2F.2-2.

For wideband operation only sub-bands which are contiguously transmitted are considered in the current version of the specification as defined in clause 6.1F.

Table 6.2F.2-1 Maximum power reduction (MPR) for shared spectrum access UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |  |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |  |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |  |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |  |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. |  |  |  |  |

Table 6.2F.2-2 Exception MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | None |
| 80 | 1100, 0011, 0100, 0010 |
| 100 | 00111, 11100, 00011, 11000 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Void. |  |

Table 6.2F.2-3 Maximum power reduction (MPR) for shared spectrum access UE power class 3

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.0 | ≤ 1.5 |  |
|  | QPSK | ≤ 1.0 | ≤ 2.0 |  |
|  | 16 QAM | ≤ 1.5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 2.0 | ≤ 3.0 |  |
|  | 256 QAM | ≤ 4.5 | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 2.0 | ≤ 3.5 | ≤ [4.5] |
|  | 16 QAM | ≤ 2.5 | ≤ 3.5 | ≤ [4.5] |
|  | 64 QAM | ≤ 4.0 | ≤ 4.5 |  |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. |  |  |  |  |

Table 6.2F.2-4 Maximum power reduction (MPR)

for shared spectrum access UE power class 3 with dual Tx

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |  |
| DFT-s-ODFM |  | Pi/2 BPSK4 |  | ≤ [2.0] | ≤ [3.0] |  |  |
|  |  | QPSK |  | ≤ [2.0] | ≤ [3.0] |  |  |
|  |  | 16 QAM |  | ≤ [2.5] | ≤ [3.0] |  |  |
|  |  | 64 QAM |  | ≤ [2.5] | ≤ [3.5] |  |  |
|  |  | 256 QAM |  | ≤ [4.5] | ≤ [4.5] |  |  |
| CP-OFDM |  | QPSK |  | ≤ [3.5] | ≤ [4.0] | ≤ [5.0] |  |
|  |  | 16 QAM |  | ≤ [3.5] | ≤ [4.0] | ≤ [5.0] |  |
|  |  | 64 QAM |  | ≤ [4.5] | ≤ [4.5] |  |  |
|  |  | 256 QAM |  | ≤ [6.5] | ≤ [6.5] |  |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. |  |  |  |  |

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2F.4 apply.

<<< Unchanges sections are skipped >>>

#### 6.2F.2A.2	UE maximum output power reduction for intra-band contiguous CA

For intra-band contiguous carrier aggregation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.1-1 with contiguous RB allocation is specified in Table 6.2F.2A.2-1 and Table 6.2F.2A.2-2 for UE power class 5 CA bandwidth classes B and C. For UE maximum output power reduction, the general requirements of clause 6.2.2 do not apply but instead the UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for power class 5 according to Table 6.2F.2A.2-1 and Table 6.2F.2A.2-2.

For wideband operation only sub-bands which are contiguously transmitted are considered in the current version of the specification as defined in clause 6.1F.

Table 6.2F.2A.2-1 Maximum power reduction (MPR) for power class 5 shared spectrum access intra-band contiguous CA for bandwidth class B and class C.

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [3.0] |  | ≤ [4.0] |
|  | QPSK | ≤ [3.0] |  | ≤ [4.0] |
|  | 16 QAM | ≤ [3.5] |  | ≤ [4.5] |
|  | 64 QAM | ≤ [4.5] |  | ≤ [5.0] |
|  | 256 QAM | ≤ [6.0] |  | ≤ [6.0] |
| CP-OFDM | QPSK | ≤ [4.0] |  | ≤ [5.5] |
|  | 16 QAM | ≤ [4.5] |  | ≤ [6.0] |
|  | 64 QAM | ≤ [6.0] |  | ≤ [6.5] |
|  | 256 QAM | ≤ [7.0] |  | ≤ [7.0] |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all contiguously transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2A.2-2.. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] excluding the wideband configurations of Table 6.2F.2A.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2A.2-2. |  |  |  |  |

Table 6.2F.2A.2-2 Exception MPR mapping for intra-band CA wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions [CC1-CC2] |
| --- | --- |
| 20+20 | 1-0, 0-1 |
| 20+40 | None |
| 20+60 | 0-010, 0-100, 0-011, 1-100 |
| 20+80 | 1-1100, 0-0111 |
| 40+20 | None |
| 40+40 | 00-10, 01-00, 00-11, 11-00 |
| 40+60 | 11-100, 00-111 |
| 40+80 | 11-1000, 00-0111, 01-1000, 00-0110, 00-1000, 00-0100 |
| 60+20 | 001-0, 010-0, 001-1, 110-0 |
| 60+40 | 111-00, 001-11 |
| 60+60 | 111-000, 000-111, 011-000, 000-110, 001-000, 000-100 |
| 60+80 | 111-1000, 000-1111, 011-1000, 000-1110, 111-0000, 000-0111, 011-0000, 000-0110 |
| 80+20 | 1110-0, 0011-1 |
| 80+40 | 1110-00, 0001-11, 0110-00, 0001-10, 0010-00, 0001-00 |
| 80+60 | 1111-000, 0001-111, 0111-000, 0001-110, 1110-000, 0000-111, 0110-000, 0000-110 |
| 80+80 | 1111-1000, 0001-1111, 0111-1000, 0001-1110, 1111-0000, 0000-1111, 0111-0000, 0000-1110, 0011-0000, 0000-1100, 0001-0000, 0000-1000 |

<<< Unchanges sections are skipped >>>

### 6.2F.3	UE additional maximum output power reduction

#### 6.2F.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2F.2.

Table 6.2F.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.1-1A.

Table 6.2F.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) |  | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  |  | N/A |
| NS_28 |  | n46 | 20, 40, 60, 80 |  |  | 6.2F.3.2 |
| NS_29 |  | n46 | 20, 40, 60, 80 |  |  | 6.2F.3.3 |
| NS_30 |  | n46 | 20, 40, 60, 80 |  |  | 6.2F.3.4 |
| NS_31 |  | n46 | 20, 40, 60, 80 |  |  | 6.2F.3.5 |
| NS_53 |  | n96 | 20, 40, 60, 80, 100 |  |  | 6.2F.3.6 |
| NS_54 |  | n96 | 20, 40, 60, 80 |  |  | 6.2F.3.7 |
| NS_58 |  | n102 | 20, 40, 60, 80 |  |  | 6.2F.3.8 |
| NS_59 |  | n96 | 20, 40, 60, 80, 100 |  |  | 6.2F.3.9 |
| NS_60 |  | n96 | 20, 40, 60, 80, 100 |  |  | 6.2F.3.10 |
| NS_61 |  | n96 | 20, 40, 60, 80 |  |  | 6.2F.3.11 |
| NS_63 | 6.5F.3.3.8 | n102 | 20, 40, 80 |  |  | 6.2F.3.12 |
| NS_64 | 6.5F.3.3.9 | n102 | 20, 40, 60, 80, 100 |  |  | 6.2F.3.13 |
| NS_65 |  | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.14 |
| NS_66 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.15 |
| NS_67 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2F.3.16 |
| NS_68 |  | n102 | 20, 40, 60, 80, 100 |  | 6.2F.3.17 |
| NS_69 | 6.5F.3.3.8 | n102 | 20, 40, 80 |  | 6.2F.3.18 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2F.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  | Reserved |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 | NS_61 |  | Reserved |
| n102 | NS_01 | NS_58 | NS_63 |  |  |  |  | Reserved |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2F.3.1-1B: Mapping of extended network signaling label

| NR band | Value of extendedAdditionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| n96 | NS_66 | NS_67 |  |  |  |  |  |  |
| n102 | NS_64 | NS_65 | NS_68 | NS_69 |  |  |  |  |  |
| NOTE: extendedAdditionalSpectrumEmission corresponds to an information element of the name [EXTENDED_ additionalSpectrumEmission] defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

#### 6.2F.3.2	A-MPR for NS_28

When “NS_28” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.2-1.

Table 6.2F.3.2-1: A-MPR for NS_28 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK4 | ≤ 4.0 | ≤ 6.0 | See Table 6.2F.2-1 |
|  | QPSK | ≤ 4.0 | ≤ 6.0 |  |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 4.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 5.5 | ≤ 6.5 |  |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 |  |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

#### 6.2F.3.3	A-MPR for NS_29

When “NS_29” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3-1.

Table 6.2F.3.3-1: A-MPR for NS_29 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz |  | 60 MHz, 80 MHz |  |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | See Table 6.2F.2-1 | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | QPSK |  | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 2.5 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: Larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

#### 6.2F.3.4	A-MPR for NS_30

When “NS_30” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.4-1 for power class 5 and Table 6.2F.3.4-2 for power class 3.

Table 6.2F.3.4-1: A-MPR for NS_30 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK5 | ≤ 9.0 | ≤ 15.0 | ≤ 2.5 | ≤ 5.0 | See Table 6.2F.2-1 |
|  | QPSK | ≤ 9.0 | ≤ 15.0 | ≤ 2.5 | ≤ 5.0 |  |
|  | 16 QAM | ≤ 9.0 | ≤ 15.5 | ≤ 3.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 9.0 | ≤ 15.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 9.0 | ≤ 16.0 | ≤ 5.5 | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 14.0 | ≤ 4.0 | ≤ 6.0 |  |
|  | 16 QAM | ≤ 9.5 | ≤ 14.5 | ≤ 4.0 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 9.5 | ≤ 15.0 | ≤ 5.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 9.5 | ≤ 15.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. NOTE 5: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

Table 6.2F.3.4-2: A-MPR for NS_30 power class 3 with single Tx

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK5 | ≤ 12.0 | ≤ 15.5 | ≤ 3.5 | ≤ 5.5 | See Table MPR table for PC3 |
|  | QPSK | ≤ 12.5 | ≤ 16.0 | ≤ 4.0 | ≤ 6.5 |  |
|  | 16 QAM | ≤ 12.5 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
| CP-OFDM | QPSK | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.5 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. NOTE 5: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

#### 6.2F.3.5	A-MPR for NS_31

When “NS_31” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.5-1.

Table 6.2F.3.5-1: A-MPR for NS_31 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table 6.2F.2-1 | ≤ 4.0 | ≤ 6.5 |
|  | QPSK |  | ≤ 4.0 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 64 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.5 |
| CP-OFDM | QPSK |  | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

#### 6.2F.3.6	A-MPR for NS_53

When “NS_53” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.6-1 for power class 5 and in Table 6.2F.3.6-2 for power class 3.

Table 6.2F.3.6-1: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ [1.5] | ≤ [2.5] |
|  | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ [1.5] | ≤ [2.5] |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ [1.5] | ≤ [2.5] |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [2.0] | ≤ [3.0] |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 | ≤ [3.5] | ≤ [4.5] |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [3.5] | ≤ [4.5] |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [4.0] | ≤ [4.5] |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ [5.5] | ≤ [5.5] |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2F.3.6-2: A-MPR for NS_53 power class 3 with single Tx

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 8.5 | ≤ 6.5 | ≤ 7.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2F.3.7	A-MPR for NS_54

When “NS_54” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.7-1 for power class 5 and Table 6.2F.3.7-2 for power class 3.

Table 6.2F.3.7-1: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table 6.2F.2-1 | ≤ 2.5 | ≤ 5.0 |
|  | QPSK |  | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

Table 6.2F.3.7-2: A-MPR for NS_54 power class 3 with single Tx

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table PC3 MPR | ≤ 2.0 | ≤ 4.5 |
|  | QPSK |  | ≤ 3.0 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 5.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.0 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3 and NOTE 5. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

#### 6.2F.3.8	A-MPR for NS_58

When “NS_58” is indicated in the cell, the A-MPR is specified in Table 6.2F.3.8-1 for power class 5. The Table 6.2F.3.8-2 is applicable for power class 3 with single Tx.

Table 6.2F.3.8-1: A-MPR for NS_58 power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-OFDM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 2.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

Table 6.2F.3.8-2: A-MPR for NS_58 power class 3 with single Tx

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 4.5 |
|  | QPSK | ≤ 2.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 3.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

#### 6.2F.3.9	A-MPR for NS_59

When "NS_59" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.9-1.

Table 6.2F.3.9-1: A-MPR for NS_59 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |
| --- | --- | --- | --- |
|  |  | 20 MHz |  |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

#### 6.2F.3.10	A-MPR for NS_60

When "NS_60" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.10-1 for power class 5. The Table 6.2F.3.10-2 is applicable for power class 3 with single Tx and Table 6.2F.3.10-2 is applicable for power class 3 with dual Tx.

Table 6.2F.3.10-1: A-MPR for NS_60 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: For larger channels than 80MHz the A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |  |  |  |  |  |  |

Table 6.2F.3.10-2: A-MPR for NS_60 power class 3 with single Tx

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2F.3.10-2: A-MPR for NS_60 power class 3 with dual Tx

| Pre-coding | Modulation |  |  | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 20 MHz |  |  | 40 MHz |  |  | 60 MHz |  |  |  | 80 MHz |  |  | 100 MHz |  |  |
|  |  |  |  | Full (dB) | Partial (dB) |  |  | Full (dB) | Partial (dB) |  | Full (dB) | Partial (dB) | Full (dB) |  | Partial (dB) | Full (dB) |  | Partial (dB) |  |
| DFT-s-OFDM | PI/2 BPSK2 |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 |  | ≤ 6.0 | ≤ 3.0 |  | ≤ 5.5 |  |
|  | QPSK |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 |  | ≤ 6.0 | ≤ 3.0 |  | ≤ 5.5 |  |
|  | 16 QAM |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 |  | ≤ 6.0 | ≤ 3.0 |  | ≤ 5.5 |  |
|  | 64 QAM |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 |  | ≤ 6.0 | ≤ 3.0 |  | ≤ 5.5 |  |
|  | 256 QAM |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 4.5 | ≤ 7.0 | ≤ 4.5 |  | ≤ 6.0 | ≤ 4.5 |  | ≤ 5.5 |  |
| CP-OFDM | QPSK |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 |  | ≤ 6.0 | ≤ 4.5 |  | ≤ 5.5 |  |
|  | 16 QAM |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 |  | ≤ 6.0 | ≤ 4.5 |  | ≤ 5.5 |  |
|  | 64 QAM |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 |  | ≤ 6.0 | ≤ 4.5 |  | ≤ 5.5 |  |
|  | 256 QAM |  |  | ≤ 9.5 | ≤ 11.5 |  |  | ≤ 6.0 | ≤ 9.0 |  | ≤ 5.0 | ≤ 7.0 | ≤ 5.5 |  | ≤ 6.5 | ≤ 5.5 |  | ≤ 6.0 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<< Unchanges sections are skipped >>>

#### 6.2F.3.12	A-MPR for NS_63

When "NS_63" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.12-1.

Table 6.2F.3.12-1: A-MPR for NS_63 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3, Note 4) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table PC5 MPR | ≤ 2.0 | ≤ 3.0 |
|  | QPSK |  | ≤ 2.5 | ≤ 3.5 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3 and NOTE 4. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to [6415 MHz], 40 MHz channels centered at the nearest NR-ARFCN corresponding to [6405 MHz] and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6385 MHz]. NOTE 5: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 6: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |

#### 6.2F.3.13	A-MPR for NS_64

When "NS_64" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.13-1.

Table 6.2F.3.13-1: A-MPR for NS_64 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 12 | ≤ 14 | ≤ 8 | ≤ 10 |
|  | QPSK | ≤ 12 | ≤ 14 | ≤ 8 | ≤ 10 |
|  | 16 QAM | ≤ 12 | ≤ 15 | ≤ 9 | ≤ 10 |
|  | 64 QAM | ≤ 12 | ≤ 15 | ≤ 11 | ≤ 11 |
|  | 256 QAM | ≤ 13 | ≤ 15 | ≤ 13 | ≤ 14 |
| CP-OFDM | QPSK | ≤ 13 | ≤ 15 | ≤ 10 | ≤ 10 |
|  | 16 QAM | ≤ 13 | ≤ 15 | ≤ 11 | ≤ 11 |
|  | 64 QAM | ≤ 13 | ≤ 15 | ≤ 13 | ≤ 13 |
|  | 256 QAM | ≤ 15 | ≤ 15 | ≤ 15 | ≤ 15 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |

#### 6.2F.3.14	A-MPR for NS_65

When "NS_65" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.14-1.

Table 6.2F.3.14-1: A-MPR for NS_65 power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 6.0 | ≤ 6.0 |
|  | QPSK | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all contiguously transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

#### 6.2F.3.15	A-MPR for NS_66

When "NS_66" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.15-1.

Table 6.2F.3.15-1: A-MPR for NS_66 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | QPSK | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 256 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
| CP-OFDM | QPSK | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 256 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2F.3.16	A-MPR for NS_67

When "NS_67" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.16-1.

Table 6.2F.3.16-1: A-MPR for NS_67 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2F.3.17	A-MPR for NS_68

When "NS_68" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.17-1.

Table 6.2F.3.17-1: A-MPR for NS_68 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | QPSK | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2F.3.18	A-MPR for NS_69

When "NS_69" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.18-1.

Table 6.2F.3.18-1: A-MPR for NS_69 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.5 | ≤ 6.5 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 6.5 | ≤ 8.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 6.0 | ≤ 6.0 | ≤ 8.0 | ≤ 7.0 | ≤ 8.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |

<<< Unchanges sections are skipped >>>

##### 6.5F.2.4.1	Shared spectrum channel access ACLR

The Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing.  The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

Instead of the general ACLR requirement in clause 6.5.2.4, if the measured adjacent channel power is greater than –47 dBm then the ACLR shall be higher than the value specified in Table 6.5F.2.4.1-1.

Table 6.5F.2.4.1-1: Shared spectrum channel access ACLR requirement

|  | Power class 5 | Power class 3 |
| --- | --- | --- |
| ACLR | 27 dB | 27 dB |

<<< Unchanges sections are skipped >>>

##### 6.5F.3.3.5	Requirements for network signalling value "NS_53" or "NS_54" or "NS_60" or “NS_66” or “NS_67”

When "NS_53" or "NS_54" or "NS_60" or “NS_66” or “NS_67” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

<<< Unchanges sections are skipped >>>

##### 6.5F.3.3.8	Requirements for network signalling value “NS_63” or “NS_69”

When "NS_63" or “NS_69” is indicated in the cell, the power of any UE emission for channels assigned within 5945-6425 MHz shall not exceed the levels specified in Table 6.5F.3.3.8-1. These requirements also apply for frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.8-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| 6425 ≤ f ≤ X | -13 | 1 MHz |
| f ≥ X | -19 | 1 MHz |
| Note 1: In case of NS_63 the parameter X is defined as 6435.9MHz for 20MHz channel, 6440.1MHz for 40MHz channel, and 6440.4MHz for 80MHz. Note 2: In case of NS_69 the parameter X is defined as 6425.5MHz for 20MHz channel, 6425.4MHz for 40MHz channel, and 6425.2MHz for 80MHz. |  |  |

ACLR is specified for the first adjacent channel (ACLR1) which centre frequency is ±CBW from assigned channel centre and for the 2nd adjacent channel (ACLR2) which centre frequency is ±2*CBW from assigned channel centre. The assigned channel power and ACLR1/ACLR2 are measured with rectangular filters with measurement bandwidth of CBW.

Instead of the general ACLR requirement in clause 6.5.2.4 and 6.5F.2.4.1, if the measured adjacent channel power is greater than –47 dBm then the ACLR shall be higher than the value specified in Table 6.5F.3.3.10-2.

Table 6.5F.3.3.10-2: Shared spectrum channel access ACLR requirement

|  | ACLR1 | ACLR2 |
| --- | --- | --- |
| Spectrum emission limit (dB) | 25 | 40 |

##### 6.5F.3.3.9	Requirements for network signalling value “NS_64”

When "NS_64" is indicated in the cell, the power of any UE emission for channels assigned within 5945-6425 MHz shall not exceed the levels specified in Table 6.5F.3.3.9-1. These requirements also apply for frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.9-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 694 | -54 | 100 kHz |
| f ≤ 5935 | -45 | 1 MHz |

<<< Unchanges sections are skipped >>>

Annex L (normative):
ModifiedMPR-Behavior

# L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n30 | 0 (leftmost bit) | Requirements for network signalling value NS_21 as defined in Clause 6.5.2.3.y of 38.101-1 v17.6.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v17.6.0. | This bit shall be set to 1 by a UE supporting the Rel-17 version of the specification. If the bit is not set, then requirements for NS_21 as defined in Clause 6.5.2.3.3 of 38.101-1 v16.11.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v16.11.0 apply. |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| n77 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n78 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n79 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |
| n102 | 0 (leftmost bit) | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |

<<< End of changed sections >>>