3GPP TSG-RAN4 Meeting #107	R4-2308690

# Incheon, KR , 22th – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3284 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction to interruption requirements for inter-band CA R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R17, due to reomving the endorsed interruption requirements for FR2 inter-band CA with CBM, there cause some typos for interruption requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To correctly describe the case “the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair” for interruption requirements at SCell addition/release for EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption requirements at SCell addition/release for EN-DC are not properly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell in MCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to max{Y1 slot + TSMTC_duration, 5ms} if the active serving cells are in the same band as any of the E-UTRA SCells being added or released, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

Where X1 and Y1 are specified in Table 8.2.1.2.3-1.

When one SCell in SCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slot,  if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair

or

-	of up to Y1 slot + TSMTC_duration if the active serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

Where X1 and Y1 are specified in Table 8.2.1.2.3-2.

Table 8.2.1.2.3-1: Interruption length X1 and Y1 at E-UTRA SCell addition/Release

|  | NR Slot length | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | N/A | - N/A |

Table 8.2.1.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

<End of Change 1>