3GPP TSG RAN Meeting #102							RP-233808

Edinburgh, Scotland, December 11-15, 2023

Agenda Item:		9.3.1.1

Source:			Ericsson

Title:			Summary for WI on Enhancement of NR Dynamic spectrum sharing (DSS)

WI code(s):			NR_DSS_enh

leading WG:		RAN1

Release:			Rel-18

### 1	Introduction

Dynamic spectrum sharing (DSS) provides a very useful migration path from LTE to NR by allowing LTE and NR to share the same carrier. DSS was included in Rel-15 and additional features related to it were specified in Rel-16 and Rel17. LTE UEs are likely to be around for a long time and hence it is important to continue to evolve DSS, especially there is room for performance improvements in scenarios where NR traffic starts to dominate with very low LTE traffic.

NR PDCCH control signalling would be a bottleneck of DSS with increasing NR traffic. To maximize the resource utilization and increase the PDCCH capacity for DSS, UE reception of NR PDCCH candidates overlapping with LTE CRS is specified in this Rel18 WI.

Transmission of LTE CRS from neighbouring LTE cells can cause interference to NR cells. In order to mitigate this interference, it would be useful to have multiple LTE CRS rate matching patterns. However, so far two overlapping LTE rate matching patterns are only supported by UEs together with support and configuration of multi-TRP transmissions. UE support for two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP is specified in this Rel18 WI.

### 2	Description

The work item introduces the following:

- UE reception of NR PDCCH candidates that overlap with LTE CRS REs

- UE support, and configuration with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP

Changes triggered by the work item are captured into TS 38- series specifications [3]-[8].

### 3	References

[1]	RP-232957 - “Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#102.

[2]	RP-232956 – Status report for WI: Enhancement of NR Dynamic spectrum sharing (DSS), Ericsson, RAN#102.

[3]	R1-2308727 – “Introduction of dynamic spectrum sharing enhancements”, Ericsson, CR0112 to 38.211, RAN1#114.

[4]	R1-2308694 – “Introduction of dynamic spectrum sharing (DSS) enhancements”, Samsung, CR0500 to 38.213, RAN1#114.

[5]	R1-2308724 – “Introduction of dynamic spectrum sharing enhancements”, Nokia, CR0443 to 38.214, RAN1#114.

[6]	R2-2132995 – “Introduction of R18 DSS”, Ericsson, ZTE Corporation, CR4360 to 38.331, RAN2#124

[7]	R2-2313676 - “Introduction of Rel-18 UE capabilities”, Intel Corporation, CR4510 to 38.331, RAN2#124

[8]	R2-2313677 – “Introduction of Rel-18 UE capabilities”, Intel Corporation, CR1015 to 38.306, RAN2#124