3GPP TSG RAN Meeting #103	 RP-240055

Maastricht, NL, March18-21, 20024

Agenda item:	9.2.3	Study on solutions for Ambient IoT in NR [New RAN1 SI: FS_Ambient_IoT_solutions]

Source:	Nokia, Nokia Shanghai Bell

Title:	Rel-19 study on Ambient IoT

Document for:	Discussion

# 1	Introduction

In RAN#103 the Study on solutions for Ambient IoT (Internet of Things) in NR [1] was approved.

In this paper, we propose a small addition to the scope of the SID [1].

# 2	Discussion

## 2.1	Introduction

The scope of Ambient IoT

General Scope Motivation:

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X  is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

- For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

- Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum.

## 2.2	Motivation & proposal

One of the key use cases for Ambient IoT is deployment in enterprise/private networks. Most enterprise/private networks are using TDD frequency bands. The current scope of the Ambient IoT scope is “FR1 licensed spectrum in FDD”. Thus, the proposal is to add FR1 licensed spectrum in TDD.

Proposal 1: Include FR1 licensed spectrum in TDD in the Ambient IoT study item. Change scope C to:

C: FR1 licensed spectrum in FDD & TDD.

# 3	Conclusions

Proposal 1: Include FR1 licensed spectrum in TDD in the Ambient IoT study item. Change scope C to:

C: FR1 licensed spectrum in FDD & TDD.

# References

- RP-234058: New SID: Study on solutions for Ambient IoT (Internet of Things) in NR