3GPP TSG RAN meeting #103										RP-240070

Maastricht, Netherlands, March 18-21, 2024

Status Report to TSG

Agenda item:			9.4.1.2

| WI / SI Name | Multi-carrier enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MC_enh |  |  |  |  |
| Unique ID | 940094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222251 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 99% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Hiroki Harada |
|  | Company | NTT DOCOMO, INC. |
|  | Email | hiroki.harada.sv@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#116

R1-2401589	Feature lead summary#1 on multi-cell PUSCH/PDSCH scheduling with a single DCI	Moderator (Lenovo)

Agreement

Adopt following TP for TS38.213.

- Change reason: Unicast DCI formats do not include DCI format 1_3 and 0_3.

- Change summary: Add DCI format 1_3 and 0_3 in unicast DCI format list.

- Consequence if not approved: Incomplete unicast DCI format list.

# 9 UE procedure for reporting control information <text omitted> In the following, DCI formats with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI are also referred to as unicast DCI formats and DCI formats with CRC scrambled by multicast-MCCH-RNTI, G-RNTI for multicast or G-CS-RNTI are also referred to as multicast DCI formats. Corresponding unicast DCI formats are DCI formats 0_0/0_1/0_2/0_3/1_0/1_1/1_2/1_3 and multicast DCI formats are DCI formats 4_0/4_1/4_2 [4, TS 38.212]. PDSCH receptions scheduled by unicast or multicast DCI formats are referred as unicast or multicast PDSCH receptions. HARQ-ACK information associated with unicast or multicast DCI formats for PDCCH receptions in RRC_CONNECTED state are also respectively referred as unicast or multicast HARQ-ACK information. <text omitted>

Agreement

Adopt the following TP for sub-clause 9.1.2.1 in TS38.213.

9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP on the primary cell or, if the PUCCH transmission is indicated by a DCI format to be on the PUCCH-sSCell as described in clause 9A, on a set of slot timing values K1 associated with the active UL BWP on the PUCCH-sSCell - If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1/ or DCI format 1_2/1_3 for serving cell c, or the active DL BWP for serving cell c is dormant BWP, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for SCS configuration of PUCCH transmission μ≤3, {7, 8, 12, 16, 20, 24, 28, 32} for μ=5, and {13, 16, 24, 32, 40, 48, 56, 64} for μ=6 - If the UE is configured to monitor PDCCH for DCI format 1_1/1_3 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 - If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1/1_3 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 - If the UE is configured to monitor PDCCH for DCI format 1_1/1_3 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-DCI-1-2-r17 - If an inapplicable value in dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 is provided, the value is excluded from K1

Agreement

A UE does not expect a DCI format 0_3/1_3 schedules an SCell with valid FDRA value and indicates the SCell to switch to dormant BWP.

Conclusion

For a cell scheduled by DCI format 0_3 with valid FDRA value, UE does not expect that OLPC/CAPC/TPMI/SRI in the DCI format indicates a code point that does not correspond to a configuration for the cell.

- No spec impact

Conclusion

FDRA validity for a cell is determined based on the indicated BWP of the cell.

- No spec impact

Agreement

Adopt the following TP to 38.212 for DMRS sequence initialization in DCI format 0_3:

7.3.1.1.4 Format 0_3 <omitted text> DMRS sequence initialization –1 bit if transform precoder is disabled at least for one cell configured by higher layer parameter ScheduledCell-ListDCI-0-3 in the scheduled cell set is configured with disabled transform precoder; otherwise, 0 bit. This field is applied to all the scheduled cells with transform precoder disabled and indicated by Scheduled cells indicator field or Frequency domain resource assignment field independently. <omitted text>

Agreement

TP1 in section 8 of R1-2401589 is agreed for TS38.214.

R1-2401510	Summary of discussion on Multi-carrier UL Tx switching scheme	Moderator (NTT DOCOMO, INC.)

Agreement

Agree on following TP

--------------------------------------- TP of TS 38.214 start----------------------------------------- ### 6.1.6 Uplink switching < Unchanged parts are omitted > If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is - determined based on the switching gap defined for a single Tx switching in [8, TS 38.101-1] when the Tx switching involves more than two bands, and there are at least two UL transmissions after switching on two switch-to bands that trigger the uplink switching, which are at least partially overlapped in time domain, - the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], otherwise. < Unchanged parts are omitted > ----------------------------------------------- TP end------------------------------------------------

Conclusion

Following scheduling restriction is covered by section 6.1.6 in TS38.214.

- For UL Tx switching, if an UL transmission with starting symbol at T0 is scheduled to a UE, the UE is not expected to be scheduled with another UL transmission overlapping with the first UL transmission in time and resulting in interruption on the first UL transmission by a DCI arriving later than T0-Toffset.

R1-2401590	Feature lead summary#2 on multi-cell PUSCH/PDSCH scheduling with a single DCI	Moderator (Lenovo)

Agreement

Adopt the following TP covering multi-cell scheduling in TS38.300.

10.X	Multi-cell scheduling by a single DCI

Multi-cell scheduling by a single DCI allows the PDCCH of a serving cell to schedule PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI but with the following restrictions:

- When a serving cell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on a cell set, the PUSCH/PDSCH on serving cells in the cell set is always scheduled by a PDCCH on the serving cell;

- When PCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, that PCell’s PDSCH and PUSCH cannot be scheduled by a PDCCH on an SCell;

- When an SCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, PCell is not included in the cell set;

- The scheduling PDCCH and the scheduled PDSCH(s)/PUSCH(s) can use the same or different numerologies;

- The co-scheduled PDSCH(s) with a PDCCH use the same numerology.

- The co-scheduled PUSCH(s) with a PDCCH use the same numerology.

Send an LS to RAN2 to convey the above TP. Final LS is in R1-2401716.

Agreement

TP2 in Section 8 for TS38.213 is agreed in principle. TS38.213 editor to provide final TP.

R1-2401639	Summary#2 of discussion on Multi-carrier UL Tx switching scheme	Moderator (NTT DOCOMO, INC.)

Agreement

RAN1 replies to RAN2 LS in R1-2400007 as below.

- RAN1 confirms that the first RAN2 agreement in the LS R1-2400007/R2-2313959 has no issue from RAN1 perspective, except for a case where Rel-18 UL Tx switching is configured with band combination {A, B} to a UE reporting support of UL Tx switching for band combination {A, B, C} /{A, B, C, D} and no UL-MIMO on band A nor band B. For the case, RAN1 continues to discuss it.

Final LS is in R1-2401776.

R1-2401591	Feature lead summary#3 on multi-cell PUSCH/PDSCH scheduling with a single DCI	Moderator (Lenovo)

Agreement

- When a PDCCH MO that provides a DCI format 1_3 is before active UL BWP change on the PUCCH cell, and the PUCCH indicated by the DCI format 1_3 is to be transmitted after the active UL BWP change on the PUCCH cell, the corresponding HARQ-ACK information for the DCI format 1_3 is skipped.

- FFS: When a PDCCH MO that provides a DCI format 1_3 is before an active DL BWP change on a cell of co-scheduled cells by the DCI format 1_3, and the DCI format 1_3 does not trigger the active DL BWP change for the cell, and the PUCCH indicated by the DCI format 1_3 is to be transmitted after the active DL BWP change on the cell,

  - For type 2 codebook for generating the second sub-codebook, the corresponding HARQ-ACK information for that cell with BWP switching is generated with NACK bit

  - For type 1 codebook and for type 2 codebook for generating the first sub-codebook, follow the legacy behaviour (the corresponding HARQ-ACK information for that cell with BWP switching is skipped)

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#125

Agreements

1	To refine the RAN4 agreed UE capability as below:

For a given BC supporting UL Tx switching across up to 4 bands:

-	When the optional capability is not reported, it means all the fallback BCs are supported by the UE as legacy with the same switching capabilities reported in the parent BC including switching period as legacy.

-	When the optional capability is reported, it means all the fallback BCs are supported by the UE with the largest switching period value, i.e. 210us.

-	No matter the optional capability is reported or not, the UE can advertise fallback band combinations with different/same switching period by separate BandCombination entries as legacy in case of different fallback.

2	To refine the RAN4 agreed RRC configuration as below: For each band pair, a RRC parameter is introduced to configure switching period value between value 35 us and 140 us. When the RRC parameter is absent, 210us is applied.

Agreements

- Rely on switchingAdditionalPeriodDualUL-r18 to report min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

- Send an LS to RAN4 to explain the full RAN2 solution.  Pending RAN4 response RAN2 will revisit agreement if needed

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#110

RF core requirements

Following LS, CRs, and WF are approved/agreed/endorsed:

- R4-2403657	 [Draft] Reply LS on UL Tx switching

- R4-2400851 CR for 38.101-1: Correction on time mask for Rel-18 Tx switching

- R4-2400937	 CR for 38.101-1: Capability update for tx switching across three or four uplink bands

- R4-2401844	 Clarification of three-band switching gap lengths for CA and SUL

- R4-2403865 WF on MC UE RF requirements and feature list

Agreements captured in R4-2403865

Issue 1-1-1: R4-2400022 LS on UL Tx switching

Online Agreement:

- Extend the 3/4 band requirements to also cover 2 band fallback cases in Rel-18.

Issue 1-2-2: UE feature list

Online Agreement:

- Do not distinguish one TAG and two TAG

- Do not distinguish 3 band and 4 band cases

- Align the wording for 38-4 and 38-5 based on RAN4 agreements

Agreement:

The following feature groups are agreed to be captured in LS that to be sent to RAN2.

#Editor’s note: Feature groups are omitted here. Please refer to R4-2403865

RRM performance requirements

Agreement:

  - No need of additional DL interruption test cases for the new RRC signaling of the fallback band combination.

2.4.2	Remaining Open issues

Solving remaining issue if any on RRM test cases related to core requirements on UL Tx switching schemes across up to 3 or 4 bands

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#116

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2400040 | Remaining issues on Multi-Carrier Enhancements for NR | Spreadtrum Communications |
| R1-2400136 | Maintenance of Rel-18 Multicarrier Enhancements | Huawei, HiSilicon |
| R1-2400198 | Remaining issues on Rel-18 multi-carrier enhancements | Lenovo |
| R1-2400223 | Maintenance on Multi-Carrier Enhancements for NR | vivo |
| R1-2400293 | Maintenance on Multi-Carrier Enhancements for NR | ZTE |
| R1-2400311 | Remaining issues on Multi-carrier enhancements for NR | CMCC |
| R1-2400413 | Maintenance on Multi-Carrier Enhancements for NR | CATT |
| R1-2400542 | Remaining issues on Multi-Carrier Enhancements for NR | xiaomi |
| R1-2400596 | Remaining issues in multi-carrier enhancement for NR | OPPO |
| R1-2400654 | On Rel-18 MC-DCI scheduling | Nokia, Nokia Shanghai Bell |
| R1-2400681 | Maintenance on Multi-Carrier Enhancements for NR | Langbo |
| R1-2400712 | Remaining issues on multi-cell PUSCH/PDSCH scheduling with a single DCI | Samsung |
| R1-2400764 | Remaining issues on SCell dormancy indication by DCI format 0_3/1_3 | Fujitsu |
| R1-2400993 | On maintenance issues for Rel-18 MC enhancements | Apple |
| R1-2401095 | Maintenance on multi-carrier enhancements | NTT DOCOMO, INC. |
| R1-2401140 | Maintenance for Rel-18 multi-carrier enhancements | Ericsson |
| R1-2401289 | On maintenance for multi-carrier enhancement | MediaTek Inc. |
| R1-2401323 | Remaining issues on multi-cell scheduling with single DCI | LG Electronics |
| R1-2401510 | Summary of discussion on Multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2401589 | Feature lead summary#1 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2401590 | Feature lead summary#2 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2401591 | Feature lead summary#3 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2401639 | Summary#2 of discussion on Multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2401715 | Draft LS on TS38.300 TP for Multi-cell scheduling in Rel-18 | NTT DOCOMO, INC. |
| R1-2401716 | LS on TS38.300 TP for Multi-cell scheduling in Rel-18 | RAN1, NTT DOCOMO |
| R1-2401776 | Reply LS on UL Tx switching | RAN1, NTT DOCOMO |
| R1-2401802 | Summary#3 of discussion on Multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2401848 | Feature lead summary#4 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2400044 | Remaining issues on UE features for multi-carrier enhancement | Spreadtrum Communications |
| R1-2400084 | Remaining issues of MC-enhancements UE features | Nokia, Nokia Shanghai Bell |
| R1-2400137 | Discussion on UE features for MC enhancements | Huawei, HiSilicon |
| R1-2400229 | Discussion on Rel-18 Multi-carrier enhancements UE features | vivo |
| R1-2400298 | Discussion on UE feature for MC enhancements | ZTE |
| R1-2400451 | Discussion on UE features for MC enhancements | CATT |
| R1-2400537 | Discussion on UE features for MC enhancements | xiaomi |
| R1-2400597 | Discussion on UE features for multi-carrier enhancement | OPPO |
| R1-2400718 | UE features for multi-carrier enhancements | Samsung |
| R1-2400998 | On remaining issues for UE features for Rel-18 MC enhancements | Apple |
| R1-2401104 | Discussion on UE features for multi-carrier enhancements | NTT DOCOMO, INC. |
| R1-2401141 | UE features for Rel-18 multi-carrier enhancements | Ericsson |
| R1-2401290 | On UE feature discussion for Rel-18 multi-carrier enhancement | MediaTek Inc. |
| R1-2401324 | Discussion on UE features for MC enhancements | LG Electronics |
| R1-2401427 | UE features for MC enhancements | Qualcomm Incorporated |
| R1-2401512 | Summary of discussion on UE features for MC enhancements | Moderator (NTT DOCOMO, INC.) |
| R1-2401641 | Summary#2 of discussion on UE features for MC enhancements | Moderator (NTT DOCOMO, INC.) |
| R1-2401713 | Session Notes of AI 8.12.4 | Ad-Hoc Chair (NTT DOCOMO, INC.) |

RAN2#125

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2400064 | LS on Rel-18 Tx switching enhancement (R4-2321986; contact: Huawei) | RAN4 |
| R2-2400236 | Left Issues on Tx-Switching | OPPO |
| R2-2400738 | RRC RIL issue list for Rel-18 Multi-carrier enhancements | Huawei, HiSilicon |
| R2-2400739 | Rapp RRC CR for Rel-18 Multi-carrier enhancements | Huawei, HiSilicon, NTT DOCOMO INC., Intel, Qualcomm |
| R2-2400741 | On ambiguity issue of switching period (LS R4-2321986) | Huawei, HiSilicon, OPPO, Apple, Ericsson, NTT DOCOMO INC. |
| R2-2401178 | [N041] Multicarrier DCI Scheduling | Nokia, Nokia Shanghai Bell |
| R2-2401216 | Discussion on UL Tx switching for parallel switching on four bands | MediaTek Inc. |
| R2-2401225 | [DRAFT] Reply LS on Rel-18 UL Tx switching for parallel switching on four bands | MediaTek Inc. |
| R2-2401331 | Discussion on change of MAC spec for Multi-carrier enhancements | NTT DOCOMO, INC. |
| R2-2401334 | Introduction of Multi-carrier enhancements | NTT DOCOMO INC. |
| R2-2401441 | Discussion on remaining issues of Rel-18 UL Tx switching | ZTE Corporation |
| R2-2401854 | Introduction of Multi-carrier enhancements | NTT DOCOMO INC. |
| R2-2401969 | Reply LS on Rel-18 UL Tx switching for parallel switching on four bands | RAN2 |
| R2-2401970 | Introduction of Multi-carrier enhancements | NTT DOCOMO INC. |

RAN4#110

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2400756 | Topic summary for [110][220] NR_MC_enh | Moderator (Huawei) |
| R4-2400851 | (NR_MC_enh-Core) CR for 38.101-1: Correction on time mask for Rel-18 Tx switching | Huawei, HiSilicon |
| R4-2400937 | CR for 38.101-1: Capability update for tx switching across three or four uplink bands | China Telecom |
| R4-2401088 | Topic summary for [110][129] NR_MC_enh_UERF | Moderator(China Telecom) |
| R4-2401106 | Views on RAN2 LS and UE feature list for Rel-18 Tx switching | NTT DOCOMO INC. |
| R4-2401235 | Remaining issues for DL interruption test case due to Tx switching | China Telecom |
| R4-2401277 | Reply LS on UL Tx switching | ZTE Corporation |
| R4-2401347 | Modification on DL interruption for Tx switching across 3/4 bands | Huawei, HiSilicon |
| R4-2401348 | Test case for DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA for single TAG | Huawei, HiSilicon |
| R4-2401523 | Discussion of LS on UL Tx switching | vivo |
| R4-2401524 | [Draft] Reply LS on UL Tx switching | vivo |
| R4-2401525 | Corrections of UL Tx switching period applicability for 2 band case for Rel-18 | vivo |
| R4-2401542 | Discussions and updated Schedule for Rel-18 TRP TRS requirements work | vivo |
| R4-2401844 | (NR_MC_enh-Core) Clarification of three-band switching gap lengths for CA and SUL | Ericsson |
| R4-2401986 | draftCR on DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA for two TAGs | ZTE Corporation |
| R4-2401987 | CR on Implementation of two-band Tx switching with dual TAG | ZTE Corporation |
| R4-2402307 | Draft CR for 38.133 - Addition of new Test case for DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA for 2 TAGs | Nokia, Nokia Shanghai Bell |
| R4-2402312 | (NR_MC_enh-Core)Discussion on the UE feature list for MC_enh | MediaTek Inc. |
| R4-2402904 | Draft CR on TC for DL interruption at UL Tx switching across three bands in TDD-TDD CA for single TAG | MediaTek inc. |
| R4-2403321 | Test case for DL interruptions at Tx switching across four uplink bands in FDD-TDD CA in SA for single TAG | Huawei, HiSilicon |
| R4-2403322 | draftCR on DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA for two TAGs | ZTE Corporation |
| R4-2403323 | Draft CR for 38.133 - Addition of new Test case for DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA for 2 TAGs | Nokia, Nokia Shanghai Bell |
| R4-2403324 | Draft CR on TC for DL interruption at UL Tx switching across three bands in TDD-TDD CA for single TAG | MediaTek inc. |
| R4-2403569 | Big CR on RRM performance requirements for NR Multi-carrier enhancements | Huawei |
| R4-2403657 | Reply LS on UL Tx switching | vivo |
| R4-2403658 | WF on MC UE RF requirements and feature list | China Telecom |
| R4-2403865 | WF on MC UE RF requirements and feature list | China Telecom |
| R4-2403900 | Big CR for 38.101-1 on MC | China Telecom |

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template