3GPP TSG RAN meeting #103									          RP-240103

Maastricht, Netherlands, March 18 - 21, 2024

## Status Report to TSG

Agenda item:			9.4.3.1

| WI / SI Name | Mobile IAB (Integrated Access and Backhaul) for NR |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes | Testing part: No |
| Acronym | NR_mobile_IAB |  |  |  |
| Unique ID | 941009 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-233320 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 60% | Testing part: N/A |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

The core part of this work item has concluded.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

The core part of this work item has concluded.

## 2.4	RAN4

#### 2.4.1	Agreements

The core part of this work item has concluded.

R4#110

RF:

WF can be found in R4-2402962 and official summary in R4-2402650.

CRs on core requirement 38.174

- R4-2402502: Draft CR to 38.174 for mIAB RF requirement maintenance: Adding the reference to 9.3.2 and 9.3.3 in Section 9.3.1B output power for mIAB-MT.

WID update for mIAB performance work:

- WID update is needed to capture impacted IAB TSs (i.e., 38.176-1, 38-176-2, and 38.175).

General RF conformance requirements:

- Follow IAB RF specifications on how to handle conformance requirements in TS 38.176

mIAB-MT output power dynamics test requirements:

- For mIAB-MT type 1-H, type 1-O and type 2-O, proposal to adopt the same test tolerance for Local area IAB-MT given in TS 38.176-1 and TS 38.176-2.

Relative power tolerance:

- RAN4 to not specify test requirements for Relative power tolerance for mIAB-MT, following legacy IAB-MT in TS 38.176-1 and TS 38.176-2.

Endorsed CRs for performance part:

- R4-2401993: BigCR for 38.176-1 introduction of mobile IAB RF conformance requirements

- R4-2401994: BigCR to TS 38.176-2 on RF conformance requirements for NR Mobile IAB

- R4-2402250: Draft CR to TS 38.175 with mobile IAB introduction to EMC specification

Demod:

WF can be found in R4-2402871 and official summary in R4-2402666.

mIAB-MT demodulation requirements

Issue 1-1-1: Test case scope

Agreement:

Select PBCH demodulation requirements from 38.101-4 for mIAB-MT.

Issue 1-1-2: Frequency range

Agreement:

Both FR1 and FR2-1will be considered for mIAB-MT demodulation requirements.

Issue 1-1-3: Duplex mode for FR1 requirements

Agreement:

Only TDD is considered for FR1  mIAB-MT requirements.

Issue 1-1-4: How to introduce requirements for mIAB-MT demodulation?

Agreement:

Reuse all IAB-MT demodulation requirements except the new selected cases from 38.101-4 with similar configurations besides channel model.

Issue 1-2-2: FR1 PDSCH demodulation requirements selection from 38.101-4: Rank.

Agreement:

Only rank 1 is considered.

Issue 1-2-3: FR1 PDSCH demodulation requirements selection from 38.101-4: Antenna configuration.

Agreement:

2x4 for conducted and 2x2 for radiated requirements.

Issue 1-2-8: FR2-1 PDSCH demodulation requirements selection from 38.101-4: Antenna configuration

Agreement:

2x2 for radiated requirements.

Issue 1-3-1: FR1 PDCCH demodulation requirements selection from 38.101-4: Channel model

Agreement:

TDLC300-100.

Issue 1-4-1: Requirement selection of CSI reporting from 38.101-4

Agreement:

Only select CQI report requirement with fading channel for mIAB-MT.

mIAB-DU demodulation requirements

Issue 2-1-1: How to reuse legacy IAB-DU demodulation requirements?

Agreement:

Reuse all legacy IAB-DU requirements for FR1 and FR2-1 mIAB-DU.

mIAB-MT demodulation requirements

Issue 1-2-1: FR1 PDSCH demodulation requirements selection from 38.101-4: Modulation order and Channel model.

Way forward:

Both option 1 and option 2 are introduced, but option 2 replaces a corresponding existing test case.

Issue 1-2-4: FR1 PDSCH demodulation requirements selection from 38.101-4: TDD pattern

Way forward:

- Option 1: FR1 30-1, 7D1S2U, S=6D4G4U.

- Option 2: FR1 30-5, DSUU, S=12D2G.

- Other options are not precluded.

Issue 1-2-5: FR1 PDSCH demodulation requirements selection from 38.101-4: Test case

Way forward:

- Conducted:

  - Option 1: 1-8 and 1-4 in 5.2.3.2.1.

  - Option 2: test num. 1-2, 1-4 in table 5.2.3.2.1-3.

  - Option 3: Test 1-1 in Table 5.2.3.2.1-3.

  - Other options are not precluded.

- Radiated:

  - Option 1: Test 1-1 in Table 5.2.2.2.1-3.

  - Other options are not precluded.

Issue 1-2-6: FR2-1 PDSCH demodulation requirements selection from 38.101-4: Rank.

Way forward:

Consider rank 1 and FFS on rank 2.

Issue 1-2-7: FR2-1 PDSCH demodulation requirements selection from 38.101-4: Modulation order and Channel model.

Way forward:

Consider 64QAM with TDLA30-300 and FFS on other modulations.

Issue 1-2-9: FR2-1 PDSCH demodulation requirements selection from 38.101-4: TDD pattern.

Way forward:

- Option 1: FR2 120-1, DDDSU, S=10D2G2U.

- Option 2: FR2.120-2, DDSU, S=11D3G0U.

- Other options are not precluded.

Issue 1-2-10: FR2-1 PDSCH demodulation requirements selection from 38.101-4: Test case

Way forward:

- Option 1: test 1-1, 1-3 and 2-3 in 7.2.2.2.1 of TS 38.101-4.

- Option 2: test 1-1, 1-2, 1-3 in 7.2.2.2.1 of TS 38.101-4.

- Option 3: test 1-1 in 7.2.2.2.1 of TS 38.101-4.

- Other options are not precluded.

Issue 1-3-2: FR1 PDCCH demodulation requirements selection from 38.101-4: Antenna configuration

Way forward:

- Conducted

  - 1Tx4Rx

  - FFS on 2Tx4Rx.

- Radiated

  - 1Tx2Rx

  - FFS on 2Tx2Rx.

Issue 1-3-3: FR1 PDCCH demodulation requirements selection from 38.101-4: CORESET duration.

Way forward:

- Option 1: Only 1.

- Option 2: 1 and 2.

Issue 1-3-4: FR1 PDCCH demodulation requirements selection from 38.101-4: Aggregation level

Way forwards:

- Option 1: 4 for 1Tx and 8 for 2Tx.

- Option 2: 2 and 4 for 1Tx.

- Other options are not precluded.

Issue 1-3-5: Test case selection for FR1 PDCCH demodulation requirements from 38.101-4

Way forward:

- Conducted

  - 1Tx4Rx

    - Option 1: test number 2 in table 5.3.3.2.1-1.

  - 2Tx4Rx

    - Option 1: test number 1 in table 5.3.3.2.2-1

- Radiated

  - 1Tx2Rx

    - Option 1: Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

Issue 1-3-6: FR2-1 PDCCH demodulation requirements selection from 38.101-4: Antenna configureation

Way forward:

- 1Tx2Rx

- FFS on 2Tx2Rx.

Issue 1-3-7: FR2-1 PDCCH demodulation requirements selection from 38.101-4: CORESET duration

Way forward:

- Option 1: Only 1.

- Option 2: 1 and 2.

Issue 1-3-8: FR2-1 PDCCH demodulation requirements selection from 38.101-4: Aggregation level

Way forward:

- Option 1: 4.

- Option 2:

  - 2 and 4 for 1Tx

  - 8 and 16 for 2Tx

Issue 1-3-9: FR2-1 PDCCH demodulation requirements selection from 38.101-4: Test case

Way forward:

- 1Tx2Rx

  - Option 1: All tests in Table 7.3.2.2.1-1.

  - Option 2: Test 1- 2 in Table 7.3.2.2.1-1.

  - Other options are not precluded.

- 2Tx2Rx

  - Option 1: All tests in Table 7.3.2.2.2-1.

  - Other options are not precluded

Issue 1-4-2: Test case selection for CQI reporting from 38.101-4.

Way forward:

- Wideband CQI reporting

  - Conducted:

    - Option 1: Section 6.2.3.2.2.1.

    - Option 2: Section 6.2.3.2.2.

    - Other options are not precluded.

  - Radiated:

    - Option 1: Section 8.2.2.2.2 for FR2-1.

    - Other options are not precluded.

- Sub-band CQI reporting

  - Conducted:

    - Option 1: Section 6.2.3.2.2.2.

    - Option 2: Section 6.2.3.2.2.

    - Other options are not precluded.

Issue 1-5-1: PBCH demodulation requirements selection from 38.101-4: SSB block index configuration

Way forward:

- Option 1: both known and unknown.

- Option 2: Other options are not precluded.

Issue 1-5-2: PBCH demodulation requirements selection from 38.101-4: Antenna configuration

Way forward:

- Conducted:

  - Option 1: Only 4Rx.

  - Option 2: Other options are not precluded.

- Radiated

  - Option 1: 2Rx.

Issue 1-6-1: Impacted specifications

Way forward:

- Option 1: Specify new mIAB performance requirements in TS 38.174 and conducted/radiated conformance testing in TS 38.176-1/2.

Issue 1-6-2: How to add mIAB-MT requirement to current specification?

Way forward:

- Option 1: Add a new section to state that demodulation performance requirements for mIAB-MT

  - Option 1a: New section index with suffix (e.g., Suffix A).

  - Option 1b: New section name includes “mobile IAB”.

Issue 1-6-3: Applicability rule for mIAB-MT requirement

Way forward:

- Option 1: Specify applicability rules for mIAB-MT performance requirements to define clearly which IAB-MT test are applicable since some of legacy IAB_MT test shall be reused.

mIAB-DU demodulation requirements

Issue 2-2-1: How to introduce mIAB-DU demodulation requirements?

Way forward:

Start with section 4.6 in TS38.174 on applicability of requirements to apply IAB-DU requirements for mIAB-DU. Companies could bring proposals and draft CR in the next meeting.

RRM:

WF can be found in R4-2400767 and official summary in R4-2403326.

Random access requirements applicability

- Random access requirements in Clause 12.1.1.2, TS 38.174 are applicable to mIAB-MT.

Issue 1-4: Timing requirements

- Existing timing requirements in Section 12.2, TS 38.174  are applicable to mIAB-M

Issue 1-5: Timer accuracy requirements

- RAN4 to define mIAB-MT timer accuracy requirements by reusing UE requirements from Clause 7.2 of TS 38.133.

Issue 1-6: UL spatial relation switch requirements

- RAN4 to introduce Uplink spatial relation switch delay requirements for mIAB-MTs by reusing UE requirements in Section 8.12 from TS 38.133

Issue 1-8: 2-step RACH

- Only introduce 4-step non-contention based RA test based on the corresponding UE test in TS 38.133

Issue 1-9: Measurement performance requirements

- The measurement performance requirements for mIAB-MT should be introduced in Clause 12.5B of TS 38.174. The requirements to be reused from TS 38.133 can be found in the list below:

    - Clause 10.1.2 – FR1 RSRP

    - Clause 10.1.3 – FR2 RSRP

    - Clause 10.1.6 – RSRP reporting mapping

    - Clause 10.1.7 – FR1 RSRQ

    - Clause 10.1.8 – FR2 RSRQ

    - Clause 10.1.11 – RSRQ reporting mapping

    - Clause 10.1.12 – FR1 SINR

    - Clause 10.1.13 – FR2 SINR

    - Clause 10.1.16 – SINR report mapping

    - Clause 10.1.19 – FR1 L1-RSRP

    - Clause 10.1.20 – FR2 L1-RSRP

Issue 1-10: Measurement accuracy test cases

- Introduce the following measurement accuracy test cases based on the corresponding tests from TS 38.133:

- FR1 SS-RSRP accuracy test : Section A.6.7.1.1

- FR1 SS-RSRQ accuracy test: Section A.6.7.2.1

- FR1 SS-SINR accuracy test: Section A.6.7.3.1

- FR1 L1-SINR accuracy test: Section A.6.7.4.1

- FR2 SS-RSRP accuracy test : Section A.7.7.1.1

- FR2 SS-RSRQ accuracy test: Section A.7.7.2.1

- FR2 SS-SINR accuracy test: Section A.7.7.3.1

- FR2 L1-SINR accuracy test: Section A.7.7.4.1

Issue 1-11: Transmit timing test handling

- Legacy NR IAB-MT Transmit Timing Tests for FR1/FR2-1 (Clauses G.2.2.1.1/2 of TS 38.174) shall be applicable and reused for mIAB-MTs.

Issue 1-12:  UL spatial relation switching tests

- RAN4 to define MAC-CE based uplink spatial relation switch and RRC based spatial relation switch test cases by reusing UE test cased defined in A.5.5.9.1 and A.5.5.9.1 of TS 38.133.

Issue 1-13: Test configurations

- Proposals

  - Option 1: Reuse the test configurations in Annex G.1 for the mIAB-MT RRM tests.

  - Option 2: changes/additions are needed to the test configuations

  - Option 3: discuss needed changes based on the draft CRs

  - Option 4: others

- Agreement:

  - Take Option 1 as baseline, discuss changes/additions based on the draft CRs.

Issue 1-14: HO test cases

- Agreement: Postpone discussion to next meeting and consider all draft CRs at the same time

Issue 1-11: CR split

| Requirements/Tests | Company |
| --- | --- |
| Measurement Accuracy requirements | Qualcomm |
| Measurement Accuracy test cases | Qualcomm |
| Handover tests | Ericsson |
| Active TCI switch tests | Huawei |
| Measurement procedures – intra-frequency measurement tests |  |
| Transmit timing tests | Nokia |
| Signaling characteristics | Qualcomm |
| UL spatial relation switch | Huawei |

#### 2.4.2	Remaining Open issues

60% of items related to the Performance Part have been accomplished.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #110 contributions:

R4-2402502	Draft CR to 38.174 for mIAB RF requirement maintenance, Ericsson.

R4-2400650	On mobile-IAB RF conformance requirements, Qualcomm Germany.

R4-2400651	DraftCR to TS 38.176-1 on RF conformance requirements for NR Mobile IAB, Qualcomm Germany

R4-2400652	DraftCR to TS 38.176-2 on RF conformance requirements for NR Mobile IAB, Qualcomm Germany

R4-2402251	Discussion on mobile IAB conformance testing, Nokia, Nokia Shanghai Bell.

R4-2402499	On mIAB RF Conformance test, Ericsson.

R4-2402500	Draft CR to 38.176-2 for mIAB RF conformance testing, Ericsson.

R4-2402501	Draft CR to 38.176-1 for mIAB RF conformance testing, Ericsson.

R4-2400649	Discussion on mobile-IAB demodulation performance requirements, Qualcomm Germany.

R4-2401558	On demodulation requirement for mIAB-MT and mIAB-DU, Ericsson.

R4-2401582	View on BS demodulation requirements for mobile IAB, Samsung.

R4-2401706	Discussion on demodulation performance requirements for mobile IAB, Huawei,HiSilicon.

R4-2402564	On mIAB Demodulation Performance Requirements, Nokia, Nokia Shanghai Bell.

R4-2401335	DraftCR on maintenance for RRM requirements for mIAB, Huawei,HiSilicon.

R4-2401606	RRM performance requirements for mobile IAB, Qualcomm Incorporated

R4-2402054	Discussion on Mobile IAB RRM Performance Requirements, Dell Technologies.

R4-2402561	On mIAB RRM Core Requirements Maintenance, Nokia, Nokia Shanghai Bell.

R4-2402562	draftCR to TS38.174 on Missing and Applicable mIAB-MT Requirements, Nokia, Nokia Shanghai Bell.

R4-2402563	On mIAB RRM Performance Requirements, Nokia, Nokia Shanghai Bell.

R4-2402824	Draft CR to 38.133 on handover test cases for mIAB-MT in Annex G of TS 38.174, Ericsson.

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template