3GPP TSG RAN Meeting #103								          RP-240134

Maastricht, Netherlands, March 18-21, 2024

Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | Low-power wake-up signal and receiver for NR (LP-WUS/WUR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LPWUS |  |  |  |  |
| Unique ID | 1020094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234056 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: n/a | Core part: 8% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Xueming Pan |
|  | Company | vivo |
|  | Email | panxueming@vivo.com |
| Rapporteur | Name | Shinya Kumagai |
|  | Company | NTT DOCOMO |
|  | Email | shinya.kumagai.yw@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

LP-WUS and LP-SS design

Agreement

Support both OOK-1 and OOK-4 for LP-WUS.

- FFS how OOK-1 and OOK-4 are specified

- For OOK-4, M<=4, FFS supported values

- The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol

  - FFS different SCS.

Agreement

Further study the following options for LP-SS:

- Option 1: OOK-1

- Option 2: OOK-4 with M=1,2,4,[8]

- The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation

  - FFS: different SCS

Agreement

For LP-SS design from RAN1 perspective, consider at least the following as the design target:

- For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s.

  - FFS: X

  - Note: Y is chosen for evaluating LP-SS design.

  - Network overhead and network power consumption are to be considered

Agreement

The ‘ON-OFF’ pattern for OOK symbols of LP-SS is based on binary sequence(s)

- FFS binary sequence(s) details, including the sequence type, the number of sequences, and the sequence length

- FFS overlaid OFDM sequences, if supported

Agreement

For the overlaid OFDM sequence(s) for LP-SS, consider the following options for further down-selection:

- Option 1: Do not specify the overlaid OFDM sequences(s)

- Option 2: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation without targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

- Option 3: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

- For Option 3, it is up to RAN4 to make decision on whether/how to define the RRM measurement requirement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

Agreement

For RAN1 evaluation purpose, the SNR to achieve the coverage of PUSCH for message3 is determined for OOK-based LP-WUR and OFDM-based LP-WUR, respectively.

- Companies are encouraged to report the SNR, together with the associated assumptions as listed in the table below.

|  | Bandwidth for LP-WUS signal (MHz) | NF for LP-WUR (dB) | Gain of antenna element (dBi) assumed for LP-WUR: e.g., -3 dBi for redcap UE and e.g., 0dBi for non-redcap UE | # of Tx chains for LP-WUS/LP-SS transmission, e.g., 2 Note: The number of Tx chains for LP-WUS/LP-SS transmission is assumed the same as the number of RX chains for MSG3 reception | MIL value of MSG3: taking redcap UE /non-redcap UE @dense urban 2.6GHz | The SNR (dB) to achieve the coverage of PUSCH for message3 |
| --- | --- | --- | --- | --- | --- | --- |
| Companyname-01 |  |  |  |  |  |  |

LP-WUS operation in IDLE/INACTIVE modes

Agreement

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period.

  - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time

  - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported)

- FFS eDRX, if supported

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

LP-WUS operation in CONNECTED modes

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.

Agreement

For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits

- FFS value X, which is no more than [8 or 16]

Agreement

For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following

- LP-WUS processing time

- MR transition time for ramp up

- Time/frequency synchronization of MR

- FFS whether UE can report supported minimum time gap from candidate values

FFS: Whether the minimum time gap values can be more than one

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

2.1.2	Remaining Open issues

- Remaining details of an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes

  - Remaining details of OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

    - Remaining details of LP-SS based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences.

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template