3GPP TSG RAN Meeting #103									RP-240137

Maastricht, Netherlands, March 18-21, 2024

Status Report to TSG

Agenda item:		9.4.4.12

| WI / SI Name | Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_TRP_TRS_enh |  |  |  |  |
| Unique ID | 970082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234064 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Dec/2023 | Performance part: Jun/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 75% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #110

- General

- Topic summary for FR1 TRP TRS WI in [33]

- Ad-hoc minutes for NR_FR1_TRP_TRS_enh was approved in [37]

- CR to TR 38.870 on TRP TRS test method was endorsed in [17]

- Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results is presented in [19]

- WF for Rel-18 FR1 TRP TRS was approved in [31]

Issue 1-1-1: Performance metric for Coherent UE support multiple TPMI index 2~5

Options for next meeting:

  - Option 1 (averaging TRPTPMIx), TRPavg_TPMI: Option 1a averaging 4 TPMIs TRPs, Option 1b averaging 2 TPMIs TRPs

  - Option 2 (Max EIRPTPMIx), TRPmax_EIRP_TPMI

  - Option 3 (averaging Weighted Radiated Powers) with 4 TPMIs, FFS naming

    - Averaging of 4 partial TRPs

  - Option 4 (weighted averaging TRPsTPMIx), TRPweighted_avg_TPMI

Agreements:

RAN4 will consider above options and make decisions on a reference/baseline metric next meeting based on majority view.

The following comparison criteria should be considered for making decisions next meeting.

| # | Criteria |
| --- | --- |
| 1 | Testing time (calculated based on R4-2311672), considering multiple AC stabilization times |
| 2 | Performance metric consistency |
| 3 | Regulatory impacts |
| 4 | Statistical properties |
| 5 | Alignment with Other SDOs |
| 6 | OEM antenna design |
| 7 | Operator network deployment |
| 8 | Representative of the operation in the field （Realism） |

Option 3 and option 4 description listed in Annex of this WF is for information.

Issue 1-1-3: Correction of phase error assumption agreed last meeting for UL MIMO

Agreements:

- correct the range of variable PV in the annex of R4-2321209 from [0, 10(4/10)] to [10(0/10), 10(4/10)].

Issue 1-1-5: Test mode for UL MIMO (including coherent/non-coherent UE)

Agreements:

- UE Test mode is not needed for UL MIMO.

Issue 2-1-1: Analysis and conclusion of RAN4 Rel-18 AC Lab alignment activity

Agreement:

- Based on analysis in R4-2401539 and agreed pass fail limits, RAN4 conclude AC lab alignment activity with the following outcome:

  - All 8 labs are aligned at band n78

  - 6 labs are aligned at band n28

Issue 2-1-2: Updated AC Lab alignment activity

Agreement:

- Unaligned n28 labs (lab1 and lab3) are allowed to do retesting before RAN4#110bis meeting. They can also contribute n28 measurement campaign results, the results can be considered as RAN4 data pool when lab alignment is confirmed.

Issue 2-2-1: RC Lab alignment criteria

Agreement:

- Confirm the RC lab alignment pass/fail limits as 0.75*MU for both TRP and TRS, i.e., 0.75*1.9dB for TRP browsing mode, 0.75*2.08 for TRP talk mode, 0.75*2.28dB for TRS browsing mode, 0.75*2.43dB for TRS talk mode.

Issue 2-2-2: RC Lab alignment outcome

Agreement:

- based on the analysis in R4-2401539, confirm the phase 1 RC lab alignment outcome, the 5 labs are well aligned with Reverb Chamber system.

- After receiving last two labs’ RC results, the reference value will be updated, and the alignment outcome of all RC test labs should be rechecked and confirmed.

Issue 2-2-3: updated RC Lab alignment schedule

Agreement:

- Full RC lab alignment activity is expected for completion in RAN4#110bis meeting. Last 2 RC labs are expecting to receive the LADs to their lab ASAP.

Issue 2-2-4: RC vs AC harmonization criteria

Agreement:

AC results are reference for comparison. some initial options for further consideration

- Option 1: compare the averaged value of each method

- Option 2: compare the max deviation of RC and AC from each test lab

- Option 3: compare the max deviation of RC and AC across test labs with same device

Further discuss whether some of RC configurations should be clearly specified, if harmonization conclusion is reached.

Issue 2-3-1: updated working procedure for Rel-18 TRP TRS Performance Test Campaign to define requirements

Agreement:

- The devices for measurement campaign should be within the year of production: from 2021 to 2024

- Confirm the UE information disclosure should use the template in R4-2321191

Issue 2-3-2: Confirm TRP TRS measurement campaign

Agreement:

- Confirm the updated performance measurement campaign arrangements.

  - Browsing mode measurement campaign and requirements: measurement campaign started after RAN4#110 meeting.

  - Talk mode measurement campaign and requirements: measurement campaign started after RAN4#110 meeting.

  - Collecting measurement results from aligned test labs in RAN4#110bis meeting, start the discussion of potential requirements.

  - Conclude final TRP TRS requirements in RAN4#111 meeting, close performance WI.

- Further contribution of measurement results to RAN4 data pool after RAN4#110bis is allowed.

Issue 2-3-3: Measurements plan shared by test labs for performance measurement campaign

For information: Latest plan from test labs on device measurement for RAN4 data pool

Within the above table, the following devices from operators have been shared to volunteer labs at RAN4#110 meeting for measurement campaign before RAN4#110bis:

Agreement:

- Based on current feedback of test labs, EU operators could hand carry and transfer devices to test labs during RAN4#110 and RAN#103 meeting (18th Mar).

- Test labs share the UE model information to MCC based on agreed working procedure via template R4-2321191 as soon as possible, better no later than RAN#103 meeting. According to actual measurement progress in each test labs, each test lab can further update the UE information with MCC.

- Statistics of UE information can be shared by RAN4 secretary after post-processing to RAN4 reflector before RAN4#110bis meeting.

Issue 2-4-1: single requirements or 2 set of requirements in RAN4 and other certification bodies?

- Options

  - Option 1: Single requirement each band in RAN4 spec, list scaling factor as a note for these band

  - Option 2: Two sets of requirements for these bands in RAN4 spec

Way Forward

- Further discuss next meeting.

Issue 2-4-2: whether UE should pass both requirements?

Agreements:

- Based on traditional certification process of OTA requirements. Passing single requirement per band of each UE is sufficient.

Issue 2-4-3: How to scale the defined large CBW to narrow CBW requirements?

Agreement:

- OTA Requirements for additional CBW should be defined based on scaling of current OTA requirements at that band.

- The REFSENS RB scaling factor as a starting point which will be checked and confirmed based on measurement results. Considering additional factors is not precluded.

Issue 2-4-4: Testing time reduction method for two CBW measurement

Agreement:

- RAN4 further study the feasibility of single point offset method for different CBW measurements of same band.

Issue 2-4-5: detailed test parameters for additional CBW

- Options

  - Option 1: Case B in figure below (new CBW with different center frequency of original CBW)

  - Option 2: Case C in figure below (new CBW with same center frequency of original CBW)

Figure 1: two options for test parameter of alternative CBW

Agreement:

- Option 1: Case B for additional CBW parameters, confirm next meeting.

- Some measurements results are needed to confirm case B.

Issue 2-4-6: A measurement activity among some test labs to confirm the above scaling factor and proper center frequency

Agreement:

- Some measurements are needed.

Issue 2-4-7: Limit the additional CBW only for bands n28/n41/n77/n78 without further extension and stated in the spec

Agreement:

- Limiting the additional CBW only for bands n28/n41/n77/n78 without further extension and stated in the spec.

Issue 2-6-2: How to define TRP PC3 requirements based on PC2

Agreement:

- RAN4 further discuss and decide how to define TRP PC3 requirements based on PC2. FFS Whether TDD and FDD could be different.

Issue 2-6-4: Schedule of Rel-18 TRP TRS requirements.

Ad-hoc agreement:

- The AC lab alignment and measurement campaign is progressing good, no WI extension is considered.

Annex: Definition of Options 3 and 4 for Issue 1-1-1 (for information)

Option 3

The procedure named option 3 as proposed in R4-2400269 may run into difficulties for some of radiation pattern shapes. A simplified procedure is given below. This procedure is simple to implement and avoids the potential difficulties such as finding the main lobe, coordinate transformation, etc.

- Measure radiated power for all 4 TPMI values, namely TPMI 2, 3, 4 and 5 at all grid points. The measurement can be done either for a single TPMI for all grid points or repeat all 4 TPMI per grid point. The aim is to make available the radiated power values at all grid points for all 4 TPMI indexes.

- Define an EIRP threshold for the total component EIRPtotal,threshold, above which the weighting value Wi is assigned to a default value of Wi=1.

EIRPtotal,threshold is defined to be [-10 to -15] dB below the peak of the respective radiation pattern (total component)

For the grid points with EIRPs (total component) below EIRPtotal,threshold, Wi is defined to be [0.5 to 0.75].

- Calculate Weighted Radiated Power (WRP) using equation (1) and (2) with the above weights Wi

WRP= 1Kθ=0πϕ=02πWiEIRPθθ,ϕ+EIRPϕθ,ϕsinθdϕdθ		         (1)

where Wi denotes weight at each grid point and K is given below

K= θ=0πϕ=02πWisinθdϕdθ		        (2)

- Repeat step 2 to 3 for all 4 TPMI indexes to obtain 4 WRP values, e.g. WRPTPMI2, WRPTPMI3, WRPTPMI4, WRPTPMI5 corresponding TPMI index 2 to 5

- The final ERP metric for Option 3 is WRPOption3 and defined as the average of WRP values WRPTPMI2, WRPTPMI3, WRPTPMI4, WRPTPMI5

Option 4:

The procedure would be as follows:

- Perform the spherical measurement using the 4 TPMI as described in the common procedure in TR 38.870, section 7.4.3.3.

- Calculate the TRP per TPMI as per clause 5.1.1 in TR 38.870, resulting in:

TRPTPMI=2 ; TRPTPMI=3 ; TRPTPMI=4 ; TRPTPMI=5

- Determine the maxTRPTPMI=2 ; TRPTPMI=3 ; TRPTPMI=4 ; TRPTPMI=5. For the example, let assume TRPTPMI=2provide the highest number.

- Determine the TRPweighted_avg applying a larger scaling factor to the max case identified in step 3 providing the best overall TRP, while the remaining values are scaled with a smaller factor.

TRPweighted_avg=10logw2·10TRPTPMI=2+w3·10TRPTPMI=3+w4·10TRPTPMI=4+w5·10TRPTPMI=5

The weights need to be discussed, but total sum of weights must be equal to 1:

i=25wi=1

Using the example with TRPTPMI=2being the highest number:

TRPweightedavg=10log0.4·10TRPTPMI=2+0.2·10TRPTPMI=3+0.2·10TRPTPMI=4+0.2·10TRPTPMI=5

Please note this is just a simplistic formulation for discussion purposes.

2.4.2	Remaining Open issues

- Performance part

    - Concluding coherent UL MIMO performance metric

    - Concluding RC harmonization activity

    - Rel-18 TRP TRS measurement campaign

    - Specify TRP TRS Requirements

2.5	RAN5

2.5.1	Agreements

RAN5 #102

- N/A [RAN5 work has been finalized]

2.4.2	Remaining Open issues

- None

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #110

- R4-2400155	CR to TR38.870 on UL MIMO radiated output power metric	Apple

- R4-2400156	On multi-Tx radiated output power methodologies	Apple

- R4-2400217	3GPP Rel-18 TRP TRS LAD measurements for AC lab alignment - Sporton	Sporton International Inc

- R4-2400222	Discussion on Single-layer UL-MIMO TRP test method	Qualcomm Incorporated

- R4-2400268	On AC and RC alignment criteria	Huawei Tech.(UK) Co.. Ltd

- R4-2400269	on TRP metric for UL MIMO	Huawei Tech.(UK) Co.. Ltd

- R4-2400270	on UL MIMO phase drift	Huawei Tech.(UK) Co.. Ltd

- R4-2400523	Views on Rel18 TRP TRS lab alignment timelines and performance campaign working procedure	Apple

- R4-2400551	3GPP Rel-18 TRP TRS LAD measurements for AC lab alignment - Sporton n28	Sporton International Inc

- R4-2400706	On Single-Layer UL MIMO for non-coherent/coherent UEs	Keysight Technologies UK Ltd

- R4-2400717	ELEMENT - Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment LAD1 and LAD2	Element Materials Technology

- R4-2400718	ELEMENT - Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment LAD3 and LAD4	Element Materials Technology

- R4-2400870	3GPP Rel-18 TRP TRS RC lab alignment activity from SRTC	SRTC

- R4-2400871	3GPP Rel-18 TRP TRS AC lab alignment activity from SRTC	SRTC

- R4-2400980	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

- R4-2401183	Discussion on 2TX configuration	xiaomi

- R4-2401537	draft CR to TR 38.870 on TRP TRS test method	vivo

- R4-2401538	General discussions and MU impacts of TRP TRS topics	vivo

- R4-2401539	Analysis of 3GPP TRP TRS AC lab alignment and RC harmonization measurement results-phase 2	vivo

- R4-2401540	Measurement results of 3GPP Rel-18 TRP TRS AC lab alignment activity-full set	vivo

- R4-2401541	Measurement results of 3GPP Rel-18 TRP TRS RC harmonization activity-full set	vivo

- R4-2401543	Performance metric for coherent UL-MIMO	vivo, CAICT, Huawei, HiSilcon, Xiaomi, OPPO, MediaTek

- R4-2401800	TRP simulation of coherent UE with power and phase variation	OPPO

- R4-2401801	3GPP Rel-18 TRP TRS LAD measurement for RC harmonization_n28	OPPO

- R4-2401802	Update on LAD1 measurement data for lab alignment	OPPO

- R4-2401803	3GPP Rel-18 TRP TRS LAD measurement for AC lab alignment_n28	OPPO

- R4-2401833	Discussion on TRP requirements	CAICT

- R4-2401834	CAICT measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity-v2	CAICT

- R4-2401835	CAICT measurement results for 3GPP Rel-18 TRP TRS RC harmonization activity-v2	CAICT

- R4-2402254	Discussion on FR1 TRP TRS remaining issues	Samsung

- R4-2402368	CR for CA in section 4.3.5 of TR 38.870	Huawei, HiSilicon

- R4-2402418	Adding NR channel bandwidths for OTA TRP/TRS requirements testing	Orange, Vodafone, T-Mobile USA, Deutsche Telekom

- R4-2402671	Topic summary for [110][331] NR_FR1_TRP_TRS_enh	Moderator (vivo)

- R4-2402823	Effect of Random Phase Offset on UL MIMO	Lenovo

- R4-2402852	Issues related to the measurement campaign for FR1 OTA TRP/TRS minimum performance requirements definition	TELECOM ITALIA S.p.A., Orange, Vodafone

- R4-2402875	Way forward on [110][331] NR_FR1_TRP_TRS_enh	Vivo

- R4-2402985	Ad-hoc meeting minutes for [110][331] NR_FR1_TRP_TRS_enh	Vivo

- R4-2402986	Huawei measurement results for 3GPP Rel-18 TRP TRS RC harmonization	Huawei

- R4-2403066	Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity	SGS Wireless

RAN5 #102

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template