3GPP TSG-RAN5 Meeting #102	R5-240689

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2909 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM test case 17.7.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM test case 17.7.2.2 - Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell |  |  |  |  |  |  |  |  |
| Summary of change: |  | RedCap RRM test case 17.7.2.2 - Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.7.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

17.7.2.2	Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is still missing.

17.7.2.2.1	Test purpose

To verify that the inter-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

17.7.2.2.2	Test applicability

This test applies to all types of NR RedCap UE from Release 17 onwards.

17.7.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.17.7.2.2.

17.7.2.2.4	Test description

Same as the test description given in clause 7.7.2.2.4.

17.7.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 7.7.2.2.4.1 with following exceptions:

- Table 7.7.2.2.4.1-1 is replaced by Table 17.7.2.2.4.1-1;

- Table 7.7.2.2.4.1-2 is replaced by Table 17.7.2.2.4.1-2;

Table 17.7.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.7.2.2-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 17.7.2.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.15-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.7.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.3.4.1 |  |
| Exceptions to connection diagram | N/A |  |  |

17.7.2.2.4.2	Test procedure

Same as the test procedure given in clause 7.7.2.2.4.2 with following exceptions:

-	Table 7.7.2.2.5-1 is replaced by Table 17.7.2.2.5-1;

-	Table 7.7.2.2.5-2 is replaced by Table 17.7.2.2.5-2;

-	Table 7.7.2.2.5-3 is replaced by Table 17.7.2.2.5-3;

-	Table 7.7.2.2.5-4 is replaced by Table 17.7.2.2.5-4.

17.7.2.2.4.3	Message contents

Same as the message contents given in clause 7.7.2.2.4.3.

17.7.2.2.5	Test requirement

Same as the test requirements given in clause 7.7.2.2.5 with followings exceptions:

-	Table 7.7.2.2.5-1 is replaced by Table 17.7.2.2.5-1;

-	Table 7.7.2.2.5-2 is replaced by Table 17.7.2.2.5-2;

-	Table 7.7.2.2.5-3 is replaced by Table 17.7.2.2.5-3;

-	Table 7.7.2.2.5-4 is replaced by Table 17.7.2.2.5-4.

Table 17.7.2.2.5-1: Cell-specific test parameters

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  |  | Freq1 | Freq2 |
| SSB Configuration |  |  | SSB.1 FR2 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP Dedicated DL BWP |  | DLBWP.0.1 |  |
|  |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP Dedicated UL BWP |  | ULBWP.0.1 |  |
|  |  |  | ULBWP.1.1 |  |
| TRS configuration |  |  | TRS.2.1 TDD | - |
| TCI state |  |  | TCI.State.0 | - |
| Data RBs allocated |  |  | 66 |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD | - |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC configuration |  |  | SMTC.1 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation conditions |  |  | AWGN |  |
| Antenna configuration |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table 17.7.2.2.5-2: OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | Setup 1 in clause A.9.1 |  |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  |  |
| Note1 | dBm/15kHzNote4 | -94.03+TT | -94.03+TT | -94.03+TT | -94.03+TT |
| Note1 | dBm/SCSNote3 | -85.0+TT | -85.0+TT | -85.0+TT | -85.0+TT |
|  | dB | -1.75+TT |  | -1.75+TT |  |
| SSB_RPNote2 | dBm/SCS Note4 | -86.75+TT | -86.75+TT | -88+TT | -88+TT |
| SS-RSRQNote2 | dB | -14.75+TT | -14.75+TT | -15.56+TT | -15.56+TT |
|  | dB | -1.75+TT | -1.75+TT | -3+TT | -3+TT |
| IoNote2 | dBm/95.04 MHz Note4 | -53.8+TT | -53.8+TT | -54.25+TT | -54.25+TT |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Table 17.7.2.2.5-3: evaluation limits for the reported values for Test 1 and Test 2 absolute accuracy rules R1, R2, R4, R5

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (Cell 1) | FFS | FFS |
| Highest reported value (Cell 1) | FFS | FFS |
| Lowest reported value (Cell 2) | FFS | FFS |
| Highest reported value (Cell 2) | FFS | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 All bands |
| Lowest reported value (Cell 1) | FFS | FFS |
| Highest reported value (Cell 1) | FFS | FFS |
| Lowest reported value (Cell 2) | FFS | FFS |
| Highest reported value (Cell 2) | FFS | FFS |

Table 17.7.2.2.5-4: evaluation limits for the reported values for Test 1 and Test 2 relative accuracy rules R3, R6

| UE power class 3 |  |  |
| --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 All bands |
| Lowest value (RSRQ report Cell 2 – RSRQ report Cell 1) | FFS | FFS |
| Highest value (RSRQ report Cell 2 – RSRQ report Cell 1) | FFS | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 All bands |
| Lowest value (RSRQ report Cell 2 – RSRQ report Cell 1) | FFS | FFS |
| Highest value (RSRQ report Cell 2 – RSRQ report Cell 1) | FFS | FFS |

<End of modified section 1>