3GPP TSG-RAN5 Meeting #102	R5-240798

Athens, February 26 - March 1 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2967 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) test case 17.6.2.1 including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR SA FR2-FR2 Event triggered reporting test case 17.6.2.1 is missing in 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case17.6.2.1 will remain missingin TS 38.533 and rel-17 RedCap WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.6.2.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

### 17.6.2	Inter-frequency measurements for RedCap

#### 17.6.2.0	Minimum conformance requirements

The requirements in clause 17.6.2.0 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2 Rx RedCap UE:

-	SS-RSRP related side conditions given in 38.133 [6] clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in 38.133 [6] clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 38.133 [6] 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to 38.133 [6] Annex B.2.3 for a corresponding Band.

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index_RedCap. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index_RedCap.

Tidentify_inter_without_index_RedCap = (TPSS/SSS_sync_inter_RedCap + T SSB_measurement_period_inter_RedCap) ms

Tidentify_inter_with_index_RedCap = (TPSS/SSS_sync_inter_RedCap + T SSB_measurement_period_inter_RedCap + TSSB_time_index_inter_RedCap) ms

Where:

TPSS/SSS_sync_inter_RedCap: it is the time period used in PSS/SSS detection given in table 17.6.2.0-1.

TSSB_time_index_inter_RedCap: it is the time period used to acquire the index of the SSB being measured given in table 17.6.2.0-2.

TSSB_measurement_period_inter_RedCap: equal to a measurement period of SSB based measurement given in table , table 17.6.2.0-3, table .

For 2 Rx RedCap UE:

Mpss/sss_sync_inter_RedCap: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_inter_RedCap = 64 samples. For a UE supporting FR2 power class 2, 3, 4 or 7, Mpss/sss_sync_inter_RedCap = 40 samples.

MSSB_index_inter_RedCap: For a UE supporting FR2 power class 1 or 5, MSSB_index_inter_RedCap = 40 samples. For a UE supporting FR2 power class 2, 3,4 or 7, MSSB_index_inter_RedCap = 24 samples.

Mmeas_period_inter_RedCap: For a UE supporting FR2 power class 1 or 5, Mmeas_period_inter_RedCap =64 samples. For a UE supporting FR2 power class 2, 3, 4 or 7, Mmeas_period_inter_RedCap =40 samples.

CSSFinter_RedCap: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps.

Table 17.6.2.0-1: Time period for PSS/SSS detection, (Frequency range FR2) for 2Rx RedCap UE

| Condition NOTE1 | TPSS/SSS_sync_inter_RedCap |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_inter_RedCap Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 Mpss/sss_sync_inter_RedCap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | Mpss/sss_sync_inter_RedCap DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Table 17.6.2.0-2: Time period for time index detection (Frequency range FR2) for 2Rx RedCap UE

| Condition NOTE1 | TSSB_time_index_inter_RedCap |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_inter_RedCap Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_inter_RedCap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | MSSB_index_inter_RedCap DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

When measurement gaps are provided for inter frequency measurements, the RedCap UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1A.4, 10.1A.5, 10.1A.8, 10.1A.9, 10.1A.12 and 10.1A.13, respectively, as shown in table 17.6.2.0-3.

Table 17.6.2.0-3: Measurement period for inter-frequency measurements with gaps (Frequency FR2) for 2Rx RedCap UE

| Condition NOTE1 | T SSB_measurement_period_inter_RedCap |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_inter_RedCap Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 Mmeas_period_inter_RedCap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | Mmeas_period_inter_RedCap DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

For 2 Rx RedCap UE: Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.4.1, 10.1.5.1, 10.1.9.1, 10.1.10.1, 10.1.14.1 and 10.1.15.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 × TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be within Tidentify_inter_without_index_ RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index_ RedCap. Both Tidentify_inter_without_index_ RedCap and Tidentify_inter_with_index_ RedCap are defined in TS 38.133 [6] clause 9.3B.4. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_inter_without_index_ RedCap or Tidentify_inter_with_index_ RedCap as defined in TS 38.133 [6] clause 9.3B.4 or 9.3B.7.1. If a cell which has been detectable at least for the time period Tidentify inter without index_ RedCap or Tidentify inter with index_ RedCap defined in TS 38.133 [6]  clause 9.3B.4 or 9.3B.7.1 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra_ RedCap provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clause 9.3B.2, 9.3B.4, 9.3B.5 and 9.3B.6.3.

#### 17.6.2.1	NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used

Editor's Note: This test case has been completed for the following configurations:

Test frequency f ≤ 40.8 GHz

UE PC3

This test case is incomplete for UE power classes other than PC3 (e.g. PC7)

This test case is incomplete for test frequencies > 40.8 GHz

17.6.2.1.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements for RedCap in 38.133 [6] clause 9.3B.4.

17.6.2.1.2	Test applicability

This test applies to all types of NR RedCap UE from release 17 onwards.

17.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.17.6.2.1.

17.6.2.1.4	Test description

17.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 17.6.1.2.4.1-1.

Table 17.6.2.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 17.6.2.1-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Configure the test requirement and the DUT according to the parameters in Table 17.6.1.2.4.1-2.

Table 17.6.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 17.6.2.1.4.1-3 below.

2. Message contents are defined in clause 17.6.2.1.4.3.

3. There are two cells on 2 different carriers specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 17.6.2.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause TS 38.133 [6] 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-CD-SSB parameters |  | Config 1 | SSB.2 RedCap FR2 |  |
| NCD-SSB parameters |  | Config 1 | SSB.3 RedCap FR2 |  |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5.2 for PC1; 3.5 for other PC |  |

17.6.2.1.4.2	Test procedure

Same as the test procedure given in clause 7.6.2.1.4.2.

17.6.2.1.4.3	Message contents

Same as the message contents given in clause 7.6.2.1.4.3.

17.6.2.1.5	Test requirement

Same as the test requirements given in clause 7.6.2.1.5 with following exceptions:

-	Table 7.6.2.1.5-1 is replaced by Table 17.6.2.1.5-1.

Table 17.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |  |
| Beam AssumptionNote 7 |  |  | 1,2 | Rough |  | Rough |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
|  | Dedicated DL BWP configuration |  |  | DLBWP.1.3 RedCap Note 9 |  | N/A |  |  |
|  | Dedicated UL BWP configuration |  |  | ULBWP.1.3 RedCap Note 10 |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 for CD-SSB |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |  |
| SMTC configuration for NCD-SSB |  |  | Config 1 | SMTC.2 RedCap |  | SMTC.2 RedCap |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 | -87 | -Infinity |  | -87 |
| SSBRP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity |  | -87 |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | -Infinity |  | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity |  | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: VoidNote 3: SSBRP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. Note 9: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dediacted DL BWP2 corresponding to NCD-SSB PRB index; Note 10: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |  |  |

///End of changes