3GPP TSG-RAN5 Meeting #102	R5-240614

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5488 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | E-UTRA V2X spurious emissions MU definition up to 26GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_SL_V2V-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In test case 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions, spurious emissions need to be measured up to 26GHz. However, required measurement uncertainty is not defined beyond 19GHz. Proposed to define MU for the missing frecuency range same as defined NR general spurious test case 6.5.3.1 in 38.521-1, that is 6dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For test case 6.6.3G.1, for range 19GHz < f ≤ 26GHz, defined MU=± 6.0 dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.2 UE Maximum Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.2_1 Maximum Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 for sum of powers of all CCs with f as the higher one among centre frequencies of 2 CCs. | Overall system uncertainty is maximum of UL measurement uncertainty of 2 CCs. |
| 6.2.2A.4 UE Maximum Output Power for CA (3UL CA) | For UL CA_XD: For BWagg ≤ 40MHz Same as 6.2.2 for sum of powers of all CCs For 40MHz < BWagg ≤ 60MHz ±1.4 dB for f ≤ 4.2GHz ±1.6 dB for 4.2GHz < f ≤ 6.0GHz For UL CA_XC_YA: Same as 6.2.2 for sum of powers of all CCs with f as the higher one among centre frequencies of XC or YA. For UL CA_XA_YA_ZA: FFS | Overall system uncertainty is maximum of UL measurement uncertainty of interband or non-contiguous CCs |
| 6.2.2B UE Maximum Output Power for UL-MIMO | Same as 6.2.2 for each antenna |  |
| 6.2.2B_1 HPUE Maximum Output Power for UL-MIMO | Same as 6.2.2_1 for each antenna |  |
| 6.2.2E UE Maximum Output Power for UE category 0 | Same as 6.2.2 |  |
| 6.2.2EA UE Maximum Output Power for UE category M1 | Same as 6.2.2 |  |
| 6.2.2EB UE Maximum Output Power for UE category 1bis | Same as 6.2.2 |  |
| 6.2.2EC UE Maximum Output Power for UE category M2 | Same as 6.2.2 |  |
| 6.2.2F UE Maximum Output Power for UE category NB1 and NB2 | Same as 6.2.2_1 |  |
| 6.2.2FA UE Maximum Output Power for category NB1 and NB2/Power Class 6 | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2G.1 UE maximum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.2 |  |
| 6.2.2G.2 UE maximum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2G.3 UE maximum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2G.4.1 UE Maximum Output Power for V2X Communication / Power class 2 / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.2 |  |
| 6.2.2_s UE Maximum Output Power for subslot/slot TTI | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.3_1 Maximum Power Reduction (MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_2 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.3A.2_1 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.2_2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.3A.3_1 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.3_2 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.4 Maximum Power Reduction (MPR) for CA (3UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 for each antenna |  |
| 6.2.3_2 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH | Same as 6.2.3 |  |
| 6.2.3E Maximum Power Reduction for UE category 0 | Same as 6.2.3 |  |
| 6.2.3EA Maximum Power Reduction for UE category M1 | Same as 6.2.3 |  |
| 6.2.3EB Maximum Power Reduction for UE category 1bis | Same as 6.2.3 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM | Same as 6.2.3 |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM | Same as 6.2.3 |  |
| 6.2.3F Maximum Power Reduction (MPR) for category NB1 and NB2 | Same as 6.2.3 |  |
| 6.2.3G.1.1 Maximum Power Reduction (MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.1.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.2.3 for each CC |  |
| 6.2.3G.1.4 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation | Same as 6.2.3 for sum of power |  |
| 6.2.3G.2.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.2.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3_5 Maximum Power Reduction (MPR) for UL 256QAM | Same as 6.2.3 |  |
| 6.2.3_6 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 256QAM | Same as 6.2.3 |  |
| 6.2.3_s Maximum Power Reduction (MPR) for subslot/slot TTI | Same as 6.2.3 |  |
| 6.2.4 UE Maximum Output Power with additional requirements | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping | Same as 6.2.4 |  |
| 6.2.4_4 Additional Maximum Power Reduction (A-MPR) for UL 256QAM | Same as 6.2.4 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_2 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.4A.2_1 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.4A.2_2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.2.2A.2 |  |
| 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 for each antenna |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 | Same as 6.2.4 |  |
| 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 | Same as 6.2.4 |  |
| 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis | Same as 6.2.4 |  |
| 6.2.4EC Additional Maximum Power Reduction (A-MPR) for UE category M2 | Same as 6.2.4EA |  |
| 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.4G.2 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±1.3 dB for sum of power | Maximum MU within both carriers. |
| 6.2.4_s Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5_s Configured UE Transmitted Output Power for subslot/slot TTI | Same as 6.2.5 |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | Same as 6.2.5 |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.5 for sum of powers of all CCs |  |
| 6.2.5A.1_3 Configured UE transmitted Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) for HPUE | Same as 6.2.5 for sum of powers of all CCs |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.5A.4 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | Same as 6.2.5 for each antenna |  |
| 6.2.5E Configured Transmitted Power for UE category 0 | Same as 6.2.5 |  |
| 6.2.5EA Configured UE transmitted Power for UE category M1 | Same as 6.2.5 |  |
| 6.2.5EB Configured UE transmitted Power for UE category 1bis | Same as 6.2.5 |  |
| 6.2.5F Configured UE transmitted Output Power for UE category NB1 and NB2 | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for -40 dBm <= UE powers <0dBm |  |
| 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for -40 dBm <= UE powers <0dBm |  |
| 6.2.5G.3 Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.2.5G.1 for each CC |  |
| 6.3.2 Minimum Output Power | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2A.4 Minimum Output Power for CA (3UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 for each antenna |  |
| 6.3.2E Minimum Output Power for UE category 0 | Same as 6.3.2 |  |
| 6.3.2EA Minimum Output Power for UE category M1 | Same as 6.3.2 |  |
| 6.3.2EB Minimum Output Power for UE category 1bis | Same as 6.3.2 |  |
| 6.3.2F Minimum Output Power for UE category NB1 and NB2 | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.3.2 |  |
| 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.3.2 |  |
| 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.2 |  |
| 6.3.3 Transmission ON/OFF Power | Transmission OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.2 UE Transmit OFF power for CA (inter-band DL CA and UL CA) | Same as 6.3.3A.1 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.4 UE Transmit OFF power for CA (3UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 for each antenna |  |
| 6.3.3D UE Transmit OFF power for ProSe | Same as 6.3.3 |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 | Same as 6.3.3 |  |
| 6.3.3EA UE Transmit OFF power for UE category M1 | Same as 6.3.3 |  |
| 6.3.3EB UE Transmit OFF power for UE category 1bis | Same as 6.3.3 |  |
| 6.3.3F Transmit OFF power for category NB1 and NB2 | Same as 6.3.3 |  |
| 6.3.3G.1 UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.3.3 |  |
| 6.3.3G.2 UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.3.3 for each CC |  |
| 6.3.3G.3 UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.3 for each CC |  |
| 6.3.4.1 General ON/OFF time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4.1_s General ON/OFF time mask for subslot/slot TTI | Same as 6.3.4.1 |  |
| 6.3.4.2 PRACH and SRS time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4A.1.1 |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.4 General ON/OFF time mask for CA (3UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.5 General ON/OFF time mask for CA (4UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 for each antenna |  |
| 6.3.4C.1 General ON/OFF time mask for Dual Connectivity | Same as 6.3.4A.1.1 |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 | Same as 6.3.4.2 |  |
| 6.3.4EA.1 General ON/OFF time mask for UE category M1 | Same as 6.3.4.1 |  |
| 6.3.4EA.2 PRACH and SRS ON/OFF time mask for UE category M1 | Same as 6.3.4.2 |  |
| 6.3.4EB.1 General ON/OFF time mask for UE category 1bis | Same as 6.3.4.1 |  |
| 6.3.4EB.2 PRACH and SRS ON/OFF time mask for UE category 1bis | Same as 6.3.4.2 |  |
| 6.3.4EC.2 PRACH and SRS ON/OFF time mask for UE category M2 | Same as 6.3.4.2 |  |
| 6.3.4F.1 General ON/OFF time mask for category NB1 and NB2 | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4F.2 NPRACH time mask for category NB1 and NB2 | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4G.1 General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.4G.2 General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.3.4G.3 General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.4G.1 for each CC |  |
| 6.3.4G.4 PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.5.1 Power Control Absolute power tolerance | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5.2 Power Control Relative power tolerance | ±0.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance | ±0.7 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | ±0.7 dB |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | ±0.7 dB |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.4 Power Control Absolute power tolerance for CA (3UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.5 Power Control Absolute power tolerance for CA(SRS carrier based switching) | Same as 6.3.5.1 for SCC |  |
| 6.3.5A.1.6 Power Control Absolute power tolerance for CA (4UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | Absolute power ±1dB, f ≤ 4.2GHz Relative power ±0.7 dB Both requirements apply for each CC | Absolute power is used to check when power is near lower and upper limits of the range. Relative power is used to check step sizes. |
| 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) | Same as 6.3.5A.2.2 |  |
| 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.3.5A.2.3 |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.5 Aggregate Power Control Tolerance for CA (4UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 for each antenna |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 for each antenna |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 for each antenna |  |
| 6.3.5C.2 Power Control Relative power tolerance for Dual Connectivity | Same as 6.3.5A.2.1 |  |
| 6.3.5C.2_1 Power Control Relative power tolerance for asynchronous Dual Connectivity | Same as 6.3.5A.2.1 |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 | Same as 6.3.5.2 |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 | Same as 6.3.5.3 |  |
| 6.3.5EA.1 Power Control Absolute power tolerance for UE category M1 | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5EA.2 Power Control Relative power tolerance for UE category M1 | ±0.7 dB |  |
| 6.3.5EA.3 Aggregate power control tolerance for UE category M1 | Same as 6.3.5.3 |  |
| 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) | Same as 6.3.5.3 |  |
| 6.3.5EB.1 Power Control Absolute power tolerance for UE category 1bis | Same as 6.3.5.1 |  |
| 6.3.5EC.3_1 Aggregate power control tolerance for UE category M2 (CE Mode B) | Same as 6.3.5.1 |  |
| 6.3.5F.1 Power Control Absolute power tolerance for category NB1 and NB2 | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5F.2 Power Control Relative power tolerance for category NB1 and NB2 | ±0.7 dB |  |
| 6.3.5F.3 Aggregate power control tolerance for category NB1 and NB2 | ±0.7 dB |  |
| 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±1.0 dB, f ≤ 3.0GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5G.3 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation | ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.5.1 Frequency Error | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz DL Signal level: ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.2 Frequency error for CA (inter-band DL CA and UL CA) | Same as 6.5.1 |  |
| 6.5.1A.3 Frequency error for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.4 Frequency error for CA (3UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.5 Frequency error for CA (4UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 for each antenna DL signal level same as 6.5.1 |  |
| 6.5.1D.1 Frequency Error for ProSe Direct Discovery | ±15 Hz Sidelink Signal level: ±0.7 dB, f ≤ 3.0GHz |  |
| 6.5.1D.2 Frequency Error for ProSe Direct Communication | Same as 6.5.1D.1 |  |
| 6.5.1E Frequency Error for UE category 0 | Same as 6.5.1 |  |
| 6.5.1EA Frequency Error for UE category M1 | Same as 6.5.1 |  |
| 6.5.1EB Frequency Error for UE category 1bis | Same as 6.5.1 |  |
| 6.5.1EC Frequency Error for UE category M2 | Same as 6.5.1 |  |
| 6.5.1EC_1 Frequency Error for UE category M2 (CE Mode B) | Same as 6.5.1 |  |
| 6.5.1F Frequency Error for category NB1 | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz |  |
| 6.5.1G.1 Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission | ±36 Hz, f > 3.0GHz |  |
| 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±15 Hz, f ≤ 3.0GHz ±36 Hz, 4.2GHz <f ≤ 6.0GHz |  |
| 6.5.2.1 Error Vector Magnitude | PUSCH: ± 2.5% PUCCH: ± 2.5% PRACH: ± 2.5% |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_1 EVM for CA (inter-band DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_1 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.5 Error Vector Magnitude (EVM) for CA (4UL CA) | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 for each antenna |  |
| 6.5.2.1A PUSCH-EVM with exclusion period | PUSCH : ± 2.5% |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 | Same as 6.5.2.1A |  |
| 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 | Same as 6.5.2.1 |  |
| 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 | Same as 6.5.2.1A |  |
| 6.5.2.1EB Error Vector Magnitude (EVM) for UE category 1bis | Same as 6.5.2.1 |  |
| 6.5.2.1EC.1 Error Vector Magnitude for UE category M2 | Same as 6.5.2.1 |  |
| 6.5.2.1EC.2 PUSCH-EVM with exclusion period for UE category M1 | Same as 6.5.2.1A |  |
| 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 | Same as 6.5.2.1 |  |
| 6.5.2.1G.1 Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.1 |  |
| 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ± 2.5% |  |
| 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation | ± 2.5% |  |
| 6.5.2.1_s Error Vector Magnitude (EVM) for subslot/slot TTI | Same as 6.5.2 |  |
| 6.5.2.2 Carrier leakage | ±0.8dB |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.4 Carrier leakage for CA (3UL CA) | ±0.8dB |  |
| 6.5.2A.2.5 Carrier leakage for CA (4UL CA) | ±0.8dB |  |
| 6.5.2.3 In-band emissions for non allocated RB | ±0.8dB |  |
| 6.5.2.4 EVM equalizer Spectrum flatness | ±1.4dB |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.5 In-band emissions for non allocated RB for CA (4UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 for each antenna |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 for each antenna |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 for each antenna |  |
| 6.5.2.2E Carrier Leakage for UE category 0 | Same as 6.5.2.2 |  |
| 6.5.2.2EA Carrier Leakage for UE category M1 | Same as 6.5.2.2 |  |
| 6.5.2.2EB Carrier leakage for UE category 1bis | Same as 6.5.2.2 |  |
| 6.5.2.2EC Carrier leakage for UE category M2 | Same as 6.5.2.2 |  |
| 6.5.2.2F Carrier leakage for UE category NB1 | Same as 6.5.2.2 |  |
| 6.5.2.2G.1 Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.2 |  |
| 6.5.2.2G.2 Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.5.2.2 |  |
| 6.5.2.2G.3 Carrier leakage for V2X Communication / Intra-band contiguous MCC operation | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 | Same as 6.5.2.3 |  |
| 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 | Same as 6.5.2.3 |  |
| 6.5.2.3EB In-band emissions for non allocated RB for UE category 1bis | Same as 6.5.2.3 |  |
| 6.5.2.3EC In-band emissions for non allocated RB for UE category M2 | Same as 6.5.2.3 |  |
| 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 | Same as 6.5.2.3 |  |
| 6.5.2.3G.1 In-band emissions for non-allocated RB for V2X Communication | Emission: ±0.8dB UE transmitted power ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm |  |
| 6.5.2.3G.2 Introduction of In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | same as 6.5.2.3G.1 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |
| 6.5.2.3G.3 In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation | same as 6.5.2.3G.1 for cc of E-UTRA V2X sidelink |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 | Same as 6.5.2.4 |  |
| 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 | Same as 6.5.2.4 |  |
| 6.5.2.4EB EVM equalizer spectrum flatness for UE category 1bis | Same as 6.5.2.4 |  |
| 6.5.2.4EC EVM equalizer spectrum flatness for UE category M2 | Same as 6.5.2.4 |  |
| 6.5.2.4G.1 EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.4 |  |
| 6.5.2.4G.3 EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.5.2.4 for each cc |  |
| 6.6.1 Occupied bandwidth | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 for each CC |  |
| 6.6.1A.3 Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.5 Occupied bandwidth for CA (4UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz 40MHz < f ≤ 60MHz: [800kHz] 60MHz < f ≤ 80MHz: [1000kHz] for intra-band contiguous CA Same as 6.6.1 for each CC for inter-band CA |  |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 for each antenna |  |
| 6.6.1E Occupied bandwidth for UE category 0 | Same as 6.6.1 |  |
| 6.6.1EA Occupied bandwidth for UE category M1 | Same as 6.6.1 |  |
| 6.6.1EB Occupied bandwidth for UE category 1bis | Same as 6.6.1 |  |
| 6.6.1EC Occupied bandwidth for UE category M2 | Same as 6.6.1 |  |
| 6.6.1G.1 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.6.1 |  |
| 6.6.1G.2 Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission | Same as 6.6.1 |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.1 |  |
| 6.6.1F Occupied bandwidth for UE category NB1 | 200kHz: FFS |  |
| 6.6.2.1 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 and 6.6.2.2 |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.5 Additional Spectrum Emission Mask for CA (4UL) | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 for each antenna |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 | Same as 6.6.2.1 |  |
| 6.6.2.1EA Spectrum Emission Mask for UE category M1 | Same as 6.6.2.1 |  |
| 6.6.2.1EB Spectrum Emission Mask for UE category 1bis | Same as 6.6.2.1 |  |
| 6.6.2.1EC Spectrum Emission Mask for UE category M2 | Same as 6.6.2.1 |  |
| 6.6.2.1F Spectrum Emission Mask for UE category NB1 | Same as 6.6.2.1 |  |
| 6.6.2.1G.1 Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.6.2.1G.2 Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions | same as 6.6.2.1G.1 for V2X Sidelink same as 6.6.2.1G.1 for E-UTRA Uplink |  |
| 6.6.2.1G.3 Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.2.1G.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_1 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.3 Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2A.5 Additional Spectrum Emission Mask for CA (4UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 | Same as 6.6.2.2 |  |
| 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 | Same as 6.6.2.2 |  |
| 6.6.2.2EB Additional Spectrum Emission Mask for UE category 1bis | Same as 6.6.2.2 |  |
| 6.6.2.2EC Additional Spectrum Emission Mask for UE category M2 | Same as 6.6.2.2 |  |
| 6.6.2.2G.1 Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.6.2.2 |  |
| 6.6.2.2G.2 Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | Same as 6.6.2.2 for V2X Sidelink Same as 6.6.2.2 E-UTRA uplink |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | ±0.8 dB |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | ±0.8 dB |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_5 Adjacent Channel Leakage power Ratio for UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_6 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.1_2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) | ±0.8 dB ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.6.2.3A.2_1 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.2_2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_1 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_2 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.5 Adjacent Channel Leakage power Ratio for CA (4UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 for each antenna |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3EA Adjacent Channel Leakage power Ratio for UE category M1 | Same as 6.6.2.3 |  |
| 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3F Adjacent Channel Leakage power Ratio for UE category NB1 | ±0.8 dB |  |
| 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±0.8 dB |  |
| 6.6.2.3G. 2 Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.6.2.3G.1 |  |
| 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.2.3G.1 |  |
| 6.6.2.4 Void | Void |  |
| 6.6.3.1 Transmitter Spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |
| 6.6.3.1_1 Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (3UL CA) (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (3UL CA) (inter-band 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.5 Transmitter Spurious emissions for CA (4UL CA) | Same as 6.6.3.1 |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1, at each antenna used for transmission | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 | Same as 6.6.3.1 |  |
| 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 | Same as 6.6.3.1 |  |
| 6.6.3F.1 Transmitter Spurious emissions for category NB1 and NB2 | Same as 6.6.3.1 |  |
| 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26GHz: ± 6.0 dB |  |
| 6.6.3G.1_1 Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.6.3G.1 |  |

## <<< END OF CHANGES >>>