3GPP TSG-RAN5 Meeting #102	R5-240555

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0330 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of clause 5.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Depending on the implementation the UE may start service authorisation either at SIP registration or using the SIP PUBLISH for MCX service settings There is no optional third SIP REGISTER. - Conditions for MCPTT-Info, MCVideo-Info and MCData-Info in the Message-Body of a SIP REGISTER are not specified. - Conditions for MCPTT-Info, MCVideo-Info and MCData-Info in the Message-Body of a SIP PUBLISH are not fully specified. - References to SIP NOTIFY have been wrong in Table 5.3.22.4-6 - In Table 5.3.2.3-1 at step 11, Table 5.3.2.3-2 at second EXCEPTION, Table 5.3.2.3-2A at step 1 and Table 5.3.2.3-2B at step 1 there have been notes regarding different stages during MCX registration which seem to be inconstent or wrong. In addition it is not clear what these notes are good for. - The way how the Group document subscription and notification and the Group communication key retrieval is described deviates from the TTCN implementation what may cause confusion. - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MCX registration restructured to correct the optional use of the SIP registration for service authorisation - Specific message contents for MCPTT-Info, MCVideo-Info and MCData-Info added for SIP REGISTER - Specific message contents for MCPTT-Info, MCVideo-Info and MCData-Info added for SIP PUBLISH - References to SIP NOTIFY corrected in new Table 5.3.2.7.2-7 - Notes regarding different stages during MCX registration removed - Group document subscription and Group communication key retrieval aligned with the TTCN implementation - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0334, 0335, 0336, 0339 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.2	Initial MCX Authentication, Registration, Configuration and Subscription

##### 5.3.2.1	Initial conditions

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

System Simulator:

-	SS (MCX server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [6] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCX operation in the MCX configuration document).

Implementation Under Test (IUT):

-	UE (MCX client)

-	The MCX Client has been provisioned either with the address information of the server from which the client can retrieve the MCX UE initial configuration document (steps 1a1-1a2 of procedure ‘MCX Initial Configuration and User Authentication', Table 5.3.2.3.1-1) or directly with the Initial UE Configuration Data as specified in Table 5.5.8.1-1.

-	According to TS 33.180 [94] all HTTP connections are secured by TLS.
The HTTP-1 interface authentication between the HTTP client in the MC UE and the HTTP server endpoint (HTTP proxy, IdM server or KMS) shall be performed by one-way authentication of the HTTP server endpoint based on server certificate as described in TS 33.180 [94] clause 6.1.1.

-	The UE User is provided with username/password for user authentication (px_MCX_User_A_username, px_MCX_User_A_password as provided in TS 36.579-5 [5], Table 9.2-1: MCX Client Common PIXIT)

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	The UE is attached to EPS services.

-	The UE is provisioned with the names and values of the Transport Key (TrK) and the Integrity Key (InK), since the KMS shall encrypt the key material sent to the client with the TrK and sign the response with the TrK or the InK according to TS 33.180 [94].

##### 5.3.2.2	Main Procedure

5.3.2.2.1	Procedure

Table 5.3.2.2.1-1: Initial MCX Authentication, Registration, Configuration and Subscription

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: The procedures of steps 1 and 2 happen in parallel | - | - |
| 1 | The UE (MCX client) performs procedure ‘MCX Initial Configuration and User Authentication’ as described in Table 5.3.2.3.1-1 | - | - |
| 2 | The UE (MCX client) performs procedure ‘SIP registration’ as described in Table 5.3.2.4.1-1 (NOTE 1) | - | - |
| - | EXCEPTION: The procedures of steps 3, 4 and 5 happen in parallel | - | - |
| 3 | The UE (MCX client) performs procedure ‘Publication of MCX service settings‘ as described in Table 5.3.2.5.1-1 (NOTE 1) | - | - |
| 4 | The UE (MCX client) performs procedure ‘Configuration management subscription‘ as described in Table 5.3.2.6.1-1 | - | - |
| 5 | The UE (MCX client) performs procedure ‘Group management subscription with optional GMK retrieval‘ as described in Table 5.3.2.7.1-1 | - | - |
| 6 | The SS (MCX server) sends a SIP MESSAGE for configuration of Location Info reporting. | <-- | SIP MESSAGE |
| 7 | The UE (MCX client) responds with SIP 200 (OK) | --> | SIP 200 (OK) |
| NOTE 1: Based on UE implementation, the access token may be provided using either a SIP REGISTER at initial SIP registration (Table 5.3.2.4.1-1 step 3a1) or in the SIP PUBLISH for MCPTT server settings (Table 5.3.2.5.1-1 step 1b1); the SIP REGISTER can only be used when the access token is already available. |  |  |  |

5.3.2.2.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.2.2-1: SIP MESSAGE (step 6, Table 5.3.2.2.1-1)

| Derivation Path: Table 5.5.2.7.2-1, condition LOCATION_CONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.2.2-2 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.2.2-3 |  |  | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.2.2-4 |  |  | MCDATA |

Table 5.3.2.2.2-2: MCPTT Info in SIP MESSAGE (Table 5.3.2.2.2-1)

| Derivation Path: Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |

Table 5.3.2.2.2-3: MCVideo Info in SIP MESSAGE (Table 5.3.2.2.2-1)

| Derivation Path: Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-calling-user-id | not present |  |  |  |

Table 5.3.2.2.2-4: MCData Info in SIP MESSAGE (Table 5.3.2.2.2-1)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |

##### 5.3.2.3	MCX Initial Configuration and User Authentication

5.3.2.3.1	Procedure

Table 5.3.2.3.1-1: MCX Initial Configuration and User Authentication

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1a2 describe behaviour that depends on UE implementation. | - | - | - | - |
| 1a1 | IF the UE (MCX client) is capable of downloading the MCX UE initial configuration document THEN the UE (MCX client) sends an HTTP GET Request to retrieve the initial UE configuration from the server. NOTE: Otherwise the UE needs to be preconfigured with the Initial UE Configuration Data as initial condition. | --> | HTTP GET (initial UE configuration) | - | P |
| 1a2 | The SS sends an HTTP 200 (OK) including the initial UE configuration document | <-- | HTTP 200 (OK) | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: The messages in steps 3a1-7 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70], to the authorisation endpoint of the IdM server as specified in 3GPP TS 33.180 [94] using the configured URL of the authorisation endpoint of the IdM server as specified in the "<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| - | EXCEPTION: Steps 3a1-3b1 describe behaviour that depends on UE implementation of the OpenID Connect protocol; the UE may either use an HTTP GET or an HTTP POST to send the OpenID Connect Authentication Request. | - | - | - | - |
| 3a1 | The UE (MCX client) sends an OpenID Connect Authentication Request using HTTP GET. | --> | HTTP GET (Authorization) | - | P |
| 3b1 | The UE (MCX client) sends an OpenID Connect Authentication Request using HTTP POST. | --> | HTTP POST (Authorization) | - | P |
| 4 | The SS sends an HTTP 200 (OK) including the HTML form requesting username and password. | <-- | HTTP 200 (OK) | - | - |
| 5 | Provide the UE (MCX client) with user credentials: username and password (px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 6 | The UE (MCX client) sends an HTTP POST Request containing user name and password. | --> | HTTP POST | - | P |
| 7 | The SS sends a HTTP 302 (Found) as the OpenID Connect Authentication Response containing an authorization code. | <-- | HTTP 302 (Found) | - | - |
| 8 | Void | - | - | - | - |
| - | EXCEPTION: The messages in steps 9-10 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70] to the token endpoint of the IdM server as specified in 3GPP TS 33.180 [94] using the configured URL of the token endpoint of the IdM server as specified in the "/<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| 9 | The UE (MCX client) sends an HTTP POST Request (OIDC Token Request), passing the authorization code obtained in step 7. | --> | HTTP POST | - | P |
| 10 | The SS sends an HTTP 200 (OK) providing id_token, access_token and refresh token. | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: The messages in steps 11-14 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70] to the HTTP Proxy as specified in 3GPP TS 33.180 [94] using the configured URL of the HTTP Proxy as specified in the "/<x>/OnNetwork/AppServerInfo/HTTPproxy" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| 11 | The UE (MCX client) sends an HTTP POST presenting the access token obtained in step 10. | --> | HTTP POST | - | P |
| 12 | The SS replies with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| 13 | The UE (MCX client) sends an HTTP POST presenting an access token for Key Material Request. | --> | HTTP POST | - | P |
| 14 | The SS replies to the UE with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| NOTE 1: The UE is expected to prompt the MCX user for username and password, or it may be stored on the UE. The provision of the username/password is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.2.3.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.3.2-1: HTTP GET (Step 1, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.2-1, condition UEINITIALCONFIG

Table 5.3.2.3.2-2: HTTP 200 (OK) (Step 2, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition UEINITIALCONFIG

Table 5.3.2.3.2-3: HTTP GET (Step 3a1, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.2-1, condition AUTH

Table 5.3.2.3.2-4: HTTP POST (Step 3b1, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition AUTH

Table 5.3.2.3.2-5: HTTP 200 (OK) (Step 4, Table 5.3.2.3.1-1)

| Derivation Path: Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "text/html" |  | RFC 2854 [111] |  |
| Message-body |  |  |  |  |
| HTML form | <!DOCTYPE html> <html> <body> <form action="/idms/userauth" method="post"> Username: <input type="text" name="user"><br> Password: <input type="password" name="password"><button type="submit">Login</button> </form> </body> </html> | "/idms/userauth" given by tsc_MCX_IdMS_userauth_UriPath is the URI to be used by the UE as request URI in the HTTP POST request for user authentication | HTML 4.01 Specification [105] |  |

Table 5.3.2.3.2-6: HTTP POST (Step 6, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition USERAUTH

Table 5.3.2.3.2-7: HTTP 302 (Found) (Step 7, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.8-1, condition AUTH.

Table 5.3.2.3.2-8: HTTP POST (Step 9, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition TOKEN

Table 5.3.2.3.2-9: HTTP 200 (OK) (Step 10, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition TOKEN

Table 5.3.2.3.2-10: HTTP POST (Step 11, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.33-1, condition KMSINIT.

Table 5.3.2.3.2-11: HTTP 200 (OK) (Step 12, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition KMSINIT.

Table 5.3.2.3.2-12: HTTP POST (Step 13, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.3-1, condition KMSKEY.

Table 5.3.2.3.2-13: HTTP 200 (OK) (Step 14, Table 5.3.2.3.1-1)

Derivation Path: Table 5.5.4.6-1, condition KMSKEY.

##### 5.3.2.4	SIP Registration

5.3.2.4.1	Procedure

Table 5.3.2.4.1-1: SIP Registration

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE sends an initial registration for IMS services. | --> | SIP REGISTER |
| 2 | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. | <-- | SIP 401 Unauthorized |
| - | EXCEPTION: The UE completes the security negotiation procedures, sets up a temporary set of SAs and uses those for sending another SIP REGISTER with AKAv1-MD5 credentials at step 3a1 or 3a2 | - | - |
| - | EXCEPTION: Steps 3a1-3b1 describe behaviour that depends on UE implementation and on availability of an access-token (NOTE 1) | - | - |
| 3a1 | IF the client has retrieved the access token already at MCX User Authentication (Table 5.3.2.3.1-1 steps 9-10) THEN the UE may use the SIP REGISTER to provide access token and CSK for service authorisation (NOTE 2) | --> | SIP REGISTER (access token, CSK) |
| 3b1 | ELSE the UE sends SIP REGISTER without access token and CSK | --> | SIP REGISTER |
| 4 | The SS responds with 200 OK. | <-- | SIP 200 OK |
| NOTE 1: According to TS 33.180 [94], clause 5.1.3.2.1 sending of the SIP REGISTER shall not be delayed for lack of an access token If the client does not have the access token yet, the client shall sent the SIP REGISTER without service authorisation and shall provide the access token in the SIP PUBLISH (Table 5.3.2.5.1-1, step 1b1) NOTE 2: As the MCPTT/MCVideo/MCData Info containing the access token is security protected the client also needs to provide the CSK used for cyphering and integrity protection. |  |  |  |

5.3.2.4.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.4.2-1: SIP REGISTER (Step 1, Table 5.3.2.4.1-1)

Derivation Path: Table 5.5.2.13-1, condition SIP_REGISTER_INITIAL

Table 5.3.2.4.2-2: SIP REGISTER (Step 3a1, Table 5.3.2.4.1-1)

| Derivation Path: Table 5.5.2.13-1, condition SERVICE_AUTH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.4.2-3 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.4.2-4 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.4.2-5 |  | TS 24.282 [87] clause D.1 | MCDATA |

Table 5.3.2.4.2-3: MCPTT-Info in SIP REGISTER (Table 5.3.2.4.2-2)

Derivation Path: Table 5.5.3.2.1-1, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-4: MCVideo-Info in SIP REGISTER (Table 5.3.2.4.2-2)

Derivation Path: Table 5.5.3.2.1-2, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-5: MCData-Info in SIP REGISTER (Table 5.3.2.4.2-2)

Derivation Path: Table 5.5.3.2.1-3, condition CONFIG, REGISTER

##### 5.3.2.5	Publication of MCX service settings

5.3.2.5.1	Procedure

Table 5.3.2.5.1-1: Publication of MCX service settings

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: Steps 1a1-1b1 describe behaviour that depends on whether or not the client has provided an access token for service authorisation already at SIP registration (Table 5.3.2.4.1-1) |  |  |  |  |
| 1a1 | IF the UE (MCX client) has provided the access token at SIP registration THEN the UE (MCX client) sends a SIP PUBLISH request for update of PoC-settings only. (NOTE 1). | --> | SIP PUBLISH | - | P |
| 1b1 | ELSE the UE (MCX client) sends a SIP PUBLISH request for service authorisation and update of PoC-settings. (NOTE 1). | --> | SIP PUBLISH | - | P |
| 2 | The SS (MCX server) sends SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| NOTE 1: The PoC-settings document contains the user profile index of the selected user profile. In general the UE sends the SIP PUBLISH request not before it has retrieved the user profile at step 8 in Table 5.3.2.6.1-1. |  |  |  |  |  |

5.3.2.5.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.5.2-1: SIP PUBLISH (Step 1a1, Table 5.3.2.5.1-1)

| Derivation Path: Table 5.5.2.11-1, condition POC-SETTINGS-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.5.2-3 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.5.2-4 |  |  | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.5.2-5 |  |  | MCDATA |

Table 5.3.2.5.2-2: SIP PUBLISH (Step 1a2, Table 5.3.2.5.1-1)

| Derivation Path: Table 5.5.2.11-1, condition POC-SETTINGS-EVENT, SERVICE_AUTH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.2.5.2-3 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.3.2.5.2-4 |  |  | MCVIDEO |
|  | MCData-Info as described in Table 5.3.2.5.2-5 |  |  | MCDATA |

Table 5.3.2.5.2-3: MCPTT-Info in SIP PUBLISH (Table 5.3.2.5.2-1/2)

Derivation Path: Table 5.5.3.2.1-1, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-4: MCVideo-Info in SIP PUBLISH (Table 5.3.2.5.2-1/2)

Derivation Path: Table 5.5.3.2.1-2, condition CONFIG, REGISTER_PUBLISH

Table 5.3.2.4.2-5: MCData-Info in SIP PUBLISH (Table 5.3.2.5.2-1/2)

Derivation Path: Table 5.5.3.2.1-3, condition CONFIG, REGISTER

##### 5.3.2.6	Configuration management subscription

5.3.2.6.1	Procedure

Table 5.3.2.6.1-1: Configuration management subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE (MCX client) sends a SIP SUBSCRIBE - subscription to multiple documents simultaneously - containing the access token and a resource list body containing a list adressing the following documents: MCX UE Configuration document, MCX User Profile Configuration Document, and the MCX Service configuration document. The base URI of each list entry is set to the CMS XCAP-ROOT-URI. | --> | SIP SUBSCRIBE | - | P |
| 2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 3 | The SS sends a SIP NOTIFY containing the XCAP-URIs of the documents. | <-- | SIP NOTIFY | - | - |
| - | EXCEPTION: The order of steps 4, 5, 7 and 9 depends on UE and SS implementation and is not checked by the implementation | - | - | - | - |
| 4 | The UE (MCX client) sends a SIP 200 (OK). | --> | SIP 200 (OK) | - | P |
| 5 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the MCX UE Configuration Document. NOTE: The MCX Client is requesting the MCX UE Configuration Document. | --> | HTTP GET | - | P |
| 6 | The SS sends an HTTP 200 (OK) including the MCX UE Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| 7 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the MCX User Profile Configuration Document. NOTE: The MCX Client is requesting the MCX User Profile Configuration Document. | --> | HTTP GET | - | P |
| 8 | The SS sends an HTTP 200 (OK) including the MCX User Profile Configuration Document. NOTE: The MCX User Profile Configuration Document includes information on MCX groups including for which groups the MCX Client is a member. The MCX User Profile Configuration Document includes Group A as a group for which the MCX Client is a member and is implicitly affiliated. Group A is used as the default group for all test cases in TS 36.579-2 and TS 36.579-3. | <-- | HTTP 200 (OK) | - | - |
| 9 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the MCX Service Configuration Document. NOTE: The MCX Client is requesting the MCX Service Configuration Document. | --> | HTTP GET | - | P |
| 10 | The SS sends an HTTP 200 (OK) including the MCX Service Configuration Document. | <-- | HTTP 200 (OK) | - | -- |

5.3.2.6.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.6.2-1: SIP SUBSCRIBE (Step 1, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.2.14-1, condition CONFIG

Table 5.3.2.6.2-2: SIP NOTIFY (Step 3, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.2.8-1, condition CONFIG

Table 5.3.2.6.2-3: HTTP GET (Step 5, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.2-1, condition UECONFIG.

Table 5.3.2.6.2-4: HTTP GET (Step 7, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.2-1, condition UEUSERPROF.

Table 5.3.2.6.2-5: HTTP GET (Step 9, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.2-1, condition UESERVCONFIG.

Table 5.3.2.6.2-6: HTTP 200 (OK) (Step 6, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.6-1, condition UECONFIG.

Table 5.3.2.6.2-7: HTTP 200 (OK) (Step 8, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF.

Table 5.3.2.6.2-8: HTTP 200 (OK) (Step 10, Table 5.3.2.6.1-1)

Derivation Path: Table 5.5.4.6-1, condition UESERVCONFIG.

##### 5.3.2.7	Group management subscription with optional GMK retrieval

5.3.2.7.1	Procedure

Table 5.3.2.7.1-1: Group management subscription with optional GMK retrieval

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1-1c4 describe behaviour that depends on UE implementation; the UE may either use a single SIP SUBSCRIBE for subscription to group A and the GKTP or it uses separate SIP SUBSCRIBE requests. |  |  |  |  |
| 1a1 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with an entry for subscription to the group configuration document (group A) and an entry for subscription to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | --> | SIP SUBSCRIBE (group A, GKTP) | - | P |
| 1a2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 1b1 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with a single entry for subscription to the group configuration document (group A). | --> | SIP SUBSCRIBE (group A) | - | - |
| 1b2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 1c1 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with a single entry for subscription to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | --> | SIP SUBSCRIBE (GKTP) | - | - |
| 1c2 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 1c3 | The UE (MCX client) sends a SIP SUBSCRIBE containing a resource-lists body with a single entry for subscription to the group configuration document (group A). | --> | SIP SUBSCRIBE (group A) | - | - |
| 1c4 | The SS sends a SIP 200 (OK). | <-- | SIP 200 (OK) | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: IF and only if the UE has performed steps 1b1-1b2 THEN in parallel to the events described in steps 3-6, the behaviour of Table 5.3.2.7.1-2 happens: The UE (MCX client) optionally subscribes to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | - | - | - | - |
| 3 | The SS sends a SIP NOTIFY containing the XCAP-URI of the Group Configuration document for group A. | <-- | SIP NOTIFY (group A) | - | - |
| - | EXCEPTION: The order of steps 4 and 5 depends on UE and SS implementation and is not checked. | - | - | - | - |
| 4 | The UE (MCX client) sends a SIP 200 (OK). | --> | SIP 200 (OK) | - | P |
| 5 | The UE (MCX client) sends an HTTP GET Request containing the access token and the XCAP-URI of the Group Configuration document. | --> | HTTP GET (group A) | - | P |
| 6 | The SS sends an HTTP 200 (OK) containing the Group Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: Steps 7a1-7a2 describe behaviour that depends on whether the UE has requested a GMK at step 1a1, step 1c1 or at step 2a1 of the parallel behaviour in Table 5.3.2.7.1-2 | - | - | - | - |
| 7a1 | IF the UE has requested a GMK THEN the SS sends a SIP NOTIFY containing the group key transport payloads (GKTP) document with the GMK. | <-- | SIP NOTIFY (GKTP) | - | - |
| 7a2 | The UE (MCX client) sends a SIP 200 (OK). | --> | SIP 200 (OK) | - | P |

Table 5.3.2.7.1-2: Stand-alone group communication key request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts timer Timer_1 = 5 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 2a1-2b1 describe behaviour that depends on UE implementation; in general the group communication key retrieval is optional at initial registration. (NOTE 1) | - | - | - | - |
| 2a1 | The UE (MCX client) sends a SIP SUBSCRIBE creating a new dialog and containing a resource list body containing a single entry for subscription to the MCS GKTP document for Group communication key retrieval (GMK retrieval). | --> | SIP SUBSCRIBE (GKTP) | - | P |
| 2a2 | The SS sends a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 2a3 | The SS stops Timer_1. | - | - | - | - |
| 2b1 | Timer_1 expires | - | - | - | - |
| NOTE 1: The key retrieval from the GMS is necessary for the MCX UE under test to enable ciphering exchanged media in group communications. |  |  |  |  |  |

5.3.2.7.2	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

Table 5.3.2.7.2-1: SIP SUBSCRIBE (Step 1a1, Table 5.3.2.7.1-1)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.7.2-2 |  |  |  |

Table 5.3.2.7.2-2: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.7.2-1)

Derivation Path: Table 5.5.3.3.1A-1, condition GROUPCONFIG, GROUPKEY

Table 5.3.2.7.2-3: SIP SUBSCRIBE (Step 1b1, Table 5.3.2.7.1-1; step 1c3, Table 5.3.2.7.1-1)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.7.2-4 |  |  |  |

Table 5.3.2.7.2-4: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.7.2-3)

Derivation Path: Table 5.5.3.3.1A-1, condition GROUPCONFIG

Table 5.3.2.7.2-5: SIP SUBSCRIBE (Step 1c1, Table 5.3.2.7.1-1; step 2a1, Table 5.3.2.7.1-2)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.7.2-6 |  |  |  |

Table 5.3.2.7.2-6: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.7.2-5)

Derivation Path: Table 5.5.3.3.1A-1, condition GROUPKEY

Table 5.3.2.7.2-7: SIP NOTIFY (Step 3, Table 5.3.2.7.1-1)

Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG

Table 5.3.2.7.2-8: HTTP GET (Step 5, Table 5.3.2.7.1-1)

Derivation Path: Table 5.5.4.2-1, condition GROUPCONFIG

Table 5.3.2.7.2-9: HTTP 200 (OK) (Step 6, Table 5.3.2.7.1-1)

Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG.

Table 5.3.2.7.2-10: SIP NOTIFY (Step 7a1, Table 5.3.2.7.1-1)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.2.7.2-11 |  |  |  |

Table 5.3.2.7.2-11: Xcap-Diff Document (Table 5.3.2.7.2-10)

Derivation Path: Table 5.5.3.12-2, condition GROUPKEY