3GPP TSG-RAN5 Meeting #102	R5-240564

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0339 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of clause 5.5.3.3.1A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Use of conditions GROUPCONFIG and GROUPKEY is not precise |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use of conditions GROUPCONFIG and GROUPKEY changed to make clear in which case there are two entries in the resource-lists |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.5.3.3.1A	Resource-lists from the UE for initial configuration

Table 5.5.3.3.1A-1: Resource-lists from the UE for initial configuration

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | encrypted (NOTE 1) |  | TS 24.481 [11] TS 24.484 [14] |  |
| list[1] | encrypted (NOTE 1) |  |  |  |
| name attribute | not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 1, 2 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID-ue-config & “/users/” & XUID & “/” & MCSUEID & “/” | UE Configuration document (NOTE 3) |  |  |
|  | AUID-ue-config & “/users/” & XUID & “/” | Editor’s note: It is not clear in the core specs whether both options are allowed or only one of both; if the UE is allowed not to include the MCSUEID, it is not clear where the MC server gets it from |  |  |
| display-name | Not present |  |  |  |
| entry[2] | NOTE 1, 2 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID-user-profile & “/users/” & XUID & “/” | UE User Profile document (NOTE 3) |  |  |
| display-name | Not present |  |  |  |
| entry[3] | NOTE 1, 2 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID-service-config & “/global/service-config.xml” | UE Service Configuration document (NOTE 3) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 1, 2 |  | TS 24.484 [14] | GROUPCONFIG |
| uri attribute | “org.openmobilealliance.groups/global/byGroupID/” & Group-ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[2] | NOTE 1, 2 |  | TS 24.481 [11] | GROUPCONFIG AND GROUPKEY |
| uri attribute | Doc-Sel_T & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 3) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 1, 2 |  | TS 24.481 [11] | GROUPKEY AND NOT GROUPCONFIG |
| uri attribute | Doc-Sel & “~~” & Node-Sel | MCPTT-GKTP document (NOTE 3) |  |  |
| display-name | Not present |  |  |  |
| NOTE 1: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry’s uri attribute as described in Table 5.5.13.3-1 NOTE 2: When a resource-lists document contains more than one entry, the entries may be in any order. NOTE 3: The terms AUID-ue-config, AUID-user-profile, AUID-service-config, XUID, Group-ID, Doc-Sel, Node-Sel and MCSUEID are defined in table 5.5.3.3.1A-2. |  |  |  |  |

Table 5.5.3.3.1A-2: Terms used in Resource-lists' URIs

| Term | Value | Condition |
| --- | --- | --- |
| AUID-ue-config | "org.3gpp.mcptt.ue-config" | MCPTT |
|  | "org.3gpp.mcvideo.ue-config" | MCVideo |
|  | "org.3gpp.mcdata.ue-config" | MCData |
| AUID-user-profile | "org.3gpp.mcptt.user-profile" | MCPTT |
|  | "org.3gpp.mcvideo.user-profile" | MCVideo |
|  | "org.3gpp.mcdata.user-profile" | MCData |
| AUID-service-config | "org.3gpp.mcptt.service-config" | MCPTT |
|  | "org.3gpp.mcvideo.service-config" | MCVideo |
|  | "org.3gpp.mcdata.service-config" | MCData |
| XUID | "sip:" & px_MCPTT_ID_User_A | MCPTT |
|  | "sip:" & px_MCVideo_ID_User_A | MCVideo |
|  | "sip:" & px_MCData_ID_User_A | MCData |
| Group-ID | px_MCPTT_Group_A_ID | MCPTT |
|  | px_MCVideo_Group_A_ID | MCVideo |
|  | px_MCData_Group_A_ID | MCData |
| Doc-Sel | "org.3gpp.MCPTT-GKTP/global/byGroupID/" & Group-ID & "/" |  |
| Node-Sel | "/group/list-service/mgktp:GKTPs?xmlns(mgktp=urn:3gpp:ns:mcpttGKTP:1.0)" |  |
| MCSUEID | Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) |  |