3GPP TSG RAN meeting #103										RP-240368

Maastricht, Netherlands, March 18-21, 2024

Status Report to TSG

Agenda item:			9.4.1.4

| WI / SI Name | NR MIMO evolution for downlink and uplink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_evo_DL_UL |  |  |  |  |
| Unique ID | 940096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-233028 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 12/2023 | Performance part: 06/2024 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 50% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Yubo Yang |
|  | Company | Huawei |
|  | Email | yangyubo1@huawei.com |
| Rapporteur | Name | Riki Okawa |
|  | Company | NTT DOCOMO |
|  | Email | riki.ookawa.rp@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#110 Feb 26 – March 01 2024, Athens, Greece

The following agreements and conclusions were made in RAN4 #110:

- RRM related

WF on RRM requirements for NR_MIMO_evo_DL_UL was approved in R4-2403372.

Issue 3-1-1: Whether to define TDCP accuracy requirement/test case for TDCP?

< Agreement>:

- Define test cases for TDCP under certain conditions, and the conditions can be discussed in Issue 3-1-3.

Issue 3-1-2: Whether to define additional criteria for TDCP?

< Agreement>:

- Not to introduce additional L1-SINR or L1-RSRP reporting along with TDCP reporting in Rel-18.

Issue 3-1-3: Design for the test case of TDCP

< Agreement>:

  - Test metric:

    - Step 1: Determine the configuration (e.g., SNR, doppler spread) for the test.

    - Step 2: for the configuration agreed in step 1, determine the TDCP accuracy range (i.e., TDCP accuracy value X1 to X2) from the simulation results from different companies.

    - Step 3: Define the test case for the configurations agreed in step 1 and the TDCP value UE should report to pass the test is determined from the range of values determined in step 2 (i.e., X1 to X2). [For the repeated tests UE should pass the test at least [Y2] % of the times. Y2 can be FFS.]

      - The text in [] is the baseline, and other proposal is not precluded.

  - Test Parameters:

    - Channel model: fading channel

      - As baseline, TDL-A30

    - Pick one value from SNR = 10dB, 15dB, 20dB

    - The distance between two TRSs: 1 slot as baseline, further discuss on 10 slots

Issue 3-1-4: TDCP Measurement Report Mapping – amplitude

< Agreement>:

- Define TDCP mapping table of amplitude in section 10 of RRM spec.

Issue 3-1-5: TDCP Measurement Report Mapping - phase

< Agreement>:

- Define TDCP mapping table of phase in section 10 of RRM spec.

Issue 3-2-1: Whether to define TCs for two TAs?

< Agreement>:

- Define TC for two TAs

- FFS: both of UE uplink transmit timing and UE timing advance adjustment accuracy or just one of them.

Issue 3-2-3: Whether to define TCs for s-DCI mTRP cases?

< Agreement>:

- Define TCs for s-DCI mTRP cases:

    - Including separate DL TCI state switch and UL TCI state switch.

    - All are known TCI state

    - Joint TCI state switch. Note: further check of simultaneous reception of the TC.

    - Dual TCI state. Note: further check the testability

- Demod and CSI reporting related

WF on Demod and CSI reporting requirements for NR_MIMO_evo_DL_UL was approved in R4-2402868. Generally, the requirements scope of demod and CSI reporting for NR_MIMO_evo_DL_UL are stablished.

Issue 1-1-1: clarify if introduce PMI reporting requirements for ‘typeII-Doppler-r18’ codebook

Agreement:

- Introduce PMI reporting requirements for ‘typeII-Doppler-r18’ codebook.

Issue 1-1-2: clarify if introduce PMI reporting requirements for ‘typeII-CJT-r18’ codebook

Agreement:

- Introduce PMI reporting requirements for ‘typeII-CJT-r18’ codebook.

Issue 1-1-3: clarify criteria of feasibility for ‘typeII-Doppler-r18’ codebook

Agreement:

- Use the mechanism agreement which has already agreed in RAN4#109 and CSI-RS setup in section 2.2.1.

Issue 1-1-4: clarify test metric for PMI reporting requirements with ‘typeII-Doppler-r18’ codebook

Agreement:

- Test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler is X % of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook

Issue 1-1-5: clarify test metric for PMI reporting requirements with ‘typeII-CJT-r18’ codebook

Agreement:

- For the PMI reporting requirements with typeII-CJT-r18, define the test metric as γcjt=ttypeII-CJTtrnd, where ttypeII-CJT is [90] % of the maximum throughput obtained at SNRtypeII-CJT using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-CJT with random precoding based on type I Single Panel codebook per TRP.

Issue 1-1-6: clarify how to approach random PMI precoding in mTRP case for Test metric defined as γ3=ttypeII-CJTtrnd

Agreement:

- For each TRP, use random type I SP PMI precoding independently.

Issue 1-1-7: explicitly define for clarification what “equal probability of each applicable i1, i2 combination”

Agreement:

- Explicitly define random precoding frequency domain granularities as random i1 with wideband granularity and random i2 with subband granularity with ‘typeII-Doppler-r18’ and ‘typeII-CJT-r18’ codebook.

Issue 1-2-1: clarify the details of applicability rule for PUSCH Rel-18 DMRS ports

Agreement:

- Introduce applicability rule for UL Rel-18 DMRS port, but the details of identifying the table numbers can be discussed during CR drafting.

Issue 2-1-1: Propagation channel for TypeII-Doppler-r18 PMI reporting requirements

Agreement:

- TDLA30-30 as baseline. Interested companies can also provide results with 20 and 40 Hz in addition to 30 Hz.

Issue 2-1-3: MCS

Agreement:

- MCS13 (16QAM, 0.48) for PMI reporting requirements of typeII-Doppler-r18 codebook

Issue 2-1-7: Beam steering modelling for TypeII-Doppler-r18 PMI reporting requirements

Agreement:

- Dual cluster beam steering for TypeII-Doppler-r18 PMI reporting requirements setup.

Issue 2-2-1: Propagation channel and correlation configuration for TypeII-Doppler-r18 PMI reporting requirements

Agreement:

- Use TDLA30-10 with XP high as the propagation channel and correlation configuration for Rel-18 TypeII for CJT test.

Issue 2-2-3: paramCombination-CJT-L-r18 for TypeII-CJT-r18 PMI reporting requirements

Agreement:

- Set paramCombination-CJT-L-r18 as 7 ({4, 4})

Issue 2-2-4: paramCombination-CJT-r18 for TypeII-CJT-r18 PMI reporting requirements

Agreement:

- Set paramCombination-CJT-r18 as 4 (pν=14and18, β=12)

Issue 2-2-7: codebookMode for TypeII-CJT-r18 PMI reporting requirements

Agreement:

- Set codebookMode as Mode2 for PMI reporting requirements of typeII-CJT-r18 codebook

Issue 2-2-8: X% of the maximum throughput for TypeII-CJT-r18 PMI reporting requirements

Agreement:

- Use 90% of the maximum throughput in the test metric for PMI reporting requirements of typeII-CJT-r18 codebook

Issue 2-3-1: DMRS ports for PDSCH Rel-18 DMRS

Agreement:

- {1008, 1009} for Rank 2 test; {1000, 1001, 1008, 1009} for Rank 4 test

Issue 2-3-2: Duplex mode for tests need to be defined for PDSCH Rel-18 DMRS

Agreement:

- both FDD and TDD

Issue 2-3-3: Number of Rx for tests need to be defined for PDSCH Rel-18 DMRS

Agreement:

- both 2Rx and 4Rx

Issue 2-3-4: Cases need to be defined for FR1 PDSCH Rel-18 DMRS

Agreement:

- For Rank 2 with 2Rx, Test 2-1 in Chapter 5.2.2.1.1, 5.2.2.2.1

- For Rank 4 with 4Rx, Test 4-1 in Chapter 5.2.3.1.1, 5.2.3.2.1

Issue 2-3-5: Cases need to be defined for FR2-1 PDSCH Rel-18 DMRS

Agreement:

- No need to define cases for FR2-1 Rel-18 DMRS.

Issue 3-1-1: SCS & CBW for PUSCH Rel-18 DMRS

Agreement:

- 15KHz SCS, 5MHz; 30KHz SCS, 10MHz;

Issue 3-1-2: MCS for PUSCH Rel-18 DMRS

Agreement:

- MCS16

2.4.2	Remaining Open issues

- RRM Performance part:

- Whether to define TCs for m-DCI mTRP cases.

- Details of TDCP mapping tables.

- Details of RRM test cases.

- Demod and CSI reporting related

- Details of test setup and assumptions for TypeII-Doppler-r18 codebook requirements.

- Details of test setup and assumptions for TypeII-CJT-r18 codebook requirements.

- Details of test setup and assumptions for Rel-18 DMRS demodulation requirements.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template