3GPP TSG RAN Meeting #103	RP-240460

Maastricht, Netherlands, March 18-21, 2024

# Source:	Ericsson

# Title:	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

# Document for:	Approval

Agenda Item:	8.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

Acronym: FS_4400_15350MHz_NR

Unique identifier:

{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

{The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
|  | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
|  |  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

# 3	Justification

WRC-23 (20 November to 15 December 2023) approved the agenda for the 2027 world radio communication conference (Administrative Circular CA/270).

They invited ITU-R to conduct and complete in time for WRC-27 the sharing and compatibility studies, with a view to ensuring the protection of services to which the frequency band is allocated on a primary basis for the frequencies:

- 4400 to 4800 MHz

- 7125 to 8400 MHz

- 14800 to 15350 MHz

CPM 27-1 (Dec. 18-19, 2023) assigned ITU-R WP5D as the Responsible Group for this item (Agenda Item 1.2), WP 5D has recently sent an LS to 3GPP requesting information (R4-2400333), if available, already by June 2024 and final submission June 2025 if the studies are extended.

To make their compatibility studies, ITU-R WP5D requested 3GPP to provide the technical parameters for those frequencies for IMT.

Previous work e.g., SI 7-24 GHz in TR 38.820 and 6 to 10 GHz in TR 38.921 may be taken to account as needed.

For the frequency range 4400 to 4 800 MHz RF parameters can be extracted from specifications and BS antenna parameters can be extracted from the extended array antenna model applicable for 1710 to 4990 MHz.

RAN4 should assess if the parameters for 6 425 to 7 125 MHz are relevant for 7125 to 8400 MHz and determine the parameters for the 14800 to 15350 MHz frequency range, taking into account the feasibility aspect.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequency ranges:

- 4400 to 4800 MHz

- 7125 to 8400 MHz

- 14800 to 15350 MHz

The objectives are:

- To study and evaluate following parameters:

- Transmitter and receiver characteristics for the NR UE.

- Transmitter and receiver characteristics for the NR BS.

- Prepare answering requests from ITU-R WP 5D regarding NR in 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz.

- IMT technology related parameters (template as attached in Table 1)

- Beamforming antenna characteristics for IMT (template as attached in Table 2)

- Information on any other current or future feature of IMT systems that could be relevant for the sharing and compatibility studies with respect to other services, including e.g., deterministic calculations or Monte Carlo simulations.

- As soon as the parameters for 4400 to 4800 MHz, 7125 to 8400 MHz or 14800 to 15350 MHz are ready, they will be sent to ITU, without being dependent from each other as indicated in workplan in R4-2403653.

- Use the studied transmitter and receiver characteristics when future WI covering the concerned frequencies is initiated.

- For AAS BS use the antenna array antenna model, including the sub-array extension as base line for parameter delivery.

- Provide improved description of array antenna model parameters and indicate parameter dependencies and relation to deployment aspects.

- As part of additional information requested by ITU establish a model for BS array correlation factor () roll-off to describe how the array factor collapses moving away from the centre frequency.

-----------------------------

Template for response:

TABLE 1 example template

IMT-2030 specification related parameters in [xx-xx] MHz

| No. | Parameter | Base station (non-AAS) | Base station (AAS) | Mobile station |
| --- | --- | --- | --- | --- |
| 1 | Duplex Method |  |  |  |
| 2 | Channel bandwidth (MHz) |  |  |  |
| 3 | Signal bandwidth (MHz) |  |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) |  |  |  |
| 4.2 | Spectral mask (dB) |  |  |  |
| 4.3 | ACLR (dB) |  |  |  |
| 4.4 | Spurious emissions /out of band emissions |  |  |  |
| 4.5 | Maximum output power (dBm) |  |  |  |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) |  |  |  |
| 5.2 | Sensitivity (dBm) |  |  |  |
| 5.3 | Blocking response |  |  |  |
| 5.4 | ACS |  |  |  |
| 5.5 | SINR operating range (dB) |  |  |  |

TABLE 2 example template

Beamforming antenna characteristics for IMT-2030 in [xx-xx] MHz

|  |  | Rural macro (If it’s available) | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.1 | Antenna pattern |  |  |  |  |  |
| 1.2 | Element gain (dBi) |  |  |  |  |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |  |  |  |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |  |  |  |  |
| 1.5 | Antenna polarization |  |  |  |  |  |
| 1.6 | Antenna array configuration (Row × Column) |  |  |  |  |  |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv |  |  |  |  |  |
| 1.7a | Number of element rows in sub-array, Msub |  |  |  |  |  |
| 1.7b | Vertical radiating element spacing in sub-array, dv,sub |  |  |  |  |  |
| 1.7c | Pre-set sub-array down-tilt, θsubtilt (degrees) |  |  |  |  |  |
| 1.8 | Array Ohmic loss (dB) |  |  |  |  |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element/sub-array (dBm) |  |  |  |  |  |
| 1.10 | Base station horizontal coverage range (degrees) |  |  |  |  |  |
| 1.11 | Base station vertical coverage range (degrees) |  |  |  |  |  |
| 1.12 | Mechanical downtilt (degrees) |  |  |  |  |  |
| 1.13 | Maximum base station output power/sector (e.i.r.p.) (dBm) |  |  |  |  |  |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

{Summary of what is intended to be achieved. Free text}

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| External TR | 38.9xx | Parameters for IMT studies on 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz |  | RAN#103 | {e.g. rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {E.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {E.g. "TSG#89"} | {Free text, e.g. "This TS covers Stage 2" or "This TS covers Stage 3" or "This TS covers both stages 2 and 3"} |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s).}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.
Secondary co-Rapporteur(s) are possible for specific secondary task(s)}.

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

R4

# 8	Aspects that involve other WGs

{The “aspects” can be provided by topic (e.g. “security”, “multimedia”) and/or by specifying the WG(s) e.g.: "SA2, SA3, SA5, SA6. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet"}

{For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: possible values: "Not applicable", " unknown", "CT WGs", etc.}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Ericsson