3GPP TSG RAN Meeting #103									RP-240490

Maastricht, Netherlands, March 18-21, 2024

## Status Report to TSG

Agenda item:			9.4.4.6

| WI / SI Name | Even Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | 950078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221696 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:100% RAN4: 100% | Performance Part: 50% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#110 meeting

The following tdocs have been approved/endorsed in RAN4#110 meeting for performance part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2403466 | WF for [110][207] NR_RRM_enh3_part1 | Apple |
| R4-2403482 | WF for NR_RRM_enh3_part2 | OPPO |
| R4-2403483 | DraftCR on TC for HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC | OPPO |
| R4-2401321 | DraftCR on TC2 for HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC | Huawei, HiSilicon |
| R4-2403485 | Draft CR to TS38.133 performance test case for SCG activation for FR1-FR1 NR DC | Ericsson |
| R4-2401506 | Draft CR for test case on Conditional PSCell addition and release delay of NR PSCell in FR1 | vivo |
| R4-2401738 | DraftCR on PSCell addition and release delay of unknown FR1 PSCell for FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |

RRM performance part for FR2 SCell activation delay reduction

Issue 2-1-1: Performance requirement design

Agreement:

- In the test case, RAN4 specify that the existing L1-RSRP accuracy requirements (not include the corresponding side condition) shall be fulfilled irrespective of the sweeping factor as indicated in reduceForSSB-L1-RSRP-Meas. No new accuracy requirement to be defined in TS 38.133 clause 10.

Issue 2-2-1: Generic configuration for all TCs of FG31-1/2/3

Agreement:

- To define test cases for both FR1 and FR2 SCell activation delay reduction if RAN4 has defined the corresponding core requirements.

- Use CSSF =2 with 160ms in one FR2 test for L3 report during Scell activation, and use CSSF=1 with 160ms for FR1 and FR2 for all other tests as baseline.

Issue 2-3-1: How to configure to have valid L3 report for FG31-1 TC

Agreement:

- Configure event-triggered reporting associated with target SCell MO such as A2-event along with new reporting type for L3 report-based unknown SCell activation, and maintain the SNR above threshold so that A2 event is not met.

- TE use DCI to schedule DG- PUSCH for L3 measurement report, and the M is as specified in clause 8.3.17/18 of TS38.133.

  - Further discuss the exact M values for the test.

- Configure the following conditions in TC to make UE report valid L3 result (apply to FR1+FR1 2CC CA, FR1+FR2 2CC CA, FR2+FR2 inter-band CA as examples):

  - Non-DRX is configured; and;

  - CSSF = 1, i.e., only single MO is configured on target SCC; and;

  - The SSB measured remains detectable according to the cell identification conditions specified in clause 9.2; and

  - the SCell has been configured for a time period longer than Tidentify_intra_with_index in clause 9.2.5.1, provided the SCell is newly configured in deactivated state and the side condition Ês/Iot ≥ -2dB is fulfilled.

Issue 2-4-1: FG31-2 TC design

Agreement:

- to verify the reported L1-RSRP result can meet the accuracy requirements specified in TS38.133 clause 10.1.20.1/2.

- to verify beam sweeping factor reduction of X1 and X2 in the same test case.

Issue 2-6-1: Candidate test case list based on proposals from companies

Agreement:

Following TCs shall be introduced.

| Feature | Index | Test case | CA/DC mode | Other configurations | Test applicability | Volunteer Company to lead CR |
| --- | --- | --- | --- | --- | --- | --- |
| FG31-1: L3 report based enhancement | 1 | FR2 unknown SCell activation with L3 report | - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell |  | Define test applicability and UE is required to perform one test for each FR. UE only needs to pass test in one CA/DC mode. | QC |
|  | 2 | FR2 PUCCH SCell activation delay with L3 report |  |  |  | Huawei |
|  | 5 | FR1 unknown SCell activation with L3 report | - FR1 PCell+FR1 inter-band target SCell - LTE PCell + FR1 PSCell + FR1 inter-band target SCell |  | Define test applicability and UE is required to perform one test for each FR. UE only needs to pass test in one CA/DC mode. | vivo |
|  | 7 | Multiple SCell activation delay with FR1 unknown SCell with L3 report |  |  |  | ZTE |
| FG31-2: Beam sweeping factors reduction | 9 | FR2 unknown SCell activation with FG31-2 and FG31-3 | - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell | DRX.8: DRX cycle = 320 ms and TAT = Infinity | Verify UE supporting FG 31-2 and/or FG 31-3 for FR2, depending the capabilities supported by the UE. FFS:UE only needs to pass test in one CA/DC mode. | Nokia |
| FG31-3: (1)Use SSB periodicity instead of SMTC periodicity” (2)“Performing L1-RSRP measurement in non-DRX mode even DRX is configured” | 17 | FR1 unknown SCell activation with FG31-3 | - FR1 PCell+FR1 inter-band target SCell - LTE PCell + FR1 PSCell + FR1 inter-band target SCell | DRX.8: DRX cycle = 320 ms and TAT = Infinity | Verify UE supporting FG 31-3 for FR1. UE only needs to pass test in one CA/DC mode. | MTK |

RRM performance part for FR1-FR1 NR-DC

Agreement:

- Complete the Rel-18 WI performance part by June.

- If any update on the Rel-18 core requirement will be made in the future, the corresponding change will also be discussed for the Rel-18 performance test case.

- Postpone the discussion on Rel-17 performance part till June.

Agreement:

- For the Rel-18 test case, RAN4 agree that one TC is introduced to cover both RACH-based and RACH-less based SCG activation/deactivation procedure with known PSCell.

Agreement:

- Agree to introduce the three TCs for HO with PSCell. TC1 & 3 for parallel processing and TC2 for sequential processing.

Updated test case list:

| TC# | The RRM requirement for test case | Volunteer Company |
| --- | --- | --- |
| TC1 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC (for parallel processing) | Qualcomm |
| TC2 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC (for sequential processing) | Huawei |
| TC3 | HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC (for parallel processing) | OPPO |
| TC5 | PSCell addition and release delay of unknown FR1 PSCell | Nokia |
| TC6 | PSCell activation and deactivation for FR1+FR1 NR-DC | Ericsson |
| TC7 | Conditional PSCell addition and release delay of NR PSCell in FR1 | Vivo |

#### 2.4.2	Remaining Open issues

As captured in WF R4-2403466 and R4-2403482.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #110 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2400477 | (NR_RRM_enh3-Core) CR to TS 38.133 on R18 SCell activation enhancement capability | Apple |
| R4-2400478 | On FR2 SCell activation performance part | Apple |
| R4-2400479 | On RACH-less PSCell activation testing for FR1-FR1 NR-DC | Apple |
| R4-2400540 | View on RRM performance requirements for FR2 Scell activation delay reduction | Qualcomm Incorporated |
| R4-2400541 | View on RRM performance requirements for FR1+FR1 NR-DC | Qualcomm Incorporated |
| R4-2400743 | Topic summary for [110][207] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2400744 | Topic summary for [110][208] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2400807 | Open issues on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2400808 | 38.133 draftCR on enhancement for PUCCH SCell activation | Nokia, Nokia Shanghai Bell |
| R4-2400809 | Performance aspects for FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2400810 | draftCR on performance requirements for L1-RSRP reporting | Nokia, Nokia Shanghai Bell |
| R4-2400811 | draftCR on TCs for SCell activation with beam sweeping factor reduction | Nokia, Nokia Shanghai Bell |
| R4-2400995 | DraftCR on TC for HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC | OPPO |
| R4-2400996 | On RRM test case design and work splitting for FR1+FR1 NR-DC | OPPO |
| R4-2401041 | (NR_RRM_enh3-Perf) Discussion on performance requirements for FR2 SCell activation delay reduction | CMCC |
| R4-2401042 | (NR_RRM_enh3-Core) draft CR on SCell activation enhancement | CMCC |
| R4-2401236 | Discussion on RRM performance requirements for FR2 SCell activation delay reduction | China Telecom |
| R4-2401318 | Discussion on FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2401319 | DraftCR on maintenance for R18 eFeRRM | Huawei, HiSilicon |
| R4-2401320 | Discussion on performance requirements for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2401321 | DraftCR on TC2 for HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC | Huawei, HiSilicon |
| R4-2401415 | Discussion on eFeRRM core requirement for FR1-FR1 NR-DC | Ericsson |
| R4-2401416 | Draft CR to TS38.133 performance test case for SCG activation for FR1-FR1 NR DC | Ericsson |
| R4-2401506 | Draft CR for test case on Conditional PSCell addition and release delay of NR PSCell in FR1 | vivo |
| R4-2401636 | Discussion on the performance requirements for FR2 SCell activation enhancements | ZTE Corporation |
| R4-2401651 | [NR_RRM_enh3-Core] Draft CR on R18 L3 report based SCell activation enhancement | ZTE Corporation |
| R4-2401732 | Discussion on SCG activation delay in FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2401733 | DraftCR on SCG activation delay for FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2401738 | DraftCR on PSCell addition and release delay of unknown FR1 PSCell for FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2401927 | draft CR on R18 FR2 SCell activation delay reduction | vivo |
| R4-2401928 | Discussion on test cases of FR2 SCell activation delay reduction | vivo |
| R4-2401929 | draft CR on test case for R18 FR2 SCell activation delay reduction | vivo |
| R4-2402621 | Big CR to TS 38.133 on R18 SCell activation enhancement maintenance | Apple |
| R4-2402847 | RRM performance requirements for FR2 SCell activation delay reduction | Ericsson |
| R4-2402896 | Draft CR on correction for Rel-18 SCell activation enhancement using L3 reporting | MediaTek inc. |
| R4-2402897 | Discussion on the performance requirements for FR2 SCell activation enhancement | MediaTek inc. |
| R4-2402934 | FR2 Scell delay reduction maintenance CR | Ericsson |
| R4-2403458 | (NR_RRM_enh3-Core) CR to TS 38.133 on R18 SCell activation enhancement capability | Apple |
| R4-2403459 | 38.133 draftCR on enhancement for PUCCH SCell activation | Nokia, Nokia Shanghai Bell |
| R4-2403462 | [NR_RRM_enh3-Core] Draft CR on R18 L3 report based SCell activation enhancement | ZTE Corporation |
| R4-2403463 | draft CR on R18 FR2 SCell activation delay reduction | vivo |
| R4-2403464 | Draft CR on correction for Rel-18 SCell activation enhancement using L3 reporting | MediaTek inc. |
| R4-2403466 | WF for [110][207] NR_RRM_enh3_part1 | Apple |
| R4-2403482 | WF for NR_RRM_enh3_part2 | OPPO |
| R4-2403483 | DraftCR on TC for HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC | OPPO |
| R4-2403484 | DraftCR on TC2 for HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC | Huawei, HiSilicon |
| R4-2403485 | Draft CR to TS38.133 performance test case for SCG activation for FR1-FR1 NR DC | Ericsson |
| R4-2403551 | [NR_RRM_enh3-Core] Draft CR on R18 L3 report based SCell activation enhancement | ZTE Corporation |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template