3GPP TSG RAN WG1 #116							  	     R1-2401719

Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0177 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on reportQuantity for RSRP/SINR in TS38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (NEC), Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The reportQuantity 'ssb-Index-RSRP’ and 'ssb-Index-SINR' for RSRP/SINR was mentioned for SS/PBCH blocks in corresponding resource set for Table 6.3.1.1.2-6 and Table 6.3.1.1.2-6A in current TS 38.212, and in Rel-17, CapabilityIndex for panel related report was introduced, while the new introduced reportQuantity 'ssb-Index-RSRP-Index' and 'ssb-Index-SINR-Index' were not captured. We also notice that there is no reportQuantity mentioned for CSI-RS resources in corresponding resource set, so either adding the missed reportQuantity 'ssb-Index-RSRP-Index’ and 'ssb-Index-SINR-Index' or just removing the reportQuantity 'ssb-Index-RSRP' and 'ssb-Index-SINR' for SS/PBCH blocks in corresponding resource set for Table 6.3.1.1.2-6 and Table 6.3.1.1.2-6A is fine, rather than leaving the configuration incomplete as in current TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the reportQuantity 'ssb-Index-RSRP-Index' and 'ssb-Index-SINR-Index' for SS/PBCH blocks in corresponding resource set for Table 6.3.1.1.2-6 and Table 6.3.1.1.2-6A. Or we can just remove the reportQuantity 'ssb-Index-RSRP' and 'ssb-Index-SINR' for SS/PBCH blocks in corresponding resource set for Table 6.3.1.1.2-6 and Table 6.3.1.1.2-6A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous for UE when the reportQuantity is configured as 'ssb-Index-RSRP-Index' or 'ssb-Index-SINR-Index'. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.1.1.2	  CSI only

<Unchanged part omitted>

The bitwidth for CRI, SSBRI, RSRP, differential RSRP, and CapabilityIndex are provided in Table 6.3.1.1.2-6.

Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |

where  is the number of CSI-RS resources in the corresponding resource set, and  is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index'.

The bitwidth for CRI, SSBRI, SINR, differential SINR, and CapabilityIndex are provided in Table 6.3.1.1.2-6A.

Table 6.3.1.1.2-6A: CRI, SSBRI, SINR, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI | log2KsCSI-RS |
| SSBRI | log2KsSSB |
| SINR | 7 |
| Differential SINR | 4 |
| CapabilityIndex | 2 |

where KsCSI-RS  is the number of CSI-RS resources in the corresponding resource set, and KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-SINR' or 'ssb-Index-SINR-Index'.

<Unchanged part omitted>