3GPP TSG-RAN WG1 Meeting #116	R1-2401721

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0501 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NCJT CSI-IM restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Nokia), Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE behaviour in dropping an NCJT CSI report after (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or when DRX is configured, is not defined with respect to interference measurement when CSI-IM are configured. This ambiguity is problematic for NCJT because a UE may report a valid CSI after measuring interference for all, some or none of the configured transmission hypotheses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a condition that, after (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, or when DRX is configured, one CSI-IM transmission occasion needs to be received for each of the CSI-IM resources in the corresponding Resource Set for interference measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interference measurement in the reported NCJT CSI report may be performed for all, some or none of the configured transmission hypotheses resulting in unreliable CSI measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.5	CSI reference resource definition

<omitted text>

After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.

For a CSI-ReportConfig configured with two Resource Groups and  N  Resource Pairs for channel measurement in the corresponding CSI-RS Resource Set, as described in clause 5.2.1.4.1, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-IM occasion for each of the CSI-IM resources in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time when DRX is configured, and drops the report otherwise.

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index ' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', or 'ssb-Index-RSRP- Index' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP- Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

<omitted text>