3GPP TSG RAN WG1 #116                                                                               R1-24017389

Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0506 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on sub-set of {DL TCI, UL TCI} indication for Rel-17 TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-02-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#105-e, the following agreement was made, and if either of {DL TCI, UL TCI} is indicated, UE updates the one indicated {DL TCI, UL TCI}and remains the other {DL TCI, UL TCI}. Agreement For M=N=1, on Rel-17 unified TCI, for separate DL/UL TCI, one instance of beam indication using DCI formats 1_1/1_2 (with and without DL assignment) can be used as follows: - One TCI field codepoint represents a pair of DL TCI state and UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state and UL TCI state. - One TCI field codepoint represents only a DL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state, and keeps the current UL TCI state. - One TCI field codepoint represents only an UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding UL TCI state, and keeps the current DL TCI state. FFS: the cases of M or N>1. However, the above agreement of the sub-set of {DL TCI, UL TCI} indication is not captured in the current spec. |
| Summary of change: |  | Capture the agreement of the sub-set of the {DL TCI, UL TCI} indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not capture the agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of this CR. |  |  |  |  |  |  |  |  |

### 5.1.5	Antenna ports quasi co-location

<Unchanged part omitted>

When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK.

When a UE is configured with dl-OrJointTCI-StateList, and if the UE is configured with unifiedTCI-StateType is set as ‘separate’, and if the UE receives a TCI codepoint mapped with either of {TCI-State, TCI-UL-State}, the UE shall update the one indicated {TCI-State, TCI-UL-State} and maintain the other {TCI-State, TCI-UL-State} that is not updated by the received TCI codepoint.

<Unchanged part omitted>