3GPP TSG-WG1 Meeting # 116	R1-2401921

# Athens, Greece, February 26th - March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1443 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrrections to IoT (Internet of Things) NTN (non-terrestrial network) enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on disabling of HARQ feedback for IoT (Internet of Things) NTN (non-terrestrial network) enhancements - Clarify that when multiple TBs are scheduled by a single DCI and DCI indicates HARQ feedback enabled, then the NB-IoT UE always wait for an RTT+3ms (i.e., till subframe n+Kmac+3) before monitoring NPDCCH in clause 16.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 16.6: Added condition that NB-IoT UE always wait for an RTT+3ms (i.e., till subframe n+Kmac+3) before monitoring NPDCCH when multiple TBs are scheduled by a single DCI and DCI indicates HARQ feedback enabled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation of Rel-18 IoT-NTN enhancement features. - NB-IoT UE will need to monitor NPDCCH during RTT+3ms (i.e., till subframe n+Kmac+3) when multiple TBs are scheduled by a single DCI and DCI indicates HARQ feedback enabled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 16.6	Narrowband physical downlink control channel related procedures

Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0.

<Unchanged parts are omitted>

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3, or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 except if the UE is configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’ for the same HARQ process ID, or if the NPUSCH transmission carries ACK/NACK response, as determined in clause 16.4.2, for the same HARQ process ID associated with a transport block scheduled in a NPDCCH scheduling a single transport block, and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information for the same HARQ process ID and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3] for FDD; and

-	the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 except if the UE is configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’, or if the NPUSCH transmission carries ACK/NACK response as determined in clause 16.4.2 and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3.

<Unchanged parts are omitted>