3GPP TSG-RAN WG1 Meeting #116	R1-2401908

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0538 | Rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the support for Expanded and Improved NR Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.2.1.4, align the higher-layer parameters. In clause 6.2.1, the current specification text in 38.214 does not include the description for the hyper SFN parameter of the SRS for positioning in LPHAP. In clause 5.1.6.5.2, a Rel-17 UE can perform positioning measurements inside measurement gap or PPW. According to the agreements of the 3GPP RAN1#113, a Rel-18 UE can only perform CPP positioning measurements within measurement gap. In Rel-18, the DL RSCPD and/or DL RSCP can be reported together with the Rel-16 timing measurement (RSTD / UE Rx-Tx time difference), but it needs to be clarified at 38.214 that a Rel-18 UE can only perform CPP measurement within MG. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Conclusion From RAN1’s perspective, carrier phase positioning for UE in RRC_CONNECTED state without measurement gap is not supported in Rel-18. Agreement From RAN1’s perspective, carrier phase positioning for UE in RRC_IDLE state is supported for UE-based and UE-assisted positioning in Rel-18. - Note: No additional specification work is expected specifically related to carrier phase positioning for UE in RRC_IDLE state in RAN1. In clause 5.1.6.5, 1) there is a typo in “within one or more-two time window(s)”, 2) “DL carrier phase measurement” should be replaced with DL RSCP/RSCPD measurement, 3) A number of IEs in brackets in 38.214 can be replaced with the IEs defined in TS 37.355. In clause 5.1.6.5, RAN1#115, the following agreement was made: Agreement When an LMF requests a UE, which can be a target UE and a PRU, to perform measurements on indicated DL PRS resource set(s) occurring within an indicated time window. - The UE may use the indicated DL PRS resource set(s) occurring outside the indicated time window for legacy measurements in addition to the indicated DL PRS resource set(s) occurring inside the indicated time window. - Introduce an optional UE capability for supporting to perform legacy measurements inside the indicated time window only, and an associated configuration to enable legacy measurements inside the time window only. The above agreement seems not be fully captured in TS 38.214. In clause 5.1.6.5.3, RAN4 sent an LS to ask RAN1 specifying the condition of ‘the same Tx antenna’ in RAN1 specifications. In clause 6.2.1.4.2, there are brackets in the specification. In clauses 5.1.6.5.3 and 6.2.1.4.2, some agreed conditions such as subcarrier spacing for PRS, spatial relation and resource type for SRS are not captured properly for common transmission properties of PRS or SRS bandwidth aggregation. In addition, the phase continuity for SRS bandwidth aggregation should be on the same symbol(s). In clause 5.1.6.5, some parameters are still in brackets in TS 38.214 for PRS priority. In clauses 5.1.6.5.3 and 6.2.1.4.2, make the higher layer parameters in TS 38.214 aligned with TS 37.355 In clause 5.1.6.5.3, for bandwidth aggregation, the reference should be aggregated PRS resources from different PRS resource sets. In clause 5.1.6.5.3, clarify the number of RSTD or UE Rx-Tx measurements to support the TEG reporting for PRS/SRS bandwidth aggregation In clause 6.2.1.4.1, for the timeline for determination of collision between PUSCH or PUCCH and positioning SRS with frequency hopping, it was agreed to use the smallest SCS between the SCS configured for the SRS with Tx hopping, the SCS of the PUSCH, and the SCS of the PDCCH for the calculation of N2. However, the SCS of the PUCCH should be also included for the calculation of N2. In clause 8.2.4.1.2, current description lacks clarity as to the number of RBs given by sl-RB-Number of which resource pool is referenced in determining the bandwidth of a SL PRS resource in a dedicated SL PRS resource pool. In clause 8.4.4, the current spec does not capture the anchor UE behaviour if the synchronization source is gNB or eNB. In clause 8.4.4, the current spec does not capture the UE may be provided with synchronization source type of a UE and/or the relative time difference with the associated quality metric. In clause 8.4.4, to resolve the misalignment issue about higher-layer parameter names between RAN1 and RAN2 In clause 8.2.4.2, in dedicated SL PRS resource pool, RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH. In clause 8.2.4.2, there are unresolved brackets |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.2.1.4, updated higher-layer parameter names. In clause 6.2.1, include Hyper SFN parameter for SRS configuration for TS 38.214 Clause 6.2.1. In clause 5.1.6.5.2, added the UE behavior that a Rel-18 UE can only perform DL RSCPD and/or DL RSCP measurements in measurement gap. In clause 5.1.6.5, 1) Corrected the typo, 2) Replaced “DL carrier phase measurement” with “DL RSCP/RSCPD measurement”, 3) Replaced the IEs in brackets in 38.214 with the IEs defined in TS 37.355. In clause 5.1.6.5, changed the description in 38.214 Cluse 5.1.6.5 to fully capture the above agreement. In clause 5.1.6.5.3, for the PRS/SRS BW aggregation, the aggregated PRS/SRS resources in two or three PFLs/carriers for a TRP/UE should have the same antenna port from RAN1 perspective In clause 6.2.1.4.2, removed the whole bracket and make the spec complete In clauses 5.1.6.5.3 and 6.2.1.4.2, captured the previous agreements properly in the specification. In clause 5.1.6.5, for ranking the TRPs, the term “DL PRS ID” in a similar way as in TS37.355 is used with-out having to necessarily refer to the exact RRC name dl-PRS-ID. Also, some small wording corrections are done. In clauses 5.1.6.5.3 and 6.2.1.4.2, changed the higher layer parameters in TS 38.214 to be aligned with TS 37.355 In clause 5.1.6.5.3, added the paragraph about the aggregated reference. In clause 5.1.6.5.3, added the description about the number of measurements per TEG. In clause 6.2.1.4.1, include the SCS of PUCCH for the calculation of N2 for determination of collision between PUSCH or PUCCH and positioning SRS with frequency hopping for RedCap UEs. In clause 8.2.4.1.2, clarified that the bandwidth of SL PRS in a dedicated SL PRS resource pool is same as that of the bandwidth of the resource pool indicated via the higher layer parameter sl-RB-Number of the corresponding dedicated SL PRS re-source pool by changing “a” to “the”. In clause 8.4.4, added corresponding description In clause 8.4.4, modify some higher-layer parameter name and address the remaining issue. In clause 8.2.4.2, corrected PSCCH-RSRP over the DM-RS resource elements of the PSCCH, not PSSCH. In clause 8.2.4.2, changed the “[delay budget]” to “SL PRS delay budget" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear or incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5, 5.1.6.5.2, 5.1.6.5.3, 6.2.1, 6.2.1.4, 6.2.1.4.1, 6.2.1.4.2, 8.2.4.1.2, 8.2.4.2, 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.6.5	PRS reception procedure

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer.

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource.

A DL PRS positioning frequency layer is configured by NR-DL-PRS-PositioningFrequencyLayer, consists of one or more DL PRS resource sets and it is defined by:

-	dl-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing. The supported values of dl-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211], excluding the values of 240kHz, 480 kHz, and 960 kHz.

-	dl-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-CyclicPrefix. The supported values of dl-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	dl-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resources sets belonging to the same DL PRS positioning frequency layer have a common Point A.

The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource.

The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID.

-	If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell;

-	Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell.

If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell.

If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell.

A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by:

-	nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration.

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively.

-	dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor.

-	dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceID within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap.

-	dl-PRS-MutingOption1 and dl-PRS-MutingOption2 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If dl-PRS-MutingOption1 is configured, each bit in the bitmap of dl-PRS-MutingOption1 corresponds to a configurable number provided by higher layer parameter dl-prs-MutingBitRepetitionFactor of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If dl-PRS-MutingOption2 is configured each bit in the bitmap of dl-PRS-MutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor. Both dl-PRS-MutingOption1 and dl-PRS-MutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211].

-	NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo.

-	dl-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set.

-	dl-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211]. All DL PRS resource sets belonging to the same DL PRS positioning frequency layer have the same value of dl-PRS-CombSizeN.

-	dl-PRS-ResourceBandwidth defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a DL PRS positioning frequency layer have the same value of dl-PRS-ResourceBandwidth.

-	dl-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same DL PRS positioning frequency layer have the same value of dl-PRS-StartPRB.

-	dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211].

A DL PRS resource is defined by:

-	nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211].

-	dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset.

-	dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource.

-	dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS associated with the same dl-PRS-ID, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell.

-	dl-PRS-ResourcePrioritySubset defines a subset of DL-PRS resources for the DL PRS resource for the purpose of prioritization of measurement reporting as described in [17, TS 37.355].

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

The UE may be indicated by the network that DL PRS resource(s) can be used as the reference for the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo may include a dl-PRS-ID, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs [17, TS 37.355]. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

The UE may be configured to report quality metrics NR-TimingQuality corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields:

-	timingQualityValue which provides the best estimate of the uncertainty of the measurement

-	timingQualityResolution which specifies the resolution levels used in the timingQualityValue field.

The UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and nr-DL-PRS-ExpectedRSTD-Uncertainty, which defines a search window around the nr-DL-PRS-ExpectedRSTD.

For DL UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, or UE Rx-Tx time difference, respectively.

For the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements the UE reports an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp can include the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo.

The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When the UE is expected to measure the DL PRS resource, the UE may request a measurement gap via higher layer parameter NR-PRS-MeasurementInfoList [12, TS 38.331] or as specified in clause 6.1.3.40 of [10, TS 38.321]. The UE may be preconfigured with one or more measurement gaps each associated with a measPosPreConfigGapId. When the UE requests activation or deactivation of a measurement gap as specified in clause 6.1.3.40 of [10, TS 38.321] it can request one of the preconfigured measurement gaps by referring to the measPosPreConfigGapId. The UE may have one of the preconfigured measurement gap(s) activated or deactivated as specified in clause 6.1.3.41 of [10, TS 38.321].

The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell.

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. If the UE is not configured to report with multiMeasInSameReport-r17, the up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing. If the UE is configured to report with multiMeasInSameReport-r17, the up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same measurement instance of the same report use a single reference timing.

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. When the UE reports DL PRS-RSRP measurements for a DL PRS resource, the reported multiple DL PRS-RSRP measurements associated with the same or different higher layer parameter nr-DL-PRS-RxBeamIndex may have the same or different timestamps.

The UE may be configured to measure and optionally report, subject to UE capability, up to 24 DL PRS-RSRPP for the first detected path on DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRPP measurements for a DL PRS resource, the reported multiple DL PRS-RSRPP measurements associated with the same or different higher layer parameter nr-DL-PRS-RxBeamIndex may have the same or different timestamps. When the UE reports DL PRS-RSRPP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRPP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRPP measurements associated with it within the DL PRS resource set.

The UE may be configured to optionally report a differential DL PRS-RSRPP for a DL PRS resource with reference to nr-DL-PRS-FirstPathRSRP-Result and/or a differential DL PRS RSRP with reference to nr-DL-PRS-RSRP-Result via higher layer parameter NR-DL-AoD-AdditionalMeasurementElement.

For each DL PRS resource, the UE may be configured, subject to UE capability, with dl-PRS-ResourcePrioritySubset that is associated with this DL PRS resource, where the subset of DL PRS resources associated with the DL PRS resource can be in the same or different DL PRS resource set than the DL PRS resource. The UE may include UE measurements for the subset of DL PRS resources in NR-DL-AoD-AdditionalMeasurementElement if the UE measurements of the associated PRS resource are reported, where the UE measurement can be DL PRS-RSRP and/or DL PRS-RSRPP. The UE may report DL PRS-RSRP and/or DL PRS-RSRPP measurements only for the subset of DL PRS resources. Subject to UE capability, the UE may be configured with boresight direction via higher layer parameter DL-PRS-BeamInfoElement for each DL PRS resource.

The UE may be provided with beam/antenna information via higher layer parameter NR-TRP-BeamAntennaInfo.

The UE may request to be provided with either expected DL-AoD/ZoD and uncertainty range(s) of expected DL-AoD/ZoD, or expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The UE may be provided with expected DL-AoD/ZoD and uncertainty range(s) of the expected DL-AoD/ZoD. The UE may be provided with expected DL-AoA/ZoA and uncertainty range(s) of the expected DL-AoA/ZoA. The uncertainty range(s) of the expected DL-AoD/DL-AoA may be configured within [0, 60]. The uncertainty range(s) of expected DL-ZoD/DL-ZoA may be configured within [0, 30].

The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different DL PRS positioning frequency layers.

The UE may be configured to measure and report via higher layer parameter additionalPaths or additionalPathsExt, subject to UE capability, the timing and the quality metrics of up to 8 additional detected paths, that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD or nr-UE-RxTxTimeDiff. For UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation, the UE may be configured to measure and report, subject to UE capability, the DL PRS-RSRPP of the first path and the up to 8 additional paths that are associated with each RSTD or UE Rx – Tx time difference.

The UE may be requested, subject to UE capability, to measure and report one or more of the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference measurements with either Nsample or 4 samples, where Nsample = 1 or 2 is as defined in [11, TS 38.133], via higher layer parameter reducedDL-PRS-ProcessingSamples [17, TS 37.355] which applies for all DL PRS positioning frequency layers.

The UE may be requested, subject to UE capability, to report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-IndicatorRequest. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements. The UE can report LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator associated with each dl-PRS-ID in a measurement report. For the LoS/NLoS indicator(s) associated with DL RSTD, the UE may report one indicator associated with the dl-PRS-ID indicated by higher layer parameter dl-PRS-ReferenceInfo and one indicator associated with the dl-PRS-ID of the DL RSTD measurement. A UE may be provided with LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator, and it may be associated with each DL PRS resource of each configured dl-PRS-ID or may be associated with each configured dl-PRS-ID. The values of the higher layer parameter LOS-NLOS-Indicator may be soft values (0, 0.1, …, 0.9, 1) or hard values (0, 1) with the values corresponding to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS.

If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with QCL set to 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE.

The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter DL-PPW-PreConfig. The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by UE capability. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the priority between DL PRS and SSB is defined in [11, TS 38.133] and the UE determines the DL PRS priority as indicated by higher layer parameter priority subject to UE capability or as implied by UE capability, except for SSB:

-	with value 'st1' where the DL PRS is higher priority than all the DL signals and channels, or

-	with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1, 1_2, 1_3 or 4_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals and channels, or

-	with value 'st3' where the DL PRS is lower priority than all the DL signals and channels.

Inside one DL-PPW-PreConfig the UE is only expected to measure a single DL PRS positioning frequency layer.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type1A and if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, those DL signals and channels are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type1B and if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, those DL signals and channels in the same band as the DL PRS are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type2 if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, those DL signals and channels from the impacted serving cells are not expected to be measured by the UE on the overlapped symbols with the DL PRS, where impacted serving cells refer to the serving cell on which the DL-PPW-PreConfig is configured for a frequency range 1 band, and all the serving cells in the same band as the DL PRS for a frequency range 2 band. When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type1B or type2, and if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, the UE behavior is described in [11, TS 38.133] for inter-band case for frequency range 2 for the DL signals/channels from a different frequency range 2 band than the frequency range 2 band of the DL PRS.

When the UE has an activated DL PRS processing window with type1A or type1B and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS in the DL PRS processing window no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the DL PRS processing window, the UE is expected to receive the other DL signals and channels and drop all PRS within the DL PRS processing window. When the UE has an activated DL PRS processing window with type2 and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS on a symbol configured with the DL PRS no later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the DL PRS symbol, the UE is expected to receive the other DL signals and channels and drop the DL PRS symbol.

When the UE has an activated DL PRS processing window with type1A or type1B and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS in the DL PRS processing window later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the first symbol of the DL PRS processing window, the UE is not required to receive the other DL signals and channels and may receive the DL PRS and consider the DL PRS as higher priority in the DL PRS processing window. When the UE has an activated DL PRS processing window with type2 and the UE determines the presence of other DL signals and channels, except SSB, of higher priority than the DL PRS on a symbol configured with the DL PRS later than N2 symbols, defined in clause 6.4 for the subcarrier spacing μ of the DL PRS, before the DL PRS symbols, the UE is not required to receive the other DL signals and channels and may receive the DL PRS symbol and consider the DL PRS as higher priority in that symbol.

Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed:

-	Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the DL PRS positioning frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority; except when the UE is requested to perform aggregated measurement(s), in which case:

-	A DL PRS ID associated with DL PRS bandwidth aggregation linkage has higher priority than a DL PRS ID not associated with DL PRS bandwidth aggregation linkage. If multiple DL-PRS ID are associated with DL PRS bandwidth aggregation linkage, they are sorted according to priority.

-	Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the DL PRS positioning frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the DL PRS positioning frequency layer are sorted according to priority except when the UE is requested to perform aggregated measurement(s), in which case:

-	A DL PRS resource set linked for a DL PRS bandwidth aggregation has higher priority than a DL PRS resource set not linked for DL PRS bandwidth aggregation. If multiple DL PRS resource sets are linked for DL PRS bandwidth aggregation, then they are sorted according to priority.

The UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window, is calculated by

-	Type 1 duration calculation with UE symbol level buffering capability

K=s∈SKs
Ks=Tsend-Tsstart

-	Type 2 duration calculation with UE slot level buffering capability

K=12μS

-	S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets.

-	For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential DL PRS symbols and determines the DL PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference).

-	For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S.

The UE may be configured to report one or more measurement instances, each with its own timestamp, on DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and/or UE Rx-Tx time difference measurements, in a single measurement report.

Timing Error Group(s) (TEG(s)) at UE side are defined:

-	UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin.

-	UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement.

If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE may report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a DL PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be provided with association information of DL PRS resource(s) with TRP Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers.

The UE may be configured to measure and report, via higher layer parameter [undetermined NTN related parameter] subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID, and report the UE Rx-Tx time difference subframe offset and the DL timing drift as described in [7, TS 38.215].

The UE in RRC_INACTIVE mode is expected to prioritize the reception of any other DL signals and DL channels than the reception of DL PRS.

The UE in RRC_INACTIVE mode, subject to UE capability, is expected to process DL PRS outside or inside of the initial DL BWP. For DL PRS processing outside of the initial DL BWP, the UE may be configured with the same or different subcarrier spacing and CP for DL PRS resources than those of the initial DL BWP. For DL PRS processing inside of the initial DL BWP, the UE is configured with the same subcarrier spacing and CP for DL PRS resources as those of the initial DL BWP.

For a UE configured with preconfigured Measurement gap(s) for Positioning, when the UE receives an activation command, as described in clause 6.1.3.41 of [10, TS 38.321], for a preconfigured Measurement Gap for Positioning activation/deactivation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with DL PRS Processing Window(s), when the UE receives an activation/deactivation command, as described in clause 6.1.3.42 of [10, TS 38.321], for a DL PRS processing window activation, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the command, the corresponding actions in [10, TS 38.321] and the UE assumptions shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The UE is not expected to be indicated with more than 4 activated DL PRS processing windows across all active DL BWPs and is not expected to be indicated with the activated DL PRS processing windows that overlap in time.

The UE, subject to UE capability, may be requested to perform DL RSCPD and/or DL RSCP measurements on indicated DL PRS resource sets occurring within one or two time window(s) indicated by NR-DL-PRS-MeasurementTimeWindowsConfig. Within each window indicated by NR-DL-PRS-MeasurementTimeWindowsConfig, the UE expects that the indicated DL PRS resource sets across all dl-PRS-IDs are from one DL PRS positioning frequency layer, and that the number of indicated DL PRS resource sets associated with each dl-PRS-ID are the same.

The UE, subject to UE capability, may be requested to perform DL RSTD, UE Rx – Tx time difference, DL PRS-RSRP, and DL PRS-RSRPP measurement on the indicated DL PRS resource sets only within the window(s) indicated by NR-DL-PRS-MeasurementTimeWindowsConfig. Otherwise, UE may use the indicated DL PRS resource set(s) occurring outside the indicated time window for these measurements in addition to the indicated DL PRS resource set(s) occurring inside the indicated time window(s).

<omitted text>

##### 5.1.6.5.2	PRS for carrier phase positioning

For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements, the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. For a UE in RRC_CONNECTED state, DL RSCP/RSCPD measurements are measured within the configured measurement gap.

The UE is expected to obtain each DL RSCP or DL RSCPD measurement with Nsample=1 as defined in [11, TS 38.133]. If the UE reports a DL RSTD measurement with Nsample = 2 or 4 samples as defined in [11, TS 38.133], up to Nsample DL RSCPD measurements can be reported associated with the DL RSTD measurement. If the UE reports a UE Rx-Tx time difference measurement with Nsample = 2 or 4 samples as defined in [11, TS 38.133], up to Nsample DL RSCP measurements can be reported associated with the UE Rx-Tx time difference measurement. Each DL RSCP or DL RSCPD measurement has its own timestamp.

When the UE reports a timestamp associated with a DL RSCP measurement or a DL RSCPD measurement, subject to UE capability, it may include a symbol index in the timestamp.

If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements.

The UE may be provided with [nr-PruInformation-Ue-based-DL-CPP ] which contains DL RSCP/RSCPD measurements together with DL RSTD, DL PRS-RSRP, and/or DL PRS-RSRPP measurement(s) associated with the RSCP/RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] the timestamps associated with the measurements, and the location information of the PRU.

The UE may be configured to report quality metrics [nr-CarrierPhaseQualityInfo] corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]:

-	[phase quality index] which provides the uncertainty of the measurement

-	[phase quality resolution] which specifies the resolution levels used in the [phase quality index] field.

The UE in RRC_INACTIVE or RRC_IDLE mode is expected to perform the DL carrier phase measurement from the bandwidth of a DL PRS resource including outside of the initial downlink bandwidth part.

##### 5.1.6.5.3	PRS bandwidth aggregation for positioning measurements

When the UE is expected to perform aggregated measurements for bandwidth aggregation across DL PRS positioning frequency layers, the UE expects to be configured with linkage information, via higher layer parameternr-DL-PRS-AggregationInfo, between DL PRS resource sets across DL PRS positioning frequency layers. For the linked DL PRS resource sets, the UE is expected to be configured with the same values of QCL, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, dl-PRS-ResourceTimeGap, dl-PRS-ResourceRepetitionFactor, dl-PRS-ResourceSymbolOffset, dl-PRS-MutingBitRepetitionFactor, dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix, comb size, power per subcarrier, NR-MutingPattern, and NR-DL-PRS-SFN0-Offset, and the UE is expected to be configured with DL PRS resources that maintain uniformly spaced DL PRS RE pattern within a symbol across aggregated DL PRS positioning frequency layers. The UE assumes that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, and the UE may assume phase continuity on the DL PRS resources on same symbol(s); otherwise, the UE does not assume that PRS resources from the linked DL PRS resource sets are linked for bandwidth aggregation.

The UE may be indicated by the network that aggregated DL PRS resource set(s) can be used as the reference for the aggregated DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements.

The UE may be configured to measure and report, subject to UE capability, up to 4 aggregated DL RSTD measurement(s) per pair of dl-PRS-ID, from a different pair of aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE may report up to 4 RSTD measurements associated with different aggregated DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may be configured to measure and report, subject to UE capability, up to 4 aggregated UE Rx-Tx time difference measurement(s) from aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different aggregated DL PRS resources per UE RxTx TEG per dl-PRS-ID.

The UE may be requested via higher layer parameter nr-DL-PRS-JointMeasurementRequested  to perform the aggregated DL RSTD measurement(s) or the aggregated UE Rx-Tx time difference measurement(s) across two or three DL PRS positioning frequency layers.

The UE may report via higher layer parameter nr-RSTD-BasedOnAggregatedResources or nr-UE-RxTxTimeDiffBasedOnAggregatedResources in a measurement report whether the aggregated DL RSTD measurement(s) or the aggregated UE Rx-Tx time difference measurement(s) is performed. If any aggregated measurement is performed, the two or three DL PRS positioning frequency layers to be used may also be reported by reporting PRS resource set IDs.

If the UE reports a DL PRS-RSRP or a DL PRS-RSRPP with aggregated DL RSTD measurement(s) or aggregated UE Rx-Tx time difference measurement(s), the DL PRS-RSRP or the DL PRS-RSRPP correspond to the aggregated DL PRS resources across two or three DL PRS positioning frequency layers.

For PRS resources on multiple DL PRS positioning frequency layers (PFLs) linked for aggregation, the channel over which a symbol on one PFL for PRS transmission is conveyed can be inferred from the channel over which the same symbol of another PFL or the aggregated PFL is conveyed.

<omitted text>

### 6.2.1	UE sounding procedure

The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet or SRS-PosResourceSet. For each SRS resource set configured by SRS-ResourceSet, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306]. When SRS resource set is configured with the higher layer parameter SRS-PosResourceSet, a UE may be configured with K ≥1 SRS resources (higher layer parameter SRS-PosResource), where the maximum value of K is 16. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS resource sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.

During non-active periods of cell DRX, the UE configured with cell DRX is not expected to transmit the periodic SRS, or semi-persistent SRS for channel acquisition. SRS for positioning is not impacted by cell DRX operation.

For the SRS resource set(s) configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage set to 'antennaSwitching' or 'beamManagement', the UE expects the same SRS resource set(s) with the same usage being configured in srs-ResourceSetToAddModList.

When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE can assume that SRS resource(s) in any SRS resource set, except SRS resource set for positioning and an SRS resource set configured with followUnifiedTCI-StateSRS, can be configured with TCI-State or TCI-UL-State or updated as described in clause 6.1.3.59 or 6.1.3.60 of [10, TS 38.321]. The reference RS in the TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the TCI-UL-State(s) can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

If an SRS resource set, except an SRS resource set for positioning, is configured with followUnifiedTCI-StateSRS, the UE shall transmit the target SRS resource(s) within the SRS resource set according to the spatial relation, if applicable, with a reference to the RS used for determining UL TX spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' in QCL-Info of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

When the UE is configured dl-OrJointTCI-StateList or TCI-UL-State and is having two indicated TCI-States or TCI-UL-States, and if the UE is configured with [[followUnifiedTCI-StateSRS]] to, a periodic, semi-persistent or aperiodic SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'codebook', 'nonCodebook' or 'antennaSwitching' or to an aperiodic SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'beamManagement'

-	The UE may be configured by higher layer parameter applyIndicatedTCIState to the SRS resource set to indicate whether the UE shall apply the first or the second indicated TCI-State or TCI-UL-State to the SRS resource set.

-	When a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the first and second indicated TCI-States or TCI-UL-States correspond to the indicated TCI-States or TCI-UL-States specific to coresetPoolIndex value 0 and value 1, respectively.

-	When a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, and the aperiodic SRS resource set which is not configured with higher layer parameter applyIndicatedTCIState and the aperiodic SRS resource set is triggered by PDCCH on a CORESET associated with a coresetPoolIndex value, the UE shall apply the indicated TCI-State or TCI-UL-State specific to the coresetPoolIndex value to the aperiodic SRS resource set.

-	When two SRS resource sets with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'nonCodebook' are configured, the UE does not expectthat the first indicated TCI-State or TCI-UL-State is applied to the second SRS resource set and that the second indicated TCI-State or TCI-UL-State is applied to the first SRS resource set.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource.

-	srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity.

-	Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource.

-	srs-PosHyperSFN-Index indicates whether the current hyper-frame number is even or odd for SRS for positioning transmission, if configured.

-	Support of time division mapping subsets of ports of the SRS resource into S symbols (S=2), as defined by the higher layer parameter [tdm], where the SRS ports are evenly distributed in two consecutive symbols over the symbols in a slot for the SRS resource according to clause 6.4.1.4.2 in [4, TS 38.211]. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to 'codebook', or 'antennaSwitching', and nrofSRS-Ports is set to 'n8'.

-	Comb offset hopping pattern with repetition, as defined by the higher layer parameter [combOffsetHoppingWithRepetition], where the parameter can be set to either '[per-symbol]' or '[per-R-repetition]' subject to UE capability. When the parameter is set to '[per-symbol]', the comb offset hopping pattern is determined by the symbol index, and the comb offset hopping pattern is determined by the symbol index of the first symbol of the repetition when the parameter is set to '[per-R-repetition]' according to clause 6.4.1.4.3 in [4, TS 38.211].

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured for positioning SRS, then R is equal to the number of OFDM symbols in the SRS resource.

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0.

-	Frequency hopping bandwidth , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0.

-	Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0.

-	Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khop as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khop is fixed to be 0 for all SRS symbols.

-	Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero.

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or 'codebook', subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] and [cyclicShiftHoppingSubset] are configured simultaneously for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

-	Transmission comb value, as defined by the higher layer parameter transmissionComb described in clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or 'codebook', subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously.

-	SRS sequence ID, as defined by the higher layer parameter sequenceId in clause 6.4.1.4 of [4, TS 38.211].

-	SRS cyclic shift and/or comb offset hopping ID, as defined by the higher layer parameter [hoppingID]

-	The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When the target SRS is configured by the higher layer parameter SRS-PosResourceSet, the reference RS may also be a DL PRS configured on a serving cell or a non-serving cell indicated by the higher layer parameter dl-PRS, or an SS/PBCH block of a non-serving cell indicated by the higher layer parameter ssb-Ncell. If the UE is configured with dl-OrJointTCI-StateList or ul-TCI-StateList, the reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell.

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot. NS is divisible by S*R, where S = 2 when [tdm] is configured and S = 1 otherwise, and R is the repetition factor.

If a PUSCH with a priority index 0 and SRS configured by SRS-Resource are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1 would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s).

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'periodic':

-	if the UE is configured with the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing', or 'ssb-IndexNcell', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo, spatialRelationInfo-PDC or spatialRelationInfoPos containing the ID of a reference 'srs' or 'srs-spatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResource and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS. When the SRS is configured by the higher layer parameter SRS-Resource and if the higher layer parameter spatialRelationInfo-PDC contains the ID of a reference 'dl-PRS-PDC', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS for RTT-based propagation delay compensation according to clause 9.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'semi-persistent':

-	when a UE receives an activation command, as described in clause 6.1.3.17 or 6.1.3.36 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set. When the SRS is configured with the higher layer parameter SRS-ResourceSet, each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the SRS is configured with the higher layer parameter SRS-PosResourceSet, each ID in the list of reference signal IDs may refer to a reference SS/PBCH block on a serving or non-serving cell indicated by PCI field in the activation command, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise, or DL PRS resource of a serving or non-serving cell associated with a dl-PRS-ID indicated by DL-PRS ID field in the activation command.

-	if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, the UE shall assume that the ID of the reference signal in the activation command overrides the one configured in spatialRelationInfo or spatialRelationInfoPos.

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing', or 'ssb-IndexNcell' the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'srs' or 'srs-SpatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

If the UE has an active semi-persistent SRS resource configuration and has not received a deactivation command, the semi-persistent SRS configuration is considered to be active in the UL BWP which is active, otherwise it is considered suspended.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2  symbols and an additional time duration  Tswitch. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 +14 symbols and an additional time duration Tswitch. The minimal time interval unit of OFDM symbol is counted based on the minimum subcarrier spacing given by min(µPDCCH, µUL) where µUL is given by min(µUL,carrier1, µUL,carrier2, µSRS) when the UE is configured with the higher layer parameter uplinkTxSwitchingOption set to 'dualUL' for uplink carrier aggregation, and by µSRS otherwise. µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively.

-	Tswitch, µUL,carrier1 and µUL,carrier2 are defined in clause 6.4.

-	A UE reporting its UE capability 'srs-TriggeringDCI' can be indicated with DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI as described in clause 7.3.1.1 of [5, TS 38.212]. Otherwise, except for DCI format 0_1/0_2 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1/0_2 with UL-SCH indicator of "0" and CSI request of all zero(s) as described in clause 7.3.1.1 of [5, TS 38.212].

-	If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource,

-	If ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot ,

-	otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command, respectively;

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, SRSCA and μoffset,SRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. From the first symbol carrying the SRS request DCI to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot.

-	t is configured via higher layer parameter availableSlotOffsetList with up to four different values of AvailableSlotOffset for each triggered SRS resources set and it is based on the subcarrier spacing of the triggered SRS transmission. When one or more SRS resource sets across all configured BWPs in a component carrier are configured, and at least one resource set is configured with availableSlotOffsetList parameter of more than one values, the indicated value of t is indicated by SOI field in DCI scheduling PUSCH/PDSCH and DCI 0_1/0_2 without data and without CSI request described in [5, TS 38.212]. The UE shall apply indicated value t specifically for those sets with configured availableSlotOffsetList parameter. When one or more SRS resource sets across all configured BWPs in a component carrier are configured and at least one resource set is configured with availableSlotOffsetList parameter, and the availableSlotOffsetList parameter for each SRS resource set has only one value, the UE shall apply the configured value specifically for those sets with configured availableSlotOffsetList parameter. For SRS resource set configured with availableSlotOffsetList parameter, each of resource set is configured with K values of AvailableSlotOffset. For SRS resource set configured without availableSlotOffsetList parameter, t = 0 is applied for the resource set.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource

-	if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot ,

-	otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, otherwise, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	μKoffset is the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos containing the ID of a reference 'ssb-Index', 'ssb-IndexServing' or 'ssb-IndexNcell', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo or spatialRelationInfoPos contains the ID of a reference 'srs' or 'srs-SpatialRelation', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet and if the higher layer parameter spatialRelationInfoPos contains the ID of a reference 'dl-PRS', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

The UE is not expected to be configured with different time domain behavior for SRS resources in the same SRS resource set. The UE is also not expected to be configured with different time domain behavior between SRS resource and associated SRS resources set.

For operation in the same carrier, the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter SRS-PosResource and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as 'periodic'.

For operation in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter SRS-PosResource and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'.

For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'periodic'.

For operations in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'semi-persistent' or 'aperiodic'.

For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource on different CCs, subject to UE's capability

For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource and SRS-Resource on different CCs, subject to UE's capability.

The SRS request field [5, TS38.212] in DCI format 0_1, 1_1, 0_2 (if SRS request field is present), 1_2 (if SRS request field is present), 0_3, 1_3 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38.212]. The 2-bit SRS request field in DCI format 2_3 indicates the triggered SRS resource set given in clause 7.3 of [5, TS 38.212] and defined by the entries of the higher layer parameter srs-ResourceSetToAddModList if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB', or indicates the SRS transmission on a set of serving cells configured by higher layers if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA'.

For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s).

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be indicated with a SRS transmission from a carrier and to be configured or scheduled with a PUSCH/UL DM-RS/UL PT-RS/PUCCH transmission from a different carrier in the same symbol.

In case of intra-band contiguous carrier aggregation, or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, or in case of intra-band non-contiguous CA band combination if the UE is not configured with higher layer parameter intraBandNC-PRACH-simulTx-r17, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

In case of intra-band contiguous carrier aggregation, or in inter-band CA band combination if simultaneous SRS and MsgA transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and MsgA from a different carrier.

In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured.

When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs provided by higher layer parameter simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2 is determined by the indicated CC in the MAC CE, the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s) with the same SRS resource ID for all the BWPs in the determined CCs.

When the higher layer parameter enableDefaultBeamPL-ForSRS is set 'enabled', and if the higher layer parameter spatialRelationInfo for the SRS resource, except for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'beamManagement' or for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' with configuration of associatedCSI-RS or for the SRS resource configured by the higher layer parameter SRS-PosResourceSet, or dl-OrJointTCI-StateList or ul-TCI-StateList is not configured in frequency range 2 and if the UE is not configured with higher layer parameter(s) pathlossReferenceRS, and if the UE is not configured with different values of coresetPoolIndex in ControlResourceSets, and is not provided at least one TCI codepoint mapped with two TCI states, the UE shall transmit the target SRS resource in an active UL BWP of a CC,

-	according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest controlResourceSetId in the active DL BWP in the CC. If the CORESET is activated with two TCI states, sfnSchemePdcch is configured and the UE supports sfn-DefaultUL-BeamSetup-r17, UE shall use the first TCI state as the QCL assumption.

-	according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' in the activated TCI state with the lowest ID applicable to PDSCH in the active DL BWP of the CC if the UE is not configured with any CORESET in the active DL BWP of the CC.

For a SRS resource, if the higher layer parameters [tdm] or [combOffsetHopping] are configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource the higher layer parameters [tdm] and [combOffsetHopping] are not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and in clause 6.2.1.1.

<omitted text>

#### 6.2.1.4	UE sounding procedure for positioning purposes

When the SRS is configured by the higher layer parameter SRS-PosResource and if the higher layer parameter spatialRelationInfoPos is configured, it contains the ID of the configuration fields of a reference RS according to Clause 6.3.2 of [TS 38.331]. The reference RS can be an SRS configured by the higher layer parameter SRS-Resource or SRS-PosResource, CSI-RS, SS/PBCH block, or a DL PRS configured on a serving cell or a SS/PBCH block or a DL PRS configured on a non-serving cell. If the UE is configured for transmission of SRS-PosResource in RRC_INACTIVE mode, the configured spatialRelationInfoPos is also applicable.

The UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol.

If the UE is not configured with the higher layer parameter spatialRelationInfoPos the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources.

In RRC_CONNECTED mode, the UE is only expected to transmit an SRS configured by the higher layer parameter SRS-PosResource within the active UL BWP of the UE.

When the configuration of SRS is done by the higher layer parameter SRS-PosResource, the UE can only be provided with a single RS source in spatialRelationInfoPos per SRS resource for positioning.

For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs.

Unless specified otherwise, the UE does not expect to be configured with SRS-PosResource on a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission.

Timing Error Group (TEG) at UE side is defined:

-	UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request or ue-TxTEG-RequestUL-TDOA-Config, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

The UE may report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID.

If the UE is configured with SRS resources configured by the higher layer parameter SRS-PosResource in multiple CCs, the UE should report the carrierFreq or servCellId of the SRS resources when it reports the UE Tx TEG associations.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Tx TEG ID.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

Subject to UE capability, the UE may be configured with an SRS resource for positioning associated with the initial UL BWP, and the SRS resource is transmitted inside the initial UL BWP during RRC_INACTIVE mode with the same CP and subcarrier spacing as configured for the initial UL BWP. Subject to UE capability, the UE may be configured with an SRS resource for positioning outside the initial BWP including frequency location and bandwidth, subcarrier spacing, and CP length for transmission of the SRS in RRC_INACTIVE mode. If the transmission of SRS for positioning outside the initial BWP in RRC_INACTIVE mode along with the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the SRS for positioning transmission is dropped in the symbol(s) where the collision occurs. If a SRS symbol for positioning outside the initial BWP in RRC_INACTIVE mode including the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in paired spectrum or SUL band, subject to UE capability, collides in time domain with UL signals or channels on the same carrier, the colliding SRS symbol for positioning is dropped. The SRS resource for positioning outside the initial BWP in RRC_INACTIVE mode is configured in the same band and CC as the initial UL BWP.

If the UE in RRC_INACTIVE mode is not provided SRS-PosRRC-InactiveValidityAreaConfig and determines that the UE is not able to accurately measure the configured DL RS in SRS-SpatialRelationInfoPos for a SRS resource for positioning where the DL RS is semi-persistent or periodic, the UE stops transmission of the SRS resource for positioning.

The UE is not expected to simultaneously transmit SRS resources configured by the higher layer parameter SRS-PosResource on NUL and SUL band in RRC_INACTIVE mode.

The UE may be configured with SRS, via SRS-PosRRC-InactiveValidityAreaConfi, subject to UE capability, valid in multiple cells within a validity area for RRC_INACTIVE mode. For the configured SRS via SRS-PosRRC-InactiveValidityAreaConfi, if the UE in RRC_INACTIVE mode determines that the UE is not able to accurately measure the configured DL RS in [SRS-SpatialRelationInfoPos] for a SRS resource for positioning where the DL RS is semi-persistent or periodic, the UE would not perform SRS transmission of the SRS resource for positioning. If the UE determines that the configured DL RS in [SRS-SpatialRelationInfoPos] for a SRS resource for positioning is being accurately measured, the UE is expected to perform the SRS transmission.

##### 6.2.1.4.1	SRS frequency hopping for positioning

The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain.  When the reduced capability UE is configured to perform transmit frequency hopping:

-	it expects to be configured with the following parameters:

-	starting PRB of the first hop in time domain in [higher layer parameter]

-	starting slot offset and starting symbol for each hop in [higher layer parameter]

-	number of symbols in each hop in [higher layer parameter]

-	hop bandwidth in [higher layer parameter]

-	number of overlapping resource block(s) between hops, if present, in [higher layer parameter]

-	number of hops in [higher layer parameter].

-	it does not expect to be configured with the sum of [StartingSymbol] and [Length] for a hop that exceeds a slot duration.

-	it expects to be configured with the same periodicity of each hop of an SRS resource with the transmit frequency hopping.

The reduced capability UE may be configured, via [uplinkTimeWindow-Config], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with one [cycle] of the transmit frequency hopping, including the switching time from/to active BWP required ahead of the first hop and after the last hop, that is partially overlapped with the time window.

The reduced capability UE is expected to switch back to the active BWP if the time between two consecutive hops exceeds twice the switching time from/to the active BWP.

For a transmission of a hop for an SRS resource for positioning with frequency hopping starting in symbol Nc1 and a colliding PUSCH or PUCCH transmission starting in symbol NS , the UE shall apply the dropping rules taking into account:

-	DCI(s) for which the time interval between the last symbol of PDCCH and the SRS symbol Nc1is at least N2 symbols and additional time duration TSRSh, where TSRSh is the switching time to/from the active BWP.

-	DCI(s) for which the time interval between the last symbol of PDCCH and the colliding PUSCH/PUCCH symbol NS is at least N2 symbols, where calculation of N2 is based on the smallest SCS between the SCS configured for positioning SRS with the frequency hopping, the SCS of the PUSCH/PUCCH, and the SCS of the PDCCH.

If the SRS symbol(s), including the switching time to and from the active bandwidth part, of the transmit frequency hopping collides with PUSCH or PUCCH, and if the UE determines the SRS to be dropped, the colliding SRS symbol(s) are dropped.

For operation in the same carrier, the reduced capability UE is not expected to be configured on overlapping symbols with an SRS resource of the transmit frequency hopping configured by the higher layer parameter [to_be_defined] including the switching time to or from the active bandwidth part and an SRS resource with resourceType of both SRS resources as 'periodic'.

For operation in the same carrier, the reduced capability UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with a SRS resource of the transmit frequency hopping configured by the higher layer parameter [XX] including the switching time to or from the active bandwidth part and a SRS resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'.

##### 6.2.1.4.2	SRS bandwidth aggregation for positioning measurements

The UE is expected to be configured with linkage information SRS-PosResourceSetLinkedForAggBWList on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, spatialRelationInfoPos, resourceType, subcarrier spacing, CP, and comb size, and the UE is expected to maintain phase continuity for the SRS transmission on the same symbol(s). The UE assumes that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource, along with the [switching period] when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol.

If the UE is configured with dci-TriggeringPosResourceSetLink, and if the UE receives a DCI 0_1, 0_2, 1_1, or 1_2 triggering an aperiodic SRS resource set for positioning linked for bandwidth aggregation in a CC, subject to UE capability, UE transmits SRS of the linked SRS resource sets across all CCs.

A UE in RRC_INACTIVE mode is expected to be configured with freqInfoAdditionalCcList on additional component carrier(s) with respective SRS configuration(s) for bandwidth aggregation.

When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC, there is a guard period during which the UE is not expected to transmit or receive other signals or channels, subject to UE capability.

If the UE receives an activation or deactivation command of semi-persistent SRS resource set(s) for positioning in up to  three aggregated carriers or SRS resource set(s) for positioning in up to two aggregated carriers as specified in [10, TS 38.321] and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation or deactivation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission or cessation corresponding to the SRS resource set(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where µ is the SCS configuration for the PUCCH.

For positioning SRS resources on multiple carriers linked for aggregation, the channel over which a symbol on one carrier for SRS transmission is conveyed can be inferred from the channel over which the same symbol of another carrier or the aggregated carrier is conveyed.

<omitted text>

### 8.2.4	SL PRS transmission procedure

The following parameters for SL PRS transmission are associated with each SL PRS resource:

-	SL PRS resource ID provided by sl-PRS-ResourceID indicates an identity of a SL PRS resource. The SL PRS resource is identified by the SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. For a shared SL PRS resource pool, a SL PRS resource is uniquely identified by a combination of the SL PRS resource ID, SL PRS frequency domain allocation within a slot indicated by “frequency resource assignment” field in the associated SCI format 1-A, and a starting symbol within the slot as determined by clause 8.2.4.1.1.

-	sl-CombSize and sl-PRS-comb-offset indicates a comb offset and a comb size of the SL PRS resource

-	sl-PRS-starting-symbol and sl-NumberOfSymbols indicates the starting symbol index and the number of symbols of the SL PRS resource within a slot in a dedicated SL PRS resource pool. sl-NumberOfSymbols indicates the number of symbols of the SL PRS resource within a slot in a shared SL PRS resource pool.

For a dedicated SL PRS resource pool, SL PRS resources for a same LSL-PRS,KcombSL-PRS combination of number of SL PRS symbols LSL-PRS and comb size KcombSL-PRS can be mapped to a set of consecutive symbols in a slot. SL PRS resources for different LSL-PRS,KcombSL-PRS combinations shall be mapped to non-overlapping sets of consecutive symbols in a slot. Up to four non-overlapping sets of consecutive symbols within a slot can be used to map SL PRS resources for same or different LSL-PRS,KcombSL-PRS combinations, where the case of four non-overlapping sets of consecutive symbols only applies when KcombSL-PRS=2 for all the LSL-PRS,KcombSL-PRS combinations.

Each SL PRS transmission is associated with an PSCCH transmission in the same slot.

In the case of dedicated SL PRS resource pool, that PSCCH carries the SCI format 1-B associated with the SL PRS transmission.

The UE may report the association information between the already transmitted SL PRSs of SL PRS resources and UE Tx ARP ID. The association information includes ARP ID(s) indicated by sl-POS-ARP-ID-Tx, SL PRS transmission timestamp(s) indicated by sl-TimeStamp, and optional SL PRS resource ID(s) indicated by sl-PRS-ResourceID.

<omitted text>

##### 8.2.4.1.1	Resource allocation in time domain

The UE shall transmit the SL PRS in the same slot as the associated PSCCH.

For a dedicated SL PRS resource pool, the minimum resource allocation unit in the time domain is a SL PRS resource in a slot.

The UE shall transmit the SL PRS in consecutive symbols within the slot.

A UE does not transmit multiple SL PRS resources in the same slot.

For a shared SL PRS resource pool, the UE transmits the SL PRS in PSSCH symbols according to clause 8.1.2.1, with the following restrictions:

-	the number of contiguous symbols for SL PRS transmission, LSL-PRS, shall correspond to one of the SL PRS resources in parameter SL-PRS-ResourceSharedSL-PRS-RP.

-	the UE shall not transmit SL PRS in symbols where associated PSCCH is transmitted.

-	the UE shall not transmit SL PRS and PSSCH DMRS in the same symbol.

-	the UE shall not transmit SL PRS and SL CSI-RS in the same symbol.

-	the UE shall transmit SL PRS on contiguous symbols either in between or after symbols where PSSCH DMRS is transmitted.

-	the UE shall transmit SL PRS only after the last symbol with second stage SCI.

-	For a given value of LSL-PRS, SL PRS resource is mapped to the last consecutive LSL-PRS SL symbols in the slot that meet all the other restrictions

For a dedicated SL PRS resource pool, the UE transmits SL PRS subject to the following restrictions:

-	the UE shall not transmit SL PRS and associated PSCCH in the same symbol;

-	the number of contiguous symbols and the starting symbol for SL PRS transmission shall correspond to one of the SL PRS resources in parameter SL-PRS-ResourceDedicatedSL-PRS-RP.

In sidelink resource allocation mode 1 for a shared SL PRS resource pool, the time domain behaviour for sidelink dynamic grants and sidelink configured grants for SL PRS follows the behaviour in clause 8.1.2.1.

In sidelink resource allocation mode 1 for a dedicated SL PRS resource pool, the time domain behaviour for sidelink dynamic grants and sidelink configured grants for SL PRS follows the behaviour in clause 8.1.2.1, with the following modifications:

-	"DCI format 3_0" is replaced by "DCI format 3_2".

-	"PSSCH" is replaced by "SL PRS".

##### 8.2.4.1.2	Resource allocation in frequency domain

For a shared SL PRS resource pool, the frequency domain resource assignment of a SL PRS resource is the same as PSSCH in the same slot.

For a dedicated SL PRS resource pool, the frequency domain resource assignment of a SL PRS resource is same as frequency resources of the resource pool provided by the higher layer parameter sl-RB-Number.

<omitted text>

#### 8.2.4.2	UE procedure for determining the subset of resources to be reported to higher layers in SL PRS resource selection in a dedicated SL PRS resource pool in sidelink resource allocation mode 2

In resource allocation mode 2 in a dedicated SL PRS resource pool, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this SL PRS/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining SL PRS delay budget;

-	Set of SL-PRS resource ID(s);

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for SL PRS/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	[sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.]

-	[sl-Thres-RSRP-List]: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-B and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-PRS-ResourceReservePeriodList: the resource reservation interval, Prsvp_TX, is set to the corresponding value from higher layer parameter in units of mse

-	sl-SensingWindowDedicatedSL-PRS-RP: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindowDedicatedSL-PRS-RP msec

-	sl-TxPercentageDedicatedSL-PRS-RP-List: internal parameter X for a given prioTX is defined as sl-TxPercentageDedicatedSL-PRS-RP-List (prioTX) converted from percentage to ratio

-	[sl-PreemptionEnable]: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

The UE shall perform this procedure according to clause 8.1.4, with the following modifications:

-	"packet delay budget" is replaced by "SL PRS delay budget",

-	partial sensing is not applicable in a dedicated SL PRS resource pool,

-	"candidate single-slot resource" is replaced by "candidate SL PRS resource",

-	a candidate single-slot resource for transmission Rx,y is defined as the SL PRS resource with index x within the Set of SL-PRS resource ID(s) provided by the higher layer and in slot t'ySL,

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	in step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-PRS-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met,

-	In condition b of step 6, the RSRP measurement is the PSCCH-RSRP over the DM-RS resource elements of the PSCCH;

-	In condition c of step 6 "determines according to clause 8.1.5 the set of resource blocks and slots" is replaced by "determines according to clause 8.2.4.2A the set of SL PRS resources and slots ".

#### 8.2.4.2A	UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated SL PRS resource pool

The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields 'SL-PRS resource ID (s))', 'Time resource assignment' of the associated SCI format 1-B as described below.

The set of slots is determined as in clause 8.1.5, with the following modifications:

-	"SCI format 1-A" is replaced by "SCI format 1-B",

-	[potential parameter name changes].

The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by sl-PRS-ResourceID.

The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where,

If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 2 then

PRIV=r1

If sl-MaxNumPerReserveDedicatedSL-PRS-RP is 3 then

PRIV=r2*NSL-PRS+r1

Where

-	 r1 denotes the SL PRS resource ID for the second resource

-	 r2 denotes the SL PRS resource ID for the third resource

-	 NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool.

If TRIV determined according to clause 8.1.5 indicates N < sl-MaxNumPerReserveDedicatedSL-PRS-RP, the SL PRS resource indices corresponding to sl-MaxNumPerReserveDedicatedSL-PRS-RP minus N last resources are not used.

The number of slots in one set of the time and frequency resources for transmission opportunities of SL PRS is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a SL PRS resource in slot t'mSL is determined as the time and frequency resource for SL PRS transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same SL PRS resource in slots t'm+j×Prsvp_TX'SL is also determined for SL PRS transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,… is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

<omitted text>

### 8.4.4	SL PRS reception procedure

The UE may be configured to measure and report one or more of the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL PRS-RSRPP, for the first detected path and up to 8 additional detected paths, and SL PRS-RSRP measurements. The UE may be configured to measure and report one or more of the SL AoA, SL PRS-RSRPP for the first path and up to 2 additional detected paths, and SL PRS-RSRP measurement.

The UE may report an ARP ID associated with the reported measurements. The UE may provide the ARP location information via sl-ARP-LocationInfoPerTxUE.

The UE uses the same ARP for both the transmission and reception of sidelink positioning reference signals while performing an SL Rx-Tx time difference measurement.

The UE may include SL PRS resource ID(s) when it reports one or more of the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL AoA, SL PRS-RSRP, and SL PRS-RSRPP measurements.

For the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL AoA, SL PRS-RSRP, and SL PRS-RSRPP measurements, the UE reports an associated SL PRS reception timestamp via higher layer parameter Timestamps. For SL Rx-Tx time difference, the UE may report an associated SL PRS transmission timestamp via higher layer parameter tx-TimeInfo and the UE may be configured to report a SL PRS transmission timestamp via associatedSL-PRS-TxTimeStampRequest. The timestamp includes the SFN, slot number, and optionally nr-PhysCellID, nr-ARFCN, nr-CellGlobalID, or the timestamp includes DFN and slot number. The timestamp of DFN and slot number may include synchronization source indication of DFN.

The UE may be configured to report up to N Rx-Tx time difference measurements for the same SL PRS transmission associated with N different SL PRS receptions for the same pair of UE(s). The UE may be configured to report up to N Rx-Tx time difference measurements for the same SL PRS reception associated with N different SL PRS transmissions for the same pair of UE(s).

The UE may report, LoS/NLoS indicator(s) via los-NLOS-Indicator associated with each SL RSTD, SL Rx-Tx time difference, SL RTOA, SL AoA, SL PRS-RSRP, and SL PRS-RSRPP measurements.

The UE may report synchronization source type via syncSourceType and/or relative time difference with the associated quality metric, via sl-RTD-Info. If reported syncSourceType is gNB-eNB, the UE may report cell identity information. For UE-based positioning, the UE may be provided with synchronization source type of a UE and/or the relative time difference with the associated quality metric, via syncSourceType and sl-RTD-Info, respectively. For the SL RSTD measurement, the UE may report a reference UE information.

For SL RTOA measurement, SFN or DFN initialization time may be provided to the UE by a UE or the network.

The UE may be provided with the location information of other UEs via anchorUE-LocationInformation. The UE may report the location information of the UE to the network.

The UE may be provided with expected SL AoA and uncertainty range of the expected SL AoA via expectedSL-AzimuthAoA-AndUncertainty and expectedSL-ZenithAoA-AndUncertainty.

The UE may report quality metric sl-TimingQuality corresponding to the SL RSTD, SL RTOA or SL Rx-Tx time difference measurements. The UE may report quality metric sl-AngleQuality corresponding to the SL AoA measurement.

If the 'SL PRS request' field in the SCI associated with the received SL PRS is set to 1 then this request for SL PRS transmission is reported to higher layers.

<omitted text>