3GPP TSG RAN WG1 #116	        		   			R1-2401779

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0179 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RRC parameter names for UTO-UCI indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The parameter name mismatch in Clauses 6.2.7, 6.3.2. RAN2 has chosen the parameter name nrofBitsInUTO-UCI in their latest version of 38.331, while the current version of TS 38.212 uses nrof_UTO_UCI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRC parameter change from nrof_UTO_UCI to nrofBitsInUTO-UCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different names for the same RRC parameter in TS 38.331 and TS 38.212 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.7, 6.3.2.1.3A, 6.3.2.1.4, 6.3.2.1.5, 6.3.2.4.1, 6.3.2.4.2, 6.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged parts are omitted ***

### 6.2.7	Data and control multiplexing

In case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with highest IMCS value for the initial PUSCH, where IMCS is as defined in Clause 6.1.4.1 in [6, TS 38.214]. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block. The PUSCH for UCI multiplexing in this Clause referes to the UL-SCH transport block for UCI multiplexing.

If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "when higher layer parameter cg-UCI-Multiplexing is configured" with "when UTO-UCI and HARQ-ACK are transmitted on a PUSCH".

*** Unchanged parts are omitted ***

##### 6.3.2.1.3A	UTO-UCI

For UTO-UCI bits transmitted on a CG PUSCH when the higher layer parameter nrofBitsInUTO-UCI is configured, the UTO-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiUTO-UCI  for i=0,1, …, OUTO-UCI-1 and A=OUTO-UCI, where OUTO-UCI is provided by nrofBitsInUTO-UCI, and the UTO-UCI bit sequence o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI is given by clause 9.3.1 of [5, TS 38.213].

##### 6.3.2.1.4	HARQ-ACK and CG-UCI/UTO-UCI

If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.3.2.1.4 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "When higher layer parameter cg-UCI-Multiplexing is configured" with "When UTO-UCI and HARQ-ACK have the same priority index and are jointly encoded and transmitted on a PUSCH".

*** Unchanged parts are omitted ***

##### 6.3.2.1.5	UCI with different priority indexes

If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.3.2.1.5 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part" with "is given by clause 9.3.1 of [5, TS 38.213]".

*** Unchanged parts are omitted ***

##### 6.3.2.4.1	UCI encoded by Polar code

If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedures in this clause and the clauses it refers to apply by replacing CG-UCI with UTO-UCI in all the notations and texts, when applicable.

*** Unchanged parts are omitted ***

##### 6.3.2.4.2	UCI encoded by channel coding of small block lengths

If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedures in this clause and the clauses it refers to apply by replacing CG-UCI with UTO-UCI in all the notations and texts.

*** Unchanged parts are omitted ***

#### 6.3.2.7	Multiplexing of coded UCI bits with different priority indexes to PUSCH

If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.3.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, when applicable.

*** Unchanged parts are omitted ***