3GPP TSG-RAN WG1 #116	R1-2401750

Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0510 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SRS transmission collision rule in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2024-2-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RRC_INACTIVE, the colliding SRS symbol(s) for positioning is dropped when the overlap/collision occurs for both unpaired spectrum, paired spectrum and SUL band. The consensus is that dropping is per SRS symbol instead of per SRS transmission. However, the correction and clarification [R1-2312594] is only captured for paired spectrum and SUL band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the dropping description for SRS transmission in RRC_INACTIVE state for unpaired spectrum is per symbol basis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on which symbol(s) to be dropped when the collision happened for unpaired spectrum. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<UNCHANGED PARTS OMITTED>

#### 6.2.1.4	UE sounding procedure for positioning purposes

<UNCHANGED PARTS OMITTED>

Subject to UE capability, the UE may be configured with an SRS resource for positioning associated with the initial UL BWP, and the SRS resource is transmitted inside the initial UL BWP during RRC_INACTIVE mode with the same CP and subcarrier spacing as configured for the initial UL BWP. Subject to UE capability, the UE may be configured with an SRS resource for positioning outside the initial BWP including frequency location and bandwidth, subcarrier spacing, and CP length for transmission of the SRS in RRC_INACTIVE mode. If an SRS symbol for positioning outside the initial BWP in RRC_INACTIVE mode including the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the colliding SRS symbol for positioning is dropped. If an SRS symbol for positioning outside the initial BWP in RRC_INACTIVE mode including the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in paired spectrum or SUL band, subject to UE capability, collides in time domain with UL signals or channels on the same carrier, the colliding SRS symbol for positioning is dropped. The SRS resource for positioning outside the initial BWP in RRC_INACTIVE mode is configured in the same band and CC as the initial UL BWP.

<UNCHANGED PARTS OMITTED>