3GPP TSG RAN WG1 #116	        		   			R1-2401877

# Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0600 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance of BWP operation without restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Core |  |  |  |  |  | Date: |  |  | 2024-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In RAN#102 it was agreed to remove the “Option B-1-2” capability (BW that includes the SS/PBCH block outside the active DL BWP with interruptions). - Capture the RAN1#116 agreement that the SS/PBCH blocks in Clause 11.1 for directional collision handling for a set of serving cells among multiple serving cells correspond to the SS/PBCH blocks the UE used to obtain SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove rlm-BM-BFD-CD-SSB-MeasWithInterrupt-r18 in Clause 5. - Capture the above agreement in Clause 12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.306, TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

# 5	Radio link monitoring

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell unless the UE indicates a capability rlm-BM-BFD-L3-IntraFreq-CD-SSB-MeasWithoutInterrupt [18, TS 38.306]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS.

If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell unless the UE indicates a capability rlm-BM-BFD-L3-IntraFreq-CD-SSB-MeasWithoutInterrupt [18, TS 38.306].

## *** Unchanged parts are omitted ***

# 12	Bandwidth part operation

## *** Unchanged parts are omitted ***

If a UE is provided by firstActiveDownlinkBWP-Id a first active DL BWP and by firstActiveUplinkBWP-Id a first active UL BWP on a carrier of a secondary cell, the UE uses the indicated DL BWP and the indicated UL BWP as the respective first active DL BWP on the secondary cell and first active UL BWP on the carrier of the secondary cell.

If a UE is provided NonCellDefiningSSB in BWP-DownlinkDedicated for an active DL BWP, the UE assumes that the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB. The SS/PBCH blocks provided by NonCellDefiningSSB and the SS/PBCH blocks that the UE used to obtain SIB1 have same QCL properties if they have a same index. Unless otherwise stated, handling of overlapping between downlink receptions or uplink transmissions and the SS/PBCH blocks provided by NonCellDefiningSSB is same as handling of overlapping between downlink receptions or uplink transmissions and the SS/PBCH blocks provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. The SS/PBCH blocks in Clause 11.1 for resolving directional collisions for a set of serving cells among multiple serving cells, when the UE is provided directionalCollisionHandling and indicates support of half-DuplexTDD-CA-SameSCS, correspond to the SS/PBCH blocks the UE used to obtain SIB1.

A UE does not expect to monitor PDCCH when the UE performs RRM measurements [10, TS 38.133] over a bandwidth that is not within the active DL BWP for the UE.

## *** Unchanged parts are omitted ***