3GPP TSG-RAN4 Meeting #110	R4-2400316

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0475 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_DL256QAM_FR2-Perf) Correction to 256QAM CQI reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL256QAM_FR2-Perf |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For 50 MHz bandwidth the spec demands currently a subbandsize of 8 RB and 9 subbands. However in 50 MHz there are only 32 RB available, so the number of subbands needs to be reduced to 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct csi-ReportingBand parameter to include only 4 subbands for 50 MHz bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-4 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

8.2.2.2.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI. To account for sensitivity of the input SNR the CQI reporting under frequency non-selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 8.2.2.2.2.1-1 and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified by the following:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α % of the time, where α% is specified in Table 8.2.2.2.2.1-2;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 8.2.2.2.2.1-2;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.01.

Table 8.2.2.2.2.1-1 Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  | Test 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  | 50 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |  |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |  |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |  |  |  |  |
| SNRBB |  |  | dB | 6 | 7 | 12 | 13 | 7 | 8 | 20 | 21 |
| Propagation channel |  |  |  | TDLA30-35 |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | 2×2 ULA High |  |  |  |  |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |  |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |  |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |  |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |  |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |  |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |  |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |  |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |  |  |  |  |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic |  |  |  |  |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |  |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |  |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |  |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |  |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |  |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | Not configured |  |  |  |  |  |  |  |
|  | aperiodicTriggeringOffset |  |  | 0 |  |  |  |  |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |  |  |  |  |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |  |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |  |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |  |  |  |  |  |  |  |
| ReportConfigType |  |  |  | Aperiodic |  |  |  |  |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |  |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |  |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |  |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |  |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |  |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |  |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  | 1111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | Not configured |  |  |  |  |  |  |  |
| Aperiodic Report Slot Offset |  |  |  | 6 |  |  |  |  |  |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |  |  |  |  |  |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |  |  |  |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |  |  |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |  |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |  |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |  |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |  |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |  |  |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |  |  |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.375 |  |  |  |  |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |  |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-1 |  |  |  | As specified in Table A.4-2, TBS.2-7 |  |  |  |

Table 8.2.2.2.2.1-2 Minimum requirements

|  | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
|  [%] | 2 | 2 | 2 | 2 |
|  | 1.05 | 1.05 | 1.05 | 1.05 |

#### < End of changes >