3GPP TSG-RAN4 Meeting #110	R4-2400296

# Athens, Greece, 26th Feb – 1st Mar

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3989 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf,NR_redcap-Perf) CR to FR1 SA intra-frequency SS-RSRQ measurement accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf, NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Gap Pattern ID is specified in test parameters of A.6.7.2.1, A.16.7.2.1, A.16.7.2.2, but gap seems unnecessary for intra-frequency measurement of these TCs. Besides, other similar TCs (intra-frequency SS-RSRP/ SINR measurement) do not have specified Gap Pattern ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed unnecessary Gap Pattern ID in test parameters table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.7.2.1, A.16.7.2.1, A.16.7.2.2 Isolated impact analysis: No change to UE requirements, changes test parameters only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

#### A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

##### A.6.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in Table A.6.7.2.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell 2 |  |  |  | Cell 1 |  | Cell 1 |  | Cell 2 |  | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 |  | 0 |  | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |  | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 | ms | - | 3 | - | 3 |  | - |  | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  | - |  | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -104.5 |  |
|  |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5..46 | -5.46 |
|  |  |  | dB | 3 | 3 | -2.9 |  | -2.9 |  | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 | -82 | -103.9 |  | -103.9 |  | -118 | -118 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.5 | -117.5 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117 | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116 | -116 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.5 | -115.5 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115 | -115 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  | - |  | -115 | -115 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 | -113.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 | -113 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 | -112.5 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 | -112 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 | -108.5 |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  | -16.76 |  | -17.34 | -17.34 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -70 |  |  |  | -83.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -83 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -82.5 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -82 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -81.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -81 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -80.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -80 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -77 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  | - |  |  |  | -77.4 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -76.9 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -76.4 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -75.9 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -75.4 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -74.9 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -74.4 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -73.9 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -70.9 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  | AWGN |  | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.1.1.

<<Unchanged sections skipped>>

#### A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

##### A.16.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1A.6.

##### A.16.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.16.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in Table A.16.7.2.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.16.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.16.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |  | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1, 4 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 4 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |  | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1, 4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1, 4 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,4 | ms | - | 3 | - | 3 |  | - |  | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  | - |  | 3 |
| SMTC configuration |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -104.5 |  |
|  |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5..46 | -5.46 |
|  |  |  | dB | 3 | 3 | -2.9 |  | -2.9 |  | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 | -82 | -103.9 |  | -103.9 |  | -118 | -118 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.5 | -117.5 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117 | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116 | -116 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.5 | -115.5 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115 | -115 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  | - |  | -115 | -115 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 | -113.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 | -113 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 | -112.5 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 | -112 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 | -108.5 |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  | -16.76 |  | -17.34 | -17.34 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -70 |  |  |  | -83.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -83 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -82.5 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -82 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -81.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -81 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -80.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -80 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -77 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 18.36MHz | -50 |  | - |  |  |  | -80.6 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -80.1 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -79.6 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -79.1 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -78.6 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -78.1 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -77.6 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -77.1 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -74.1 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  | AWGN |  | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x1 | 1x1 | 1x1 |  | 1x1 |  | 1x1 | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.16.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1A.6.

#### A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE

##### A.16.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1A.6.

##### A.16.7.2.2.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.16.7.2.2.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in Table A.16.7.2.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.16.7.2.2.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.16.7.2.2.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |  | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1, 4 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 4 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |  | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1, 4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1, 4 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,4 | ms | - | 3 | - | 3 |  | - |  | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 |  | - |  | 3 |
| SMTC configuration |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 |  | 0 |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -110.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -107.5 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -104.5 |  |
|  |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5..46 | -5.46 |
|  |  |  | dB | 3 | 3 | -2.9 |  | -2.9 |  | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 | -82 | -103.9 |  | -103.9 |  | -118 | -118 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.5 | -117.5 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117 | -117 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.5 | -116.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116 | -116 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.5 | -115.5 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115 | -115 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  | - |  | -115 | -115 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 | -114 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 | -113.5 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 | -113 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 | -112.5 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 | -112 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 | -108.5 |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  | -16.76 |  | -17.34 | -17.34 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -70 |  |  |  | -83.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -83 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -82.5 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -82 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -81.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -81 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -80.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -80 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -77 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 18.36MHz | -50 |  | - |  |  |  | -80.6 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -80.1 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -79.6 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -79.1 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -78.6 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -78.1 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -77.6 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -77.1 |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -74.1 |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN |  | AWGN |  | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

##### A.16.7.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1A.6.

<<End of change>>