3GPP TSG-RAN4 Meeting #110	R4-2400060

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | 0026 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_repeaters-Perf) CR for TS 38.115-1, Correction on manufacturer declarations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Perf |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To correct the note of manufacturer declarations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The manufacturer declarations are defined for repeater, not Base Station. 4.6 manufacturer declarations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The manufacturer declarations for repater would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

4.6	Manufacturer declarations

The following repeater declarations listed in table 4.6-1, when applicable to the repeater under test, are required to be provided by the manufacturer for the conducted requirements testing of the repeater type 1-C. Declarations can be made independently for UL and DL.

Table 4.6-1: Manufacturer declarations for repeater type 1-C conducted test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.1 | Repeater class | Repeater class of the repeater, declared as Wide Area repeater, Medium Range repeater, or Local Area repeater. |
| D.2 | Operating bands and passband frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the repeater and passband frequency range(s) within the operating band(s) that the repeater can operate in. Declarations shall be made per antenna connector. |
| D.3 | Spurious emission category | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [4]. |
| D.4 | Additional operating band unwanted emissions | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 2, Note 3). |
| D.5 | Co-existence with other systems | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. |
| D.6 | Co-location with other base stations, repeaters and IABs | The manufacturer shall declare whether the repeater under test is intended to operate co-located with Base Stations, repeaters and IABs of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. |
| D.7 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. |
| D.8 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D.12) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. |
| D.9 | Rated output power per passband (Prated,p,AC) | Conducted rated output power per passband, per single band connector or multi-band connector. Declared per supported passband, per antenna connector. (Note 1) |
| D.10 | Rated total output power (Prated,t,AC) | Conducted total rated output power. Declared per supported operating band, per antenna connector. For multi-band connectors declared for each supported operating band in each supported band combination. (Note 1) |
| D.11 | Rated multi-band total output power, Prated,MB,TABC | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1) |
| D.12 | Operating band combination support | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the repeater. Declared per antenna connector. |
| D.13 | Equivalent connectors | List of antenna connectors which have been declared equivalent. Equivalent connectors imply that the antenna connector are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the antenna connector are identical and the transmitter unit and/or receiver unit driving the antenna connector are of identical design. |
| D.14 | Connecting network loss range for repeater testing with ancillary RF amplifiers | Declaration of the range of connecting network losses (in dB) for repeater type 1-C testing with ancillary Tx RF amplifier only, or with Rx RF amplifier only, or with combined Tx/Rx RF amplifiers. (Note 4) |
| D.15 | Long delay repeater | Declared only if the repeater internal delay between the input and output for this repeater does not fit within the TDD transient time. The repeater is intended for situations in which it will not cause interference to other nodes. This is achieved by RF isolation or by reservation of longer guard periods, which degrades frame utilization. The length of repeaters internal delay is declared using this declaration. |
| D.16 | Input signal power level for maximum output power | Declaration of input signal power level required to reach maximum output power. Declared per passband. |
| D.17 | Repeater radiating direction | Declaration on whether the repeater is intended to radiate in DL, UL or both. Testing shall be performed only for the direction(s) in which the repeater radiates. |
| NOTE 1: If a repeater is capable of 256QAM operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM operation, and the other declaration is applicable when not configured for 256QAM operation. If a repeater is not capable of 256QAM operation, only one declaration can be made. NOTE 2: If repeater is declared to support Band n20 (D.2), the manufacturer shall declare if the repeater may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 3: If repeater is declared to support Band n24 (D.2), the manufacturer shall declare if the repeater may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 4: This manufacturer declaration is optional. |  |  |

<End of Change>