3GPP TSG-RAN4 Meeting #110	R4-2403451

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7311 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 36.133 on Clarification of Discontinuous Coverage Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current phrasing on discontinuous coverage aspects for cell reselection is ambiguous at some parts regarding the behavior related to the acquisition of SI information as discussed in R4-2402697. Besides wording is not aligned across the different sections Current FFS left by editor needs to be addressed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rephrasing the requirements for UE in discontinuous coverage scenarios Word alignment Address the FFS left by editor |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous interpretation of the specification and requirement for discontinuous coverage will remain unaplicable due to the lack of definition of “in-coverage” status. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6A.2, 4.7A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2402698 |  |  |  |  |  |  |  |  |

### 4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access

The cell reselection procedure allows the UE to select a more suitable cell and camp on it.

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency and inter-frequency cells indicated by the serving NB-IoT cell. For intra-frequency and inter-frequency cells the serving NB-IoT cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. UE measurement activity is also controlled by measurement rules defined in TS36.304, allowing the UE to limit its measurement activity.

If SystemInformationBlockType32 [2] has been received and if the UE has determined that it will be out of coverage after t-service, the UE is not required to perform cell measurements from the last slot on the last SI modification period that ends before t-Service [2]. After that, if the UE has discontinuous coverage capabilities:

- when the UE is provided with t-serviceStart-r17 the UE is not required to perform cell measurements until t-serviceStart-r17 is reached; or

- when the UE is provided with footprintInfo the UE is not required to perform cell measurements until the UE is within the area provided by footprintInfo.

<< End of change 1 >>

<< Change 2>>

### 4.7A.2	Cell Re-selection for UE category M1 for Satellite Access

The cell reselection procedure allows the UE to select a more suitable cell and camp on it.

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency and inter-frequency cells indicated by the serving cell. For intra-frequency and inter-frequency cells the serving cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only. UE measurement activity is also controlled by measurement rules defined in TS36.304, allowing the UE to limit its measurement activity.

If the UE is in normal coverage as defined in section 3.6.1, the requirements in section 4.7A.2.1 apply. If the UE is in enhanced coverage as defined in section 3.6.1, the requirements in section 4.7A.2.2 apply.

If SystemInformationBlockType32 [2] has been received and if the UE has determined that it will be out of coverage after t-service, the UE is not required to perform cell measurements from the last slot of the last SI transmission that ends before t-Service [2]. After that, if the UE has discontinuous coverage capabilities:

- when the UE is provided with t-serviceStart-r17 the UE is not required to perform cell measurements until t-serviceStart-r17 is reached; or

- when the UE is provided with footprintInfo the UE is not required to perform cell measurements until the UE is within the area provided by footprintInfo.

Editor’s note: FFS on how to capture following agreement:

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS.

<< End of change 2>>