3GPP TSG-RAN4 Meeting #110	R4-2403022

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | 0047 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.176-2, Correction on IAB-MT Output power dynamics test requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Perf |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Output power dynamics test requirement for Local Area IAB-MT under 40MHz is not correct. In addition, expanding IAB-MT type 1-O to capture different TT values for different bands . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Output power dynamics test requirement for Local Area IAB-MT under 40MHz equals [Tx Dynamic range +/- (Relative power tolerance + TT)]. For Local area Tx Dynamic range is 10 dB, while Relative power tolerance is 5.5 dB and TT is 0.7/1 dB for <40MHz and <100MHz, respectively. Thus, upper bound should be 16.2 dB (16.5) for the two frequency ranges in Table 6.4.2.1.5.1-1 and 6.4.2.1.5.2-1. Additionally, the test requirements for IAB-MT type 1-O is extended for the different bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Output power dynamics test requirements for IAB-MT will not be fully correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.4.2.1.5.1	IAB type 1-O

For IAB-MT the ΔP between the power measured in step 6 and step 8 of clause 6.4.2.1.4.2 shall be according to Table 6.4.2.1.5.1-1, Table 6.4.2.1.5.1-2 and Table 6.4.2.1.5.1-3.

Table 6.4.2.1.5.1-1: IAB type 1-0 Output power dynamics test requirements for band n41.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement (Note 1) |
| --- | --- | --- |
| Wide area | ≤40MHz | 10 log(Maximum RB) -1.2 < ΔP ≤ 10 log(Maximum RB) + 11.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) -1.5 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.8 < ΔP ≤ 10 log(Maximum RB) + 16.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 3.5 < ΔP ≤ 10 log(Maximum RB) + 16.5 |

Table 6.4.2.1.5.1-2: IAB type 1-0 Output power dynamics test requirements for band n77 and n78.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement (Note 1) |
| --- | --- | --- |
| Wide area | ≤40MHz | 10 log(Maximum RB) - 1.5 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) - 2.1 < ΔP ≤ 10 log(Maximum RB) + 12.1 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.5 < ΔP ≤ 10 log(Maximum RB) + 16.5 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 2.9 < ΔP ≤ 10 log(Maximum RB) + 17.1 |

Table 6.4.2.1.5.1-3: IAB type 1-0 Output power dynamics test requirements for band n79.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement (Note 1) |
| --- | --- | --- |
| Wide area | ≤20MHz | 10 log(Maximum RB) - 1.8 < ΔP ≤ 10 log(Maximum RB) + 11.2 |
|  | 20MHz < BW ≤ 40MHz | 10 log(Maximum RB) - 2.0 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) - 2.1 < ΔP ≤ 10 log(Maximum RB) + 17.1 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.2 < ΔP ≤ 10 log(Maximum RB) + 16.8 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 3.0 < ΔP ≤ 10 log(Maximum RB) + 17 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 2.9 < ΔP ≤ 10 log(Maximum RB) + 17.1 |

6.4.2.1.5.2	IAB type 2-O

For IAB-MT the ΔP between the power measured in step 6 and step 8 of clause 6.4.2.1.4.2 shall be according to Table 6.4.2.1.5.2-1.

Table 6.4.2.1.5.2-1: IAB type 2-0 Output power dynamics test requirements.

| IAB-MT Type | IAB-MT channel bandwidth | Requirement |
| --- | --- | --- |
| Wide area | ≤40MHz | 10 log(Maximum RB) -1.2 < ΔP ≤ 10 log(Maximum RB) + 11.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) -1.5 < ΔP ≤ 10 log(Maximum RB) + 11.5 |
| Local area | ≤40MHz | 10 log(Maximum RB) + 3.8 < ΔP ≤ 10 log(Maximum RB) + 16.2 |
|  | 40MHz < BW ≤ 100MHz | 10 log(Maximum RB) + 3.5 < ΔP ≤ 10 log(Maximum RB) + 16.5 |

END OF CHANGES