# 3GPP TSG-RAN WG3 Meeting #123	R3-240845

# 26th Feb - 1st March 2024, Athens, Greece

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1095 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IE names in TS 38413 for UAV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV-Core |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A2X function has been introduced in UAV topic in Rel-18. For A2X authorization requirement, two IEs(e.g. NR A2X Services Authorized, LTE A2X Services Authorized) have been introduced in multiple RAN3 specifications(e.g. TS 38.413, TS 38.423, TS 38.473). Currently, 2 sub IEs with the name UAV-UE and UAV- controller UE respectively has been used in the A2X Service Authorized aspects in TS 38.413. However, the abbreviation of the “UAV” has not been introduced in any RAN3 specifications(including TS 38.413). It is not appropriate for RAN3 to directly use the name “UAV” without further explanation in the related specification. Sync up with other related specifications(e.g. TS 38.423 and TS 38.473). Change the name “UAV UE” to “Aerial UE” and change the name “UAV Controller UE” to “Aerial Controller UE” in both tabular and ASN.1 part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sync up with other related specifications(e.g. TS 38.423 and TS 38.473). Change the name “UAV UE” to “Aerial UE” and change the name “UAV Controller UE” to “Aerial Controller UE” in both tabular and ASN.1 part. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UAV function can not be captured in this specification properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.247, 9.3.1.248, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Re-submission to RAN3#123. |  |  |  |  |  |  |  |  |

# Changes Start

#### 9.3.1.247	NR A2X Services Authorized

This IE provides information on the authorization status of the UE to use the NR sidelink for A2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aerial UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Aerial UE |
| Aerial Controller UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Aerial Controller UE |

#### 9.3.1.248	LTE A2X Services Authorized

This IE provides information on the authorization status of the UE to use the LTE sidelink for A2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aerial UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Aerial UE |
| Aerial Controller UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Aerial Controller UE |

--- SKIP UNCHANGED PART ---

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

--- SKIP UNCHANGED PART ---

LTE-A2X-ServicesAuthorized ::= SEQUENCE {

aerialUE									ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

aerialControllerUE						ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTE-A2X-ServicesAuthorized-ExtIEs} }		OPTIONAL,

...

}

LTE-A2X-ServicesAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

--- SKIP UNCHANGED PART ---

NR-A2X-ServicesAuthorized ::= SEQUENCE {

aerialUE									ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

aerialControllerUE						ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NR-A2X-ServicesAuthorized-ExtIEs} }		OPTIONAL,

...

}

NR-A2X-ServicesAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

# End of Changes