3GPP TSG RAN #103			RP-240719

Maastricht, Netherlands, March 18-21, 2024

Agenda item: 9.1.4.6

Source:	Apple (Moderator)

Title:   	Moderator's summary for discussion on Sidelink

Document for: Discussion

# Introduction

In RP-240019 (Proposed Summary for RAN Rel-19 Package: RAN4 Part) prepared by RAN Chair and RAN4 Chair, the following proposed scope is endorsed as a starting point for discussion in RAN#103.

This document summarizes the relevant contributions on this topic and provides moderator recommendations for further discussions with an aim to reach an agreement on R19 WI scope.

# List of relevant contributions and their views

| RP-240315 | Considerations for Release-19 RAN4-led WI/SIs | KT | do we have room to also introduce power class 3 for NR SL-U? |
| --- | --- | --- | --- |
| RP-240434 | Views on other RAN4-led topics for Rel-19 | Huawei, HiSilicon | The following objectives could be considered for Rel-19 sidelink enhancement - Specify the intra-band non-contiguous CA including 10MHz+10MHz and 10MHz+20MHz with power class 2 (PC2) and power class 3 (PC3) - Sidelink CA operation band combinations and configurations - Update channel arrangement including channel spacing, raster, sync raster, and other aspects, if needed - Tx requirements - Maximum output power and MPR for PC2 and PC3 - Update configured output power, output power dynamic, Transmit signal quality, out-of-band emission, spurious emissions, if needed - Rx requirements - Update REFSENS, Maximum input level, ACS, blocking, etc, if needed - Specify the intra-band contiguous CA with power class 2 (PC2) - Sidelink CA operation band combinations and configurations - Tx requirements - Maximum output power and MPR for PC2 - Update other RF requirements, if needed No RRM and demodulation performance requirements would be needed for above Rel-19 sidelink enhancement. |
| RP-240547 | Views on Rel-19 RAN4 cross-area and other topics | Intel | Core part: - Specify the necessary RF requirements for intra-band non-contiguous Sidelink CA with the focus on band n47. - Include CC combinations 10MHz + 10MHz and 10MHz + 20MHz for UE Power Class 2 (26 dBm) and Power Class 3 (23 dBm) |
| RP-240310 | R19 NR Sidelink continuation and enhancements in RAN4 | OPPO | 1. For SL CA enhancement - Specify requirements for intra-band non-contiguous CA with power class 3 - Targeted band combination is SL_n47(2A) - Bandwidth combinations are 10+10MHz and 10+20MHz - UE with separate chains to support non-contiguous CA is prioritized - Specify requirements for intra-band contiguous CA with non-contiguous RB allocation - Targeted band combination is SL_n47B with power class 3 2. For SL unlicensed - Specify new NS values for SL unlicensed bands n46, n96 and n102 based on companies’ input |
| RP‑240329 | Motivation on Rel-19 further NR sidelink evolution | LG Electronics, Volkswagen AG | - Objectives - Specify the following requirements for NR sidelink CA in n47. - Intra-band non-contiguous CA with power class 2 and power class 3 - Intra-band contiguous CA with power class 2 - Intra-band contiguous CA with non-contiguous RB allocation - Specify the missing requirements for NR sidelink in unlicensed spectrum. - Remaining 12 NS values - Specify the corresponding RRM core requirements for the above features. |
| RP‑240548 | Input on Rel-19 NR Sidelink CA in RAN4 | Volkswagen AG, Continental Automotive Technologies GmbH, ROBERT BOSCH GmbH, LG Electronics | Work item objectives: - Support of intra-band non-contiguous CA in band n47, including the Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz, for UE Power Class 2 (26 dBm) and Power Class 3 (23 dBm) - Introduction of Power Class 2 (26 dBm) for intra-band contiguous sidelinkCA in band n47 |
| RP‑240397 | Views on R19 topics impacting both RF and RRM | CATT | Objectives •Specify necessary RF requirements for sidelink CA •Specify necessary RRM requirements for sidelink CA if any |

# Moderator recommendations for further discussions

## Intra-band non-contiguous CA in band n47

It is agreeable to consider the following WI scope in the WID:

- Specify the RF requirements for NR sidelink CA in n47:

  - Intra-band non-contiguous CA with power class 2 and power class 3

  - Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz

- Specify the corresponding RRM core/performance requirements if identified.

## Intra-band contiguous CA in band n47

It is recommended to assess the amount of work required for support of intra-band contiguous CA with power class 2, as is requested by 5GAA in the LS (RP-240023), and then decide whether it can be included in the WID:

- Specify the RF requirements for NR sidelink CA in n47:

  - Intra-band contiguous CA with power class 2

- Specify the corresponding RRM core/performance requirements if identified.