3GPP TSG RAN meeting #103										RP-240752

Maastricht, Netherlands, March 18-21, 2024

## Status Report to TSG

Agenda item:			10.9.1

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_NTN_Ph3 |  |  |  |  |
| Unique ID | 1020096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234077 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 5% RAN1: 10% RAN2: 0% RAN3: 0% RAN4: 0% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Chun-Fan (Felix) Tsai |
|  | Company | MediaTek Inc. |
|  | Email | Chun-Fan.tsai@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

###### RAN1#116, February 26th–March 1st

Agreements on 9.11.4 IoT-NTN uplink capacity/throughput enhancement

Agreement

For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study:

- Time domain OCC where OCC spreads across:

  - Symbol-level

  - Slot-level

  - Repetition-level

  - RV-level

For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study:

- Time domain OCC where OCC spreads across:

  - Symbol-level

  - Slot-level (including Nslot level)

  - Repetition-level

  - RV-level

- Intra-symbol pre-DFT spreading OCC

Agreement

The following evaluation assumptions are used for the study of OCC for NPUSCH format 1:

|  | Parameter | value |  |  |
| --- | --- | --- | --- | --- |
| scenario | orbit | GEO | LEO600 |  |
|  | Elevation angle | 12.5 degree | 30degree |  |
| Channel and impairments | carrier frequency | 2GHz |  |  |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |  |  |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |  |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |  |  |
|  | Power imbalance | Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value |  |  |
| transmitter | SCS | 3.75KHz and 15KHz | 15kHz |  |
|  | Number of tones | Single tone | Single tone and multi tone up to 12 tones |  |
|  | Waveform | DFT-s-OFDM |  |  |
|  | Frequency hopping | w/o frequency hopping |  |  |
|  | MIMO scheme | SISO |  |  |
|  | DMRS configuration | For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |  | For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |
|  | Number of resource unit (NRU) | Up to proponent |  | Up to proponent |
|  | Modulation order (Qm) | Up to proponent |  | Up to proponent |
|  | TBS (ITBS) | Up to proponent |  | Up to proponent |
|  | Number of repetitions (Nrep) | Up to proponent |  |  |
|  | OCC length | Up to 4 |  |  |
|  | OCC sequence | Up to proponent |  |  |
|  | Number of UE | Up to 4 |  |  |
|  | Velocity of UE | 3km/h |  |  |
| receiver | Receiver algorithm | MMSE |  |  |
|  | Channel estimation | Real channel estimation |  |  |
| KPI | SNR at 10% BLER | Report for baseline and OCC schemes |  |  |
|  | Aggregated throughput | Total throughput of up to 4 UEs multiplexed |  |  |

#### 2.1.2	Remaining Open issues

Definition of necessary physical layer features enabling

- IoT-NTN uplink capacity/throughput enhancement

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN1

RAN1#116 meeting, Athens, Greece, February 26th – March 1st, 2024:

R1-2400074, Discussion on IoT-NTN uplink capacity/throughput enhancement, Spreadtrum Communications	RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400135, Discussion on UL capacity enhancements for IoT NTN, Huawei, HiSilicon, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400262, Discussion on IoT-NTN uplink capacity enhancement, vivo, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400347, Discussion on the IoT -NTN uplink capacity/throughput enhancements,	CMCC, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400358, Discussion on UL capacity enhancement for IoT NTN, ZTE, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400427, Discussion on UL capacity enhancement for IoT NTN, CATT, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400552, Discussion on IoT-NTN uplink capacity enhancement, xiaomi, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400605, Discussion on IoT-NTN uplink capacity/throughput enhancement, OPPO, RAN1#116, Athens, Greece, February 26th March 1st	2024

R1-2400629, IoT-NTN uplink capacity/throughput enhancement, InterDigital, Inc., RAN1#116, Athens, Greece, February 26th March 1st 1024

R1-2400752, Discussion on uplink capacity/throughput enhancement for IoT-NTN, Samsung, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400790, Discussion on IoT-NTN uplink capacity/throughput enhancement, LG Electronics, RAN1#116, Athens, Greece, February 26th March 1st

R1-2400876, Discussion on uplink capacity enhancement for IoT NTN, Lenovo, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2400879, IoT-NTN uplink capacity enhancement, Nokia, Nokia Shanghai Bell	, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401032, Discussion on IoT-NTN uplink capacity enhancement, Apple, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401061, IoT NTN uplink enhancement with NPRACH multiplexing, Sharp, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401195, On uplink capacity enhancements for IoT-NTN, Ericsson, RAN1#116, Athens, Greece, February 26th March 1st

R1-2401240, Discussion on IoT-NTN uplink capacity enhancement, ETRI, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401302, Discussion on IoT-NTN uplink capacity and throughput, MediaTek Inc., RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401373, IoT-NTN uplink capacity-throughput enhancement, Mavenir, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401461, IOT-NTN uplink capacity/throughput enhancement, Qualcomm Incorporated, RAN1#116, Athens, Greece, February 26th March 1st 2024

R1-2401607   FL Summary #1 for IoT-NTN  , Moderator(SONY), RAN1#116, Athens, Greece, February 26th March 1st 2024