3GPP TSG RAN Meeting #103	RP-240772

# Maastricht, Netherlands, March 18-21, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4540 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of emergency cause value for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | - |  |  |  |  |  |  |  |  |  |
| Source to TSG: | OPPO |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R18 U2N relay, the dedicated RSC has been introduced for emergency service by SA2, and it will be used by the Relay UE and Remote UE during discovery and PC5 link establishment. According to SA2 conclusion on cause value setting, if PC5 connection was requested using emergency RSC, then the U2N Relay sets the RRC Establishment cause to “emergency” when establishing an RRC connection from RRC_IDLE. RAN2 discussed the cause value setting issue including how to handle the mismatch between R18 SA2 spec and R17 RAN2 spec, and made the following conclusion: - For SL-RLC0 case, Rel-17 mechanism (up to UE implementation) is reused in R18 (no RAN2 specification impact); - For SL-RLC1 case, RAN2 will align with SA2 in that for a Rel-18 relay UE with emergency RSC, the relay UE should set an emergency cause value, FFS for the spec impact. For the FFS point on the spec impact, the following 2 approaches are discussed in RAN2 and leave the down selection to CT1 1) upper layer provides the information of emergency link for relay case, or 2) upper layer provides the emergency cause value for relay case directly. CT1 has replied to the LS in C1-239362: CT1 thanks RAN2 on their LS on emergency cause value for relay. CT1 has discussed the below question from RAN2 and provides the below answer. Q1: Which approach is to be adopted by CT1 for the emergency cause value setting for SL_RLC1 case? A1: Approach 1 is agreed by CT1 (i.e. upper layer provides the information of emergency link for relay case).. In the replied LS, CT1 has already selected approach-1 and implemented the corresponding CR in their specification, RAN2 should implement approach 1 in RAN2 specification and make the change accordingly. |
| Summary of change: |  | In section 5.3.3.3, add the case that relay UE can set the emergency if the PC5 unicast link establishment request from L2 U2N Remote UE is for emergency service based on upper layer indication in the NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The L2 U2N Relay cannot set the cause value as emergency when it initiates RRC connection establishment triggered by reception of message from a L2 U2N Remote UE via SL-RLC1, which causes the mismatch of UE behavior defined in RAN2/SA2/CT1 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.554 CR 0480 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.3.3.3	Actions related to transmission of RRCSetupRequest message

The UE shall set the contents of RRCSetupRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	set the ue-Identity to ng-5G-S-TMSI-Part1;

2>	else:

3>	draw a 39-bit random value in the range 0..239-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the 5G-S-TMSI if the UE is registered in the TA of the current cell.

1>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN):

2>	set the establishmentCause to mps-PriorityAccess;

1>	else:

2>	set the establishmentCause in accordance with the information received from upper layers;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection establishment triggered by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.3.1a, the L2 U2N Relay UE sets the establishmentCause by implementation, but: (1) for SL-RLC0, it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as establishmentCause if the same cause value is in the message received from the L2 U2N Remote UE via SL-RLC0; and (2) for SL-RLC1, it sets the establishmentCause to emergency if the message received from the L2 U2N Remote UE via SL-RLC1 is over PC5 link established for emergency service as indicated by upper layer [72].

1>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

If the UE is an (e)RedCap UE and the (e)RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.6.

NOTE 3:	For L2 U2N Remote UE in RRC_IDLE, the cell (re)selection procedure as specified in TS 38.304 [20] and relay (re)selection procedure as specified in 5.8.15.3 are performed independently and up to UE implementation to select either a cell or a L2 U2N Relay UE.

End of Change