3GPP TSG RAN meeting #103	RP-240745

Maastricht, NL 18 – 21 March 2024

## Status Report to TSG

Agenda item:			9.3.2.4

| WI / SI Name | XR (eXtended Reality) for NR Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_XR_Ph3 |  |  |  |  |
| Unique ID | 1020098 |  |  |  |  |
| TSG TDoc of latest approved WI/SI description (if any) | RP-234080 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 2025/09 | Performance part: 2026/03 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 2% | Performance Part: 0% | Testing part: N/A |  |

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Benoist Sébire |
|  | Company | Nokia |
|  | Email | benoist.sebire@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

During the first RAN1 meeting of the work item (RAN1#116), the following was agreed:

-	Consider at least solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

-	FFS: Other types of solutions.

-	Whether or not/how to account for any UE assistance information/indication in addition to other information available at the network

-	From RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements.

-	FFS: Whether or not/How to support of the case where an occasion(s) of gap/restrictions that are caused by RRM measurements are cancelled/skipped partially

-	For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider the following alternatives or combinations for further down-selection:

-	Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements.

-	Alt. 2: Semi-persistent solution to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements.

-	Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

-	Alt. 4: Dynamic solution to adapt/change gap/SMTC configuration to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

-	Alt. 5: Rule-based solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements:

-	Companies are encouraged to use the EVM in TR38.835 if they are submitting simulation results.

In addition, a working assumption was made:

-	RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements.

NOTE: UE features related to the developed solution(s) is a separate discussion.

#### 2.1.2	Remaining Open issues

The one RAN1-related objective (related to RRM measurements) remains open.

## 2.2	RAN2

RAN2-related work has not started yet.

## 2.3	RAN4

RAN4-related work has not started yet.

## 3.	Detailed progress in SA WGs since last TSG meeting (for all involved WGs)

## 4.	References

RAN1#116

- R1-2400922	Work Plan for Rel-19 on XR (eXtended Reality) for NR Phase 3	Nokia, Qualcomm (Rapporteurs)

- R1-2400258	Discussion on enabling data transmissions for XR during RRM measurements	vivo

- R1-2400677	RRM measurement gap and scheduling restriction enhancements for TX/RX of XR traffic	Ericsson

- R1-2401457	Enabling Tx/Rx for XR during RRM measurements	Qualcomm Incorporated

- R1-2400070	Discussion on enabling TX/RX for XR during RRM measurements	Spreadtrum Communications

- R1-2400129	Discussions on scheduling enhancements considering RRM measurements for XR	Huawei, HiSilicon

- R1-2400343	Discussion on enabling TX/RX for XR during RRM measurements	CMCC

- R1-2400360	Enabling TX/RX for XR during RRM measurements	TCL

- R1-2400423	Signaling control of scheduling restriction during measurement gap in support of XR services	CATT

- R1-2400476	Discussion on enabling TX/RX for XR during RRM measurements	NEC

- R1-2400498	Discussion on measurement gap for XR	ZTE, Sanechips

- R1-2400548	Discussion on enabling TX/RX for XR during RRM measurements	xiaomi

- R1-2400615	Enhancements to enable TX/RX for XR during RRM measurements	OPPO

- R1-2400748	Discussion on enabling TX/RX for XR during RRM measurements	Samsung

- R1-2400820	Discussion on Enabling TX/RX for XR during RRM measurements	InterDigital, Inc.

- R1-2400836	Enabling TX/RX for XR during RRM measurements	Lenovo

- R1-2400868	Considerations on enabling UE Tx/Rx during RRM measurements	Sony

- R1-2400886	Enhancements to enable TX/RX for XR during RRM measurements	Fraunhofer IIS, Fraunhofer HHI

- R1-2400889	Discussion on enabling TX/RX for XR during RRM measurements	Panasonic

- R1-2400921	Enabling TX/RX for XR during RRM measurements	Nokia, Nokia Shanghai Bell

- R1-2401028	Discussion on enabling TX/RX for XR during RRM measurements	Apple

- R1-2401033	Discussion on Enabling TX/RX for XR During RRM Measurements	Meta

- R1-2401129	Discussion on Enaling TX/RX for XR during RRM	NTT DOCOMO, INC.

- R1-2401211	On enabling Tx/Rx for XR during RRM measurements	Google Inc.

- R1-2401315	Discussion on enabling TX/RX for XR during RRM measurements	MediaTek Inc.

- R1-2401338	Discussion on XR during RRM measurements	LG Electronics

- R1-2401717	Moderator summary #1 - Enabling TX/RX for XR during RRM measurements	Moderator (Nokia)

- R1-2401718	Moderator summary #2 - Enabling TX/RX for XR during RRM measurements	Moderator (Nokia)