3GPP TSG RAN #107 Meeting 	RP-250043

Incheon, Republic of Korea, March 12-14th, 2025

Title:	Candidate RAN2 led IoT-NTN topics for Rel-20 5GA

Source: 	Thales

Type:	discussion

Document for:	discussion

Agenda Item:	15.2.5

# Introduction

This document proposes to define new capabilities or performance optimizations for IoT-NTN radio interface as part of Rel-20 5G Advanced work plan.

# Candidate RAN2 led IoT-NTN topics for Rel-20 5G Advanced

## Context

The following NTN related topics (capabilities or performance optimizations) have been identified for IoT-NTN by different companies.

This includes the following RAN1/2/3 led IoT-NTN related topics

- GNSS resilient operation (GNSS signal may be temporarily unavailable)

- IMS voice call over GEO

- Enhanced operation of NB-IoT-NTN in NR channel

- 5GC support of IoT NTN

- Network-controlled anchor carrier mobility

- Further S&F enhancements

- Further enhancements on TDD mode in NB-IoT

- NB-IoT NRS muting and enhancements on cell DTX/DRX

- Inter-RAT mobility between NR TN and IoT NTN (Idle and Connected mode)

- Broadcast information to assist the inter RAT cell reselection from IoT-NTN to NR-NTN and vice e versa

- NPRACH enhancement with OCC (Orthogonal Cover Code)

- CRDSA (Contention Resolution Diversity Slotted ALOHA)

## Proposed IoT-NTN enhancements for Rel-20 5G advanced

We recommend for a RAN2 led IoT-NTN activity, a number of topics in the table below to be considered in Rel-20 5G Advanced:

### GNSS resilient operation for uplink time and frequency synchronization in NTN based access

#### Justification

5G IoT-NTN and NR-NTN specifications in Releases 17, 18 and 19 (presently in progress) assume that the UE uses its GNSS capabilities to estimate its position i.e., UE position - see TS 36.300 Section 23.21.2.2 and TS 38.300 Section 16.14.2.2, and the ephemeris of the serving satellite and additional information about the feeder link delay to pre-compensate the time and frequency to correct for Doppler and timing shifts.

While GNSS generally provides a highly accurate position and time reference, its vulnerability to jamming (leading to denial of service) and spoofing (leading to incorrect location reporting and potentially denial of service) present a significant risk. Also, a terminal, or group of terminals attempting to connect over access channels or once connected to the network, could also cause denial or degradation of service for all other users in the cell if using an altered GNSS position.

Additionally, GNSS handling may cause delays to the UE operation due to positioning acquisition time (e.g., for a GNSS "cold start") and additional energy consumption.

NTN systems remain overly dependent on GNSS, and in scenarios where GNSS is unavailable or compromised, NTN availability is affected. Therefore, to enhance robustness and operational continuity, NTN UEs of all types including handheld terminals and mobile VSAT (*) devices do require alternative means of achieving and maintaining successful uplink time and frequency synchronization in NTN based access. NTN positioning can be considered as a fallback solution in 5G-Advanced. This approach does not replace GNSS, but rather ensures NTN service resilience; furthermore, enabling this feature does not change the way the initial access is currently performed – it will continue to be based on UE position, satellite ephemeris, and feeder link delay information.

(*) It is assumed that for a stationary device (e.g., fixed VSAT, static IoT device), the UE position can be pre-configured.

Other benefits provided by NTN positioning for supporting GNSS resilient NTN operation would include improved energy efficiency. Additionally, supporting NB-NTN RAT-dependent positioning in Rel-20 will fulfil unaddressed 3GPP requirements (e.g., TS 22.261 Section 6.46.10).

#### Objectives

Hypothesis

- UE (up to 250 km/h) with GNSS receiver (but GNSS service may be unavailable)

- NGSO, GSO

- In a given radio cell, it is assumed that either all UE can receive GNSS or not

- Multiple satellite in visibility of a UE

Objective

- For IoT-NTN, enhance existing positioning reference signals (e.g., NPRS, PRS, etc.) and define additional network assistance information broadcasted across multiple satellites for the UE to determine its position with sufficient accuracy (RAN4 timing requirements in TS 38.133 and frequency requirements in TS 38.101-5). This enhancement aims to ensure that UEs can perform UL timing/frequency synchronization in Idle (including initial access) and Connected modes

- Define, if needed, enhancement(s) to closed loops for timing / frequency corrections.

Note: The same enhancements should also be defined to NR-NTN

#### Working groups

- RAN1 [0.5 TU per meeting]

- RAN2 [0.25 TU per meeting]

- RAN4 RRM [0.5 TU per meeting]

- RAN4 RF [0.25 TU per meeting]

### IMS voice call over NB-IoT-NTN via GEO

#### Justification

IoT NTN connectivity was introduced in 3GPP in Rel-17 and enhanced in Rel-18 and the on-going Rel-19. However, the current specification does not support real-time services among the features supported by NB-IoT.

Satellite direct communication complements existing terrestrial networks due to their large coverage, ensuring that users always have access to a communication service. In particular, the GEO satellites present an opportunity to provide such global communication services for IoT and smartphones.

Indeed, GEO satellites provides a global coverage for low data rate services, adapted for emergency communication, i.e. SMS and voice. The market already exists and rely on proprietary solutions, which can be cost-prohibitive and may not provide a consistent user experience. The service can be addressed by the 3GPP NB-IoT NTN technology. So far, this technology targeted the satellite direct connectivity to IoT devices and services, thus, the current solution cannot enable voice call over GEO satellites.

In the SA1#108 meeting, a new Work Item Description (S1-244772) on satellite access was approved. One of its key objectives is to utilize the consolidated requirements, such as data rate and call setup time outlined in TR 22.887, as a foundation for supporting IMS voice calls via GEO satellite access. This initiative marks an important step forward in enhancing the capabilities of satellite communication for voice services. In the SA2#167, the IMS voice call over GEO via NB-IoT NTN has been agreed (S2-2502778) as a work task for Rel-20.

#### Objectives

Hypothesis

- GEO only

- UP data transfer mode only supported

- Only static UEs are supported in this release

- Both transparent and regenerative architectures are supported

- UE location determination for regulatory purpose relies on existing procedures e.g. UE coarse location reporting or up to satellite network operator

Objectives

- Study the support of IMS voice call over NB-IoT-NTN and specify necessary enhancements if any

  - assuming GBR radio bearer adapted to the new low data-rate voice codec (i.e. ~1kbps) and semi-persistent scheduling (SPS)

Note: The support of IMS voice call with the new voice codec should also be defined for NR-NTN.

#### Working groups

RAN2, RAN3

Coordination with SA2, SA4 and SA3-LI.

# Conclusion and Way forward

Proposal 1: RAN to start defining the necessary enabling functions for a RAN2 led IoT-NTN work item as part of the Rel-20 5G Advanced work plan:

- GNSS resilient operation for uplink time and frequency synchronization in IoT-NTN based access (i.e. support of multiple satellites positioning with NPRS configuration indication in SIB)

- Support of IMS voice call over NB-IoT-NTN via GEO

- Support of Multi-Standard Radio (MSR) for Enhanced in band operation of NB-IoT-NTN in NR

END