3GPP TSG RAN Meeting #107	RP-250048

Incheon, Korea, March 12-14, 2025

Agenda Item:	15

Source: 	CEPRI, SGIT

Title: 	Proposal on Service-Centric 5G High-Density Networking for Intensive Electrical Power Business

Document for: 	Discussion and Decision

Background

The power system depends on robust and efficient communication networks to facilitate seamless data transmission across all operational stages, including power generation, transmission, distribution, and consumption. The rapid advancement and widespread adoption of 5G communication technology have led to a significant increase in the number of power 5G communication network terminals. To address the escalating data service demands of a growing user base, the high-density deployment of power 5G communication networks has emerged as a critical development strategy.

Currently, 3GPP has integrated NB-IoT and eMTC into the 5G framework as standardized protocols tailored for large-scale connectivity scenarios. These technologies employ key features such as narrow bandwidth, extended discontinuous reception (eDRX), half-duplex communication, and repeated transmission to substantially reduce communication costs, minimize terminal power consumption, and improve coverage. However, these benefits come at the expense of increased communication latency, rendering them more suitable for non-continuous services.

In the new power system era, the power industry has heightened its demands for communication latency. Beyond the ultra-low latency required for control services, even data collection services now necessitate relatively low latency to enable real-time alignment of power generation and consumption, thereby reducing wind and power curtailment rates. Consequently, there exists a notable disparity between the existing large-scale connectivity technologies and the evolving requirements of the power industry.

To address the large-scale connectivity demands in certain power scenarios, reliance solely on single-layer macro station coverage is no longer viable. Instead, the collaborative and complementary deployment of macro stations and small/micro stations has emerged as an effective solution. 5G ultra-dense networks (UDN), characterized by high bandwidth, low latency, and massive connection capacity, are poised to provide robust support for the intelligent and digital transformation of the power industry.

Consideration

With the ultra-dense deployment of diverse base station types, cell sizes are progressively shrinking, and traditional regular cell configurations are becoming obsolete. Power service terminals can now simultaneously connect to multiple access nodes, while access nodes, in turn, can serve multiple terminals concurrently. In this context, macro stations deliver high-power coverage and support mobility, whereas small/micro stations handle hotspot traffic. This setup enables multi-connection and simultaneous service delivery across different base station types and terminals. However, the escalating data demands at the user end and the substantial energy consumption at the base station side are rendering traditional cellular network architectures and access modes increasingly inadequate for the demands of ultra-dense networks.

The service-centric networking architecture addresses these challenges by offloading certain base station functions to centralized servers, decoupling the control and user planes as well as uplink and downlink transmissions, and eliminating the conventional cellular structure. This architecture dynamically coordinates the allocation of multiple access nodes, making it highly suitable for high-density networking applications in power scenarios. Each base station operates as an access point (AP), and multiple APs form AP groups tailored to different service users. These groups sense the wireless environment across various service areas and coordinate network resources to mitigate mutual interference and reduce signaling overhead in dense network configurations.

This architecture comprises two main components: a central controller and access nodes. The central controller facilitates large-scale collaborative processing, coordinates and manages wireless resources, and performs intelligent management functions, interfacing with the core network through feedback links. Access nodes, which may range from simple forwarding devices to more complex processing units, are deployed extensively in designated service areas. They provide communication services to terminals via wireless access links and transmit data to the central processor through wired or wireless fronthaul links. This design enhances scalability, efficiency, and adaptability in ultra-dense network environments.

Proposals

Proposal 1: Study on the service-centric 5G high-density collaborative networking architecture for power scenario.

Proposal 2: Study on interference management for intensive access of power business.