3GPP TSG RAN Meeting #107									RP-250053

Incheon, Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242394 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 80% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#120, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification.

- The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups

  - Subject to separate UE capability

- Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling

Agreement

On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- The current beam RS and new beam RS(s) are in the same CC

  - The above does NOT imply to preclude the cross-CC TCI indication.

- The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different.

  - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

Agreement

On UE-initiated/event-driven beam reporting, for Event 7, the candidate value of RRC parameter Q = {1, 2, 3, 4, 5, 6, 7, 8}

- Note: The UE does not expect that the configured Q is greater than the number of the activated DL/joint TCI state(s).

Conclusion

There is no RAN1 consensus on the following proposal:

- On beam report transmission procedure for UE-initiated/event-driven beam reporting, multi-bit indication in the first PUCCH is supported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, the value of X symbols for determining available transmission occasion of the second UL channel is configured by RRC (FFS: subject to a corresponding UE capability)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR

- Option-1: LRR > first PUCCH > normal SR

Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, the following Option-1 is supported.

- Option-1: A CSI trigger state corresponding to UE-initiated/event-driven beam reporting can NOT be associated with legacy AP-CSI report configuration.

Agreement

Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting.

- Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received;

  - FFS: whether to reset the counting for all new beams.

  - FFS: whether to maintain the counting whose new beam is NOT updated.

- Candidate#2: [The measured current beam based on] indicated TCI state is updated;

  - In such case, the UE need to reset the counting for all new beams.

- Candidate#3: UEI beam report is transmitted;

  - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report

- Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected.

- Candidate#5: The time window expires

- Candidate#6: The threshold for event evaluation is re-configured by RRC signaling

- (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

  - FFS: RRC parameter(s)

- FFS: Other candidates

Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

Agreement

On UE-initiated/event-driven beam reporting, for Event 1, support Option-2 for RS measurement:

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - In such case, the report format for Event-2 is reused unless critical technical issue is identified

    - FFS: How to report the RSRP value of the current beam

    - FFS: RSRP of the current beam is always reported

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

  - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

- Note: Other options are not precluded.

Agreement

On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured

Above is NOT applicable for event 1.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2.

- Only a single UEI beam report is carried in the second PUSCH.

  - Additional indication of one ‘CSI report configuration’ is provided in the report format.

    - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource.

  - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations.

    - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload.

  - The reported UEI beam report should satisfy the triggering condition.

  - If multiple UE initiated beam report procedures occur, down-select one of the following options:

    - Option-1: It is up to UE implementation to select one of configuration.

    - Option-2: The UEI beam report with highest priority is reported

    - Option-3: The report triggered in the latest measurement is reported in PUSCH

  - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC.

  - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState.

  - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH

FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH.

- If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rule for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH

- Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped.

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped.

- FFS: whether/how to handle the case of different PHY priorities.

CSI

Conclusion

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, regarding timeline, the value of drelax will be decided in UE feature session

- Note: Per previous agreement, only a single non-zero value of Drelax=drelax is supported, in addition to the already agreed Drelax=0

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1),

- The legacy CSI sub-band definition and partitioning rules are reused

Agreement

For the Rel-19 Type-I SP codebook refinement for P=48, 64, 128 CSI-RS ports, regarding Scheme-B, when the UE is configured to report wideband CSI on PUCCH:

- For PUCCH format 2, one-part CSI is used

- For PUCCH formats 3 and 4, decide, by RAN1#120, between the two alternatives:

  - Alt1 (baseline). One-part CSI is used

  - Alt2. Two-part CSI is used where SD basis selection is reported in CSI-part2

    - CSI fields in CSI-part1 and part2 follows the legacy sub-band CSI

Agreement

For the Rel-19 Type-I SP codebook refinement for P=48, 64, and 128 CSI-RS ports, regarding the CSI reference resource

- For P=128 with Capability 2 timeline, when periodic or semi-persistent CSI reporting is configured, decide (by RAN1#120) between the two alternatives:

  - Alt1. nCSI_ref follows legacy specification

  - Alt2. scale nCSI_ref from the legacy specification by ceil(P/32)

- For all other cases, nCSI_ref follows legacy specification

to determine the CSI reference resource for periodic or semi-persistent CSI reporting

Conclusion

For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, whether to change the maxNumberTxPortsPerResource to maxNumberTxPortsPerResourceGroup for Rel-19 codebook triplet capability is to be discussed in UE feature session.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding the value of the RRC-configured resource-level slot offset (relative to the resource-set-level slot offset) for aperiodic CSI-RS resource set, at least the following values are supported: {0, 1, 2, 3}

- FFS: Whether to also support {4, 5, …, X-1} where X is either 8, 16, or 32 for only Type-II or all applicable codebooks

- Note: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the RRC-configured resource-level slot offset (relative to the resource-set-level slot offset) for aperiodic CSI-RS resource set does not apply to CSI-RS for tracking

Agreement

For the Rel-19 Type-I SP codebook refinement for P=48, 64, 128 CSI-RS ports, regarding Scheme-B, when the UE is configured to report wideband CSI on PUCCH formats 3 and 4, one-part CSI is used.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding the value of the RRC-configured resource-level slot offset (relative to the resource-set-level slot offset) for aperiodic CSI-RS resource set, in addition to the agreed values of {0, 1, 2, 3}, the following values are supported: {4, 5, 6, 7}

Agreement

For the Rel-19 Type-I SP codebook refinement for P=128 CSI-RS ports, with Capability 2 timeline, when periodic or semi-persistent CSI reporting is configured, nCSI_ref is the smallest value greater than or equal to 10⋅2μ, such that it corresponds to a valid downlink slot.

Agreement

Following agreement from RAN1#118bis is updated (in RED):

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource,

- Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic”

- When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report

  - Note: when periodic or semi-persistent CSI-RS is configured, in case of time-domain Measurement Restriction not configured, the UE can assume that the occasions of the NTRP CSI-RS resources are the latest no later than the CSI reference resource.

3Tx uplink

Agreement

For 3T3R antenna switching, reuse the same mechanism as xTxR for the SRS configuration as follows:

- Up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 4 with port 1003 disabled if the UE is not indicating srs-AntennaSwitching2SP-1Periodic.

- Up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 4 with port 1003 disabled.

- For 3T=3R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

Note: This is not meant to be a text proposal to be captured for the specifications.

Conclusion

To support 3T3R antenna switching for a 3TX UE,

- Reuse the power control parameters configuration principles, i.e., 3T6R, when two aperiodic SRS resource sets are configured,

  - If a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE shall expect that the two SRS resource sets are associated with the same values of the higher layer parameters p0AlphaSetforSRS and pathlossReferenceRS-Id

  - Otherwise, the UE shall expect that the two SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

Conclusion

For a 3TX UE, following agreements for codebook-based transmission can also be extended for non-codebook-based transmission,

Agreement For codebook-based transmission by a 3TX UE, - Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig, - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication. Above is only for single panel transmission. Agreement For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows: - Alt2: 2-bit indication PTRS-DMRS association when 1 PT-RS port is configured Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port Reserved.

Agreement

For non-codebook-based UL transmission for a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication for the two DMRS ports that share a PTRS port is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares the PTRS port | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares the PTRS port | 1 | 2nd DMRS port which shares PTRS port 1 |

- Number of bits used for the indication

  - 1 bit

- Above is applicable when a 3TX UE is configured with 3 SRS resources for non-codebook-based UL transmission

Conclusion

For a 3TX UE, following agreements for codebook-based transmission can also be extended for non-codebook-based transmission,

Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association, - When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition, - When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3, - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP).

Agreement

For 3T3R and 3T6R antenna switching, support the following separate UE capabilities for SRS configuration,

- “srs-AntennaSwitching3T6R2SP-1Periodic” to extend srs-AntennaSwitching2SP-1Periodic to the case of 3T6R.

- “srs-AntennaSwitching3T3R2SP-1Periodic” to extend srs-AntennaSwitching2SP-1Periodic to the case of 3T3R.

Asymmetric DL-sTRP/UL-mTRP

Agreement

For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH:

- The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH;

- The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH:

  - In this case, the PRACH configuration associated with addition PCI is used.

- This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

Conclusion

There is no RAN1 consensus to support the following proposal:

Support the UE to report either Type1 PHR or Type 3 PHR in a serving cell configured with one UL carrier and two separate SRS CLPC adjustment states. The UE determines to report Type1 PHR or Type 3 PHR according to:

- UE provides the Type 1 PHR if both the Type 1 PHR and the Type 3 PHR are based on respective actual transmissions or on respective reference transmissions.

- UE provides the PHR that is based on a respective actual transmission if either the Type 1 PHR or the Type 3 PHR is based on a respective reference transmission.

This is subject to UE capability.

This is feature is enabled by one new RRC parameter.

Conclusion

There is no consensus to introduce the restriction that only one of the indicated joint/UL TCI states can be configured with PL offset.

Conclusion

There is no consensus to include PL offset in Type 3 PHR calculation.

Conclusion

In asymmetric DL sTRP/UL mTRP deployment scenario, the PDCCH-order PRACH to UL TRP is CFRA.

Agreement

- The legacy TPC application mechanisms specified in legacy spec are reused for the TPC command for separate SRS CLPC adjustment state(s) indicated in DCI format 1_1 in Rel-19 and the TPC command for separate SRS CLPC adjustment states indicated in DCI format 2_3 in Rel-19.

  - One example specification changes in TS38.213 can be

7.3.1 UE behavior *** Unchanged parts are omitted *** - δSRS,b,f,c(m) is jointly coded with other TPC commands in a PDCCH with DCI format 2_3, as described in clause 11.4 or provided by TPC command field XXX in a PDCCH with DCI format 1_1 *** Unchanged parts are omitted *** - hb,f,c(i)=δSRS,b,f,c(i) if the UE is not configured for PUSCH transmissions on active UL BWP b of carrier f of serving cell c, or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3 or DCI format 1_1 carrying DCI field XXX KSRS,min symbols before a first symbol of SRS transmission occasion i, where absolute values of δSRS,b,f,c are provided in Table 7.1.1-1 *** Unchanged parts are omitted ***

- Note: how to capture this in specification is up to editor.

- In Rel-19, the TPC command for separate SRS CLPC adjustment state(s) indicated in DCI format 1_1 and the TPC command for separate SRS CLPC adjustment states indicated in DCI format 2_3 are applicable to periodic SRS, semi-persistent SRS and aperiodic SRS.

Agreement

Study whether/how to reset the CLPC of PUSCH/PUCCH/SRS based on the following alternatives (including if any enhancement is necessary and other alternatives are not precluded)

- Alt1: For a UE configured with two TAGs, when the UE receives a RAR of a PDCCH-order PRACH, according to the new 1-bit DCI field PL offset indication in DCI format 1_0:

  - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state.

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state.

  - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state;

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state

- Alt 2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR

  - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG.

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Remaining details on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Remaining details on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Remaining details on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Remaining details on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Remaining details on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Remaining details on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Remaining details on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - Remaining details on SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Remaining details on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Remaining details on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Remaining details on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Asymmetric DL-sTRP/UL-mTRP:

- Remaining details on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Remaining details on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Remaining details on two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

## 2.2	RAN2

#### 2.2.1	Agreements

- RAN2#128

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

RAN2#129

One PL offset value is indicated for each TCI state included in the new MAC CE.

The new MAC CE contains one serving cell ID and one BWP ID

TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed)

The new MAC CE can include flexible number of PL offset values.

Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found.

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. No need to send a LS to R1 on this.

#### 2.2.2	Remaining Open issues

MAC:

- FFS PL offset impact to PHR trigger.

- FFS PL offset impact (if any) to RACH procedure.

- FFS MAC impacts of UE-initiated/event-driven beam management based on RAN1 agreements.

RRC:

- RRC signaling design based on RAN1 RRC parameter list

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#114, the following agreements were made.

RF

3Tx uplink

For PC3 and PC2, specify requirements for FWA and handheld UE

RRM

UE-initiated/event-driven beam management

Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting?

- Agreement: Reuse current beam sweeping factor for UE/event triggered beam reporting.

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)?

Agreement:

- The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1.

TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS

  - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4

    - TReport = 0 or be removed from the measurement period

    - N: Reuse the legacy definition

    - M: FFS pending RAN1 further agreement

    - TSSB (taking SSB as example)

      - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new

      - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1

        - Option A: Max{TSSB_current, TSSB_new}

        - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new.

        - Option C: others are not precluded

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

Agreement:

- Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

Agreement

- The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

CJT calibration (CJTC) reporting

Issue 1-1: How to decide on the number of samples used for CJT calibration?

Agreement:

- RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters

  - Report resolution (MD = 128, MFO = 128).

  - Offset range (AD and AFO)

    - AD = CP

    - AFO = 0.2 ppm

  - Time offset between the two TRPs: 0.5 CP

  - Frequency offset between the two TRPs: 0.1 ppm

  - Note that report resolution and offset range are network configuration via RRC signalling.

- Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

Asymmetric DL-sTRP/UL-mTRP

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset?

Agreement:

For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2.

For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not).

RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1.

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

Note 1: the scenario 1 and 2 are based on RAN1 LS.

Note 2: The above agreements apply when 2TA are configured.

Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

  - The above agreement can be revisited if not aligned with RAN1 further agreement.

#### 2.4.2	Remaining Open issues

RF

3Tx uplink

FFS on PC1.5 with 3Tx, will be decided in the next meeting

  - Option 1: Consider it in the next release

  - Option 2: Specify requirements in Rel-19

FFS on PC2 MPR for 3Tx

- Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results

- Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case

Asymmetric DL-sTRP/UL-mTRP

FFS on UE RF impact from Asymmetric DL-sTRP/UL-mTRP

- Option 1: No UE RF impact of asymmetric DL sTRP/UL mTRP

- Option 2: Adopt following dCR, where the companies are encouraged to propose wording improvements in the next meeting

RRM

UE-initiated/event-driven beam management

FFS on the event-triggered L1 reporting delay related to TL1-RSRP_event triggered_measurement_Period

FFS on whether to define RRM requirements for with time-window-based configured.

FFS on whether to define new TCI state switching delay or existing requirements are sufficient

FFS on whether Event-1/Event-7 reuse the same requirements as Event-2

CJT calibration (CJTC) reporting

FFS on whether to have RRM impacts based on simulation results.

Asymmetric DL-sTRP/UL-mTRP

FFS on whether/how to define the timing  requirements by 2TA enhancement if PL offset is not configured.

FFS on whether to define UL TCI state activation requirement based on SRS.

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template